3GPP TSG-RAN WG3 Meeting #127	             R3-250521

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:	8.1

Source:	CMCC

Title:	Discussion on SA4 LS Reply on Multi-Modality Awareness

Document for:	Discussions & Approval

# Introduction

SA4 provides the feedback on multi-modality awareness as below [1]:

RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation). Answer: From an XR service perspective, it is generally desirable that the gNB should try to grant all associated QoS Flows for the XR service whenever possible. However, not all applications require all QoS flows to be granted. If the gNB cannot grant all associated QoS Flows, only the application layer can determine how to handle the case when the QoS flow for one modality cannot be established e.g. due to RAN resource limitations. For instance, it may be acceptable to an application if the QoS flow for video cannot be established with the requested QoS requirements but it may not be acceptable if the QoS flow for audio cannot be established. The conditions under which particular QoS flows are considered essential for an application to continue are application specific and cannot be determined by 5GS. For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the mutli-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.. Answer: For DL, from the application perspective, there are well-defined mechanisms to support synchronized playout among different media flows, e.g. RTP timestamps, jitter buffering mechanism in the RTP client. These mechanisms guarantee frame level synchronization but do not provide packet level synchronization among different media flows. There is no evidence to suggest 5GS per packet inter-modal synchronization is needed for XR services, as the very low round trip latency requirements will prevent the system from introducing unacceptable inter-modal skew. Regarding PDU Set discard, it is recommended that the 5GS not jointly discard packets across multi-modal flows. Receiving packets for some modalities is preferred than receiving none at all as this enables the application to use the received modalities to better manage the user experience (e.g., pause the video while rendering received audio packets, or render received video packets while concealing an audio packet drop).

# Discussions

According to SA4’s feedback on multi-modality awareness, XR services often have multiple types of data streams or modalities (e.g., video, audio, control signals, and sensory data). These different streams or modalities are mapped to QoS flows in the 5G network. The gNB should ideally aim to grant all associated QoS flows for an XR application, as XR services are sensitive to the simultaneous delivery of multiple data types (e.g., video with low latency and audio with high reliability).  While XR services benefit from having all QoS flows granted, not every application within the XR space requires every single QoS flow. For example, a basic AR application may function effectively even if some QoS flows, like high-bandwidth video streams, are not granted, as the application may only need minimal bandwidth for sensor data and low-latency control signals. Similarly, some XR applications might prioritize certain flows over others, depending on the modality (e.g., prioritizing video over less critical data streams such as low-resolution tracking data). Joint admission control shall take the context into consideration. Simply admit all or none is not a wise solution.

Inter-modal skew refers to the timing misalignment between different data streams (e.g., audio arriving significantly earlier than video). In XR services, this can degrade the user experience if not properly managed. Many XR applications are designed to handle minor timing variations between modalities at the application layer.  Implementing per-packet inter-modal synchronization at RAN would require additional signalling and processing, increased complexity, increased resource consumption without a corresponding improvement in user experience.

Given that receiving packets for some modalities is preferred than receiving none at all as this enables the application to use the received modalities to better manage the user experience, the coordination of discarding packets across multi-modal flows is not needed if the multi-modal flows are distributed to dual connectivity paths. Applications are capable of adapting to partial data losses in specific modalities. For instance: If video packets are lost on the MN, the application can continue using the received audio packets from the SN. On the other hand, requiring MN and SN to coordinate packet discarding could introduce additional signaling overhead and processing delays.

From RAN3 perspective, joint admission control and QoS flow to DRB mapping by using multi-modality information (e.g., MMSID) is feasible, and shall take the context into consideration. Simply admitting all or none is not a wise solution.

From RAN3 perspective, per packet inter-modal synchronization is complex to implement without expected user experience improvement.

Joint packet discard is not needed for multi-modal flows transmitted on different DRBs.

# Conclusions

In this submission, we discussed multi-modality awareness at RAN. The following proposals are made:

- From RAN3 perspective, joint admission control and QoS flow to DRB mapping by using multi-modality information (e.g., MMSID) is feasible, and shall take the context into consideration. Simply admitting all or none is not a wise solution.

From RAN3 perspective, per packet inter-modal synchronization is complex to implement without expected user experience improvement.

Joint packet discard is not needed for multi-modal flows transmitted on different DRBs.

# References

R3-250027, LS Reply on multi-modality awareness