3GPP TSG-RAN WG3 Meeting #127	R3-250528

Athens, Greece, 17-21 Feb 2025

Agenda Item:	22.2

Source:	Ericsson

Title:	QoS handling for Multi-hop Relay

Document for:	Discussion, Agreement

# Introduction

NR Sidelink Multi-hop WI has started in Rel-19 to support multi-hop Layer-2 and Layer-3 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities.

This document discusses the potential issue and provides an analysis of considerations for QoS handling.

# Discussion

As mentioned in [1], intermediate UEs in RRC_INACTIVE should be supported in Rel-19. In that case, both Sidelink Layer-2 and Layer-3 multi-hop U2N relaying will be covered. To guarantee end-to-end QoS, proper network handling will be required by the network. A key challenge remains in determining how to perform QoS splitting along the path between the Relay UE and the U2N Remote UE, especially when an intermediate relay UE is in the RRC_INACTIVE state.

- QoS handling needs to be considered for Multi-hop operation.

End-to-end QoS in relaying is managed by splitting QoS requirements, such as latency, among different hops. In case if all the relay UEs are in RRC_CONNECTED state, the gNB performs QoS splitting, similar to the Rel-17 U2N relay setup. In another case, if only the last relay UE is in RRC_CONNECTED state, the network determines the QoS split for the Uu hop, ensuring efficient resource management.

A key question remains on how to allocate QoS among PC5 hops between the last relay UE and the remote UE. Two options are considered:

(1) The network manages QoS splitting, covering DRB and PC5 link QoS across all hops.

(2) The relay UE manages its own QoS splitting, extending the U2U relay mechanism.

The preferred approach is for the network to control QoS allocation, ensuring consistency and stability during multi hops.

- RAN3 to support network splitting the QoS among the UEs for Multi-hop Relay.

# Conclusions and Proposals

In the previous sections we made the following observations and proposals:

Observation 1	QoS handling needs to be considered for Multi-hop operation.

In this paper we propose:

Proposal 1	RAN3 to support network splitting the QoS among the UEs for Multi-hop Relay.

# References

[1] R3-250527, Authorization of Intermediate Relay UEs in RRC_INACTIVE, Ericsson