3GPP TSG-RAN WG3#127 	R3-250529

Athens, Greece, February 17-21, 2025

Agenda item:	13.2 Support for inter-CU LTM

Source:	Jio Platforms (JPL)

Title:	Discussion on Inter-CU LTM Dual Connectivity

Document for:	Discussion and Decision

# 1	Introduction

As part of the ongoing standardization efforts for NR mobility enhancements in Release 19, the Inter-CU Layer 1/Layer 2 Triggered Mobility (LTM) framework plays a crucial role in ensuring seamless mobility across multiple gNB-CUs and distributed architectures. This document proposes an integrated solution that enhances the existing LTM framework by focusing on:

- Dynamic resource management and candidate cell reconfiguration.

- Optimized synchronization and handover execution mechanisms.

- Efficient handling of Timing Advance (TA) for accurate handover execution.

- Minimization of UE service interruption by reducing unnecessary RRC reconfiguration.

- Refinements for Inter-CU LTM support in NR Dual Connectivity (DC).

- Enhancements in PDU session continuity, data forwarding, and security key handling.

The proposed modifications are derived from:

- Existing vendor contributions (Rakuten, Qualcomm, Nokia, Intel).

- Observed inefficiencies in Inter-CU LTM signaling flows (Figures 1 and 2).

- Identified gaps in security, TA synchronization, and dual connectivity (NR-DC) coordination.

These enhancements ensure that 3GPP Release 19 provides a robust, low-latency, and scalable mobility framework.

Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3]

- Prioritize the case when CU is acting as MN when DC is not configured

- When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:

As secondary priority, support the case where CU is acting as SN and MN is unchanged

As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

- Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

- Coordination with SA3 needed with respect to security key handling

Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Early data forwarding can be supported for inter-CU LTM.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own.

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation.

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

The LTM Configuration IDs are allocated by the source CU.

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

Confirm the name of the new procedure as “LTM Configuration Update”.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM.

RAN3#126:

RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution.

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB.

Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation?

Continue to work on the solutions comparison and details…

# 2	Discussion on Inter-CU LTM

The Inter-CU Layer 1/Layer 2 Triggered Mobility (LTM) framework plays a crucial role in ensuring seamless mobility across multiple gNB-CUs and distributed architectures. This document proposes an integrated solution that enhances the existing LTM framework by focusing on:

- Dynamic resource management and candidate cell reconfiguration.

- Optimized synchronization and handover execution mechanisms.

- Efficient handling of Timing Advance (TA) for accurate handover execution.

- Minimization of UE service interruption by reducing unnecessary RRC reconfiguration.

- Refinements for Inter-CU LTM support in NR Dual Connectivity (DC).

- Enhancements in PDU session continuity, data forwarding, and security key handling.

The proposed modifications are derived from:

- Existing vendor contributions (Rakuten, Qualcomm, Nokia, Intel).

- Observed inefficiencies in Inter-CU LTM signaling flows (Figures 1 and 2).

- Identified gaps in security, TA synchronization, and dual connectivity (NR-DC) coordination.

These enhancements ensure that 3GPP Release 19 provides a robust, low-latency, and scalable mobility framework for future 5G-Advanced and 6G networks.

2.1 LTM Preparation Phase Optimization

- The handover request exchange between Source CU and Target CU lacks efficient Timing Advance (TA) synchronization mechanisms.

- UE context identification and UE context setup are processed sequentially, adding unnecessary latency.

- LTM candidate cell preparation does not account for resource availability updates in real-time.

- Proposal 1:

  - Optimize UE Context Identification and Context Setup Requests:

  - The UE Context Identification Request parameters (e.g., UE Capabilities, Security Context) shall be included directly within the Handover Request message (3GPP TS 38.423 XnAP). This eliminates sequential messaging and ensures the target CU receives full UE context before admission control

  - This reduces latency and ensures that the target CU has full UE context before processing admission control.

- Proposal 2:

- The TA Configuration in Handover Request/Acknowledge messages shall include:

  - TA values derived from UE uplink SRS measurements

  - A validity timestamp to account for dynamic channel conditions.

  - TA Information List (new IE in TS 38.331) containing RSRP-based TA adjustments for candidate cells."

- Proposal 3:

  - The target CU shall execute admission control and LTM Resource Configuration Updates (e.g., CSI-RS, resource blocks) in parallel, using a shared resource reservation state to prevent conflicts (3GPP TS 38.300)Instead of sequential processing, the target CU should handle admission control while preparing CSI-RS and resource configurations simultaneously

2.2 LTM Execution Phase Enhancements

- Cell Switch Notification lacks necessary parameters for seamless transition.

- Data forwarding happens sequentially, leading to delays.

- Security key refresh must be optimized to handle PDCP anchor changes efficiently.

- Proposal 4:

  - The Cell Switch Notification message shall include:

    - UE Timing Advance with RSRP validation.

    - Target Cell Beamforming Parameters (e.g., CSI-RS Config, SSB Index).

    - Validity Window for synchronization parameters

- Proposal 5:

  - The Cell Switch Notification message shall include:UE Timing Advance with RSRP validation.Target Cell Beamforming Parameters (e.g., CSI-RS Config, SSB Index).Validity Window for synchronization parameters .

  - Enable Parallel Data Forwarding and LTM Cell Switch Execution.

  - Currently, data forwarding occurs only after full UE context transfer.

  - We propose that data forwarding begins as soon as Timing Advance and security key updates are synchronized.

- Proposal 6:

  - Optimize Security Key Refresh Mechanisms:

  - For Inter-CU LTM with PDCP anchor change, use:

    - Secure RRC signaling for key refresh.

    - Candidate gNBs to perform early rekeying based on UE association update.

  - This ensures zero-delay security updates without requiring additional RRC reconfiguration.

2.3 Enhancements for Dual Connectivity (DC)

- NR-DC LTM requires improved Master Node (MN) and Secondary Node (SN) coordination.

- Simultaneous Inter-CU LTM in MCG and SCG should be avoided.

- PSCell transitions should be based on the latest UE measurements.

- Proposal 7:

  - Introduce Inter-CU LTM Coordination Between MN and SN:

  - The MN should explicitly signal Inter-CU MCG LTM configuration to the SN to avoid simultaneous triggering.

  - New Xn signaling should be introduced to exchange real-time MN-SN mobility state.

- Proposal 8:

  - PSCell Selection Based on Latest UE Measurements:

  - Instead of pre-configuring PSCell changes, UE should report latest radio conditions before finalizing PSCell handover.

- Proposal 9:

  - Enable Dual Connectivity Adaptation During LTM Execution:

  - If an Inter-CU MCG LTM triggers SCG release, the next LTM operation should have the option to re-add SCG based on delta configuration updates.

Figure 1 Inter-CU LTM

Figure 2 NR DC coordination

# 3	Conclusion

1. Proposal 1:

- Optimize UE Context Identification and Context Setup Requests by bundling UE Context Identification Requests within the initial Handover Request Message.

- This reduces latency and ensures that the target CU has full UE context before processing admission control.

2. Proposal 2:

- Include Timing Advance (TA) Configuration in Handover Request/Acknowledge Messages to allow the target gNB to optimize early synchronization.

- The TA Information List (including RSRP values) should be forwarded in UE Context Setup Response.

3. Proposal 3:

- Enable parallel execution of admission control and LTM configuration updates instead of sequential processing.

- The target CU should handle admission control while preparing CSI-RS and resource configurations simultaneously to reduce LTM preparation delay.

4. Proposal 4:

- Enhance the Cell Switch Notification Message by including UE Timing Advance values (RSRP-based validation), PDU Session Resource Information of all candidate cells, and CSI-RS resource configuration updates.

- This allows better synchronization and preparation before the actual switch.

5. Proposal 5:

- Enable parallel data forwarding and LTM cell switch execution by allowing data forwarding to begin as soon as Timing Advance and security key updates are synchronized.

- This improves efficiency by overlapping execution steps.

6. Proposal 6:

- Optimize Security Key Refresh Mechanisms for Inter-CU LTM with PDCP anchor change by using secure RRC signaling for key refresh.

- Candidate gNBs should perform early rekeying based on UE association updates to ensure zero-delay security updates without requiring additional RRC reconfiguration.

7. Proposal 7:

- Introduce Inter-CU LTM Coordination Between MN and SN, where the MN explicitly signals Inter-CU MCG LTM configuration to the SN to avoid simultaneous triggering.

- Introduce new Xn signaling to exchange real-time MN-SN mobility state.

8. Proposal 8:

- Ensure PSCell selection is based on the latest UE measurements rather than pre-configured changes.

- UE should report the latest radio conditions before finalizing PSCell handover.

9. Proposal 9:

- Enable dual connectivity adaptation during LTM execution by allowing SCG re-addition after SCG release in an Inter-CU MCG LTM scenario.

- The next LTM operation should have the option to re-add SCG based on delta configuration updates.