3GPP TSG-RAN WG3 Meeting #127	R3-250531

Athens, Greece, Feb. 17th – Feb. 21st, 2025

Agenda item:	12.3

Source:	Jio Platforms (JPL)

Title:	[TP for BL CR NR Femto 38.300] Access Control for NR Femto

Document for:	Discussion and Decision

# 1	Introduction

This contribution introduces enhancements for access control mechanisms in NR Femto, focusing on improved CAG handling and hybrid access strategies. These updates are based on discussions and alignment with previous contributions and standardization work.

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 4.X.X Access Control for NR Femto

4.X.X.1 Overview

The extended access control framework for NR Femto aims to optimize network security and accessibility through advanced Closed Access Group (CAG) handling and hybrid access strategies. These enhancements ensure that NR Femto nodes can be efficiently managed in different deployment scenarios, including public, private, and hybrid network environments.

Figure 1 NR Femto Access Control Architecture

4.X.X.2 Access Control Modes

Cells served by an NR Femto node SHALL support the following access modes to enable backward compatibility with legacy UEs while restricting access for private network (PNI-NPN) scenarios:

- Open Access (PLMN-only):

  - The NR Femto broadcasts only plmn-IdentityInfoList in SIB1.

  - Legacy UEs (not supporting CAG) and CAG-capable UEs can access the cell as a normal PLMN cell.

- Hybrid Access (PLMN + CAG):

Hybrid access implementation provides dynamic access control based on network conditions and policy configurations. The NR Femto dynamically adjusts:

  - The NR Femto broadcasts both plmn-IdentityInfoList and npn-IdentityInfoList-r16 in SIB1 without setting cellReservedForOtherUse to "true".

  - CAG-capable UEs supporting the broadcasted CAG-ID(s) SHALL prioritize CAG-based access.

  - Legacy UEs (non-CAG-capable) MAY access the cell as a normal PLMN cell, subject to operator policies (e.g., access barred if cellReservedForOperatorUse is configured).

  - Prioritization of CAG UEs: Ensuring CAG members receive preferential access.

  - Conditional Access for Non-CAG UEs: Allowing limited access under predefined conditions.

  - Adaptive Transition Between Modes: Switching between Open and Closed Access dynamically based on congestion levels and network demand.

- Closed Access (CAG-only):

  - The NR Femto broadcasts npn-IdentityInfoList-r16 and sets cellReservedForOtherUse to "true".

  - Only UEs with a matching CAG-ID in their allowed list can access the cell.

  - Legacy UEs are explicitly barred from accessing the cell.

NOTE 1: For hybrid access, operators MAY use cellReservedForOperatorUse or cellBarred to restrict legacy UEs if needed.

NOTE 2: CAG-capable UEs SHALL ignore PLMN-based access if a valid CAG-ID is broadcasted and included in their subscription.

The CAG mechanism restricts access to a predefined set of UEs, ensuring controlled connectivity. The NR Femto implements CAG access as follows:

1. UE Registration: A UE attempting to access the NR Femto sends a registration request.

2. CAG Authorization: The NR Femto queries the Access Mode Decision Engine to verify the UE’s CAG status.

3. Subscription Validation: The system cross-references the UE’s allowed CAG list stored in the network database.

4. Access Decision: Based on verification, the Access Mode Decision Engine grants or denies access.

4.X.X.3 NR Femto Gateway Role in Access Control

When NR Femtos connect through an NR Femto Gateway (NR Femto GW), the gateway plays a crucial role in enforcing access control policies:

- Centralized Authentication Processing: The NR Femto GW acts as an intermediary for authentication requests.

- NG-Flex Support: Enables simultaneous connection to multiple AMFs, ensuring load balancing.

- Policy Enforcement Coordination: Manages access control rules dynamically across multiple NR Femtos.

4.X.X.4 PNI-NPN Deployment Considerations

Cells served by an NR Femto node may be deployed as part of a **Public Network Integrated - Non-Public Network PNI-NPN)** (see clause 4.8) in order to restrict access to UEs according to the respective subscription. This enables flexible network operation where private network features are available within a public network framework.

NOTE: The NR Femto node may use the CAG mechanism for PNI NPN (see clause 16.7.4) as follows:

- 	PLMN Cell Activation: The NR Femto node may activate a PLMN cell, which can be accessed by legacy UEs without CAG access control.

- 	Shared PLMN and CAG Cell Activation: The NR Femto may activate a cell shared by both PLMN and CAG by broadcasting both the `plmn-IdentityInfoList` and the `npn-IdentityInfoList-r16` in SIB1, while excluding cellReservedForOtherUse`. This cell is accessible to UEs whose allowed CAG list includes a broadcasted CAG-ID. Legacy UEs that do not support CAG will perceive this as a standard PLMN cell.

- 	NPN-Only Cell Activation: The NR Femto node may activate an **NPN-only cell** by broadcasting the cellReservedForOtherUse` IE with a value of `true`. This ensures that only UEs with a valid CAG-ID in the allowed CAG list can access the cell.

4.X.X.5 Xn Connectivity for Coordinated Access Control

Xn connectivity between NR Femtos and gNBs facilitates:

- Seamless UE Handover: Ensuring smooth transition of UEs between NR Femto and macro cells.

- CAG List Synchronization: Sharing access control data across interconnected NR Femtos.

- Inter-Femto Coordination: Allowing NR Femtos to dynamically adjust access control policies based on mobility and network conditions.

4.X.X.6 Protocol and Interface Considerations

To support the extended access control mechanisms, the following protocol enhancements are introduced:

- NG-C Interface Enhancements: Improved signaling interactions for access verification.

- NG-U Interface Considerations: Efficient handling of data transport for hybrid access scenarios.

- Xn-Based Control Signaling: Facilitating inter-Femto coordination for access management.

### 4.X.X Access Control for NR Femto

4.X.X.1 Access Control Modes

The extended access control framework for NR Femto aims to optimize network security and accessibility through Closed Access Group (CAG) handling and hybrid access strategies. This ensure that NR Femto nodes can be efficiently managed in different deployment scenarios, including public, private, and hybrid network environments.

NR Femto nodes support multiple access control modes, ensuring flexibility for different deployment scenarios:

- Open Access Mode: Any UE within coverage can connect to the NR Femto without restrictions.

NOTE: Legacy UEs (non-CAPABLE_CAG) perceive the cell as a normal PLMN cell. Ensure cellReservedForOtherUse

- Closed Access Mode: Only UEs explicitly authorized via CAG lists can connect to the NR Femto.

- Hybrid Access Mode: Combines both Open and Closed modes, allowing prioritized access to CAG-registered UEs while providing limited access to others.

4.X.X.2 Closed Access Group (CAG) Handling

The CAG mechanism restricts access to a predefined set of UEs, ensuring controlled connectivity. The NR Femto implements CAG access as follows:

- UE Registration: UE includes CAG-ID in RegistrationRequest A UE attempting to access the NR Femto sends a registration request.

- AMF/UDM Validation: AMF checks UE’s allowed CAG list (subscriber data in UDM).

- NR Femto Role: Acts as a RAN node; does not perform CAG authorization.

3. CAG Authorization: The NR Femto queries the Access Mode Decision Engine to verify the UE’s CAG status.

4. Subscription Validation: The system cross-references the UE’s allowed CAG list stored in the network database.

5. Access Decision: Based on verification, the it grants or denies access.

Figure 1 NR Femto Access Control Architecture

4.X.X.3 Hybrid Access Implementation

Hybrid access implementation provides dynamic access control based on network conditions and policy configurations. The NR Femto dynamically adjusts:

- Prioritization of CAG UEs: Ensuring CAG members receive preferential access.

- Conditional Access for Non-CAG UEs: Allowing limited access under predefined conditions.

- Adaptive Transition Between Modes: Switching between Open and Closed Access dynamically based on congestion levels and network demand.

4.X.X.4 NR Femto Gateway Role in Access Control

When NR Femtos connect through an NR Femto Gateway (NR Femto GW), the gateway plays a crucial role in enforcing access control policies:

- Centralized Authentication Processing: Authentication is handled by AUSF/UDM..

- NG-Flex Support: Enables simultaneous connection to multiple AMFs.

- Policy Enforcement Coordination: Manages access control rules dynamically.

4.X.X.5 PNI-NPN Deployment Considerations

Cells served by an NR Femto node  be deployed as part of a **Public Network Integrated - Non-Public Network PNI-NPN)** (see clause 4.8) in order to restrict access to UEs according to the respective subscription.

NOTE: The NR Femto node use the CAG mechanism for PNI NPN (see clause 16.7.4) as follows:

- 	PLMN Cell Activation: The NR Femto node activate a PLMN cell, which can be accessed by legacy UEs without CAG access control.

- 	Shared PLMN and CAG Cell Activation: The NR Femto  activate a cell shared by both PLMN and CAG by broadcasting both the `plmn-IdentityInfoList` and the `npn-IdentityInfoList-r16` in SIB1, while excluding cellReservedForOtherUse`. This cell is accessible to UEs whose allowed CAG list includes a broadcasted CAG-ID. Legacy UEs that do not support CAG will perceive this as a standard PLMN cell.

- 	CAG-Only Cell Activation: The NR Femto node  activate an CAG-only cell by broadcasting the cellReservedForOtherUse` IE with a value of `true`. This ensures that only UEs with a valid CAG-ID in the allowed CAG list can access the cell.

4.X.X.6 Xn Connectivity for Coordinated Access Control

Xn connectivity between NR Femtos and gNBs facilitates:

- Seamless UE Handover: Ensuring smooth transition of UEs between NR Femto and macro cells.

- Inter-Femto Coordination: Allowing NR Femtos to dynamically adjust access control policies based on mobility and network conditions.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>