3GPP TSG-RAN WG3 #127	R3-250540

Athens, GR, 17 – 21 Feb. 2025

Agenda Item:	13.2. Support for inter-CU LTM

Source:	NTT DOCOMO INC.

Title:	Discussion on inter-CU LTM DC case

Document for:	Discussion, agreement

Introduction

At the last meeting, some papers for inter-CU LTM with DC case were submitted, but they were not treated. In the WID [1], following are stated:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured

  - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:

    - As secondary priority, support the case where CU is acting as SN and MN is unchanged

    - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

Also, we received LSs [2][3] from RAN2 and they include some agreements on LTM DC cases.

RAN2#127

Agreements on inter-CU LTM

…

Inter-CU SCG LTM:

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

…

RAN2#128

Agreements on inter-CU LTM:

…

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

…

8.	RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

9.	RAN2 to support intra-CU MCG LTM with SCG configuration.

…

In this contribution, we discuss open points to be discussed for support of inter-CU SCG and MCG LTM.

Discussion

Scenarios

Based on the RAN2 agreements, the scenarios to be discussed as LTM DC cases can be separated in SCG LTM and MCG LTM and supported scenarios are summarized as follows:

- From agreement 8 in RAN2#128, SCG LTM includes SN initiated intra-CU SCG LTM and MN initiated intra-CU SCG LTM.

- From agreement 14 in RAN2#127, only SN initiated inter-CU SCG LTM is supported.

- From agreement 5 and 9 in RAN2#128, intra and inter CU MCG LTM configuration can contain SCG configuration and SCG can be changed, but it cannot be selected separately from MCG via MAC CE.

- From agreement 4 in RAN2#128, inter-CU MCG LTM with SCG addition is also supported, but same as previous one, SCG cannot be selected separately from MCG.

From operator perspective, PSCell addition without MCG change should be also included in the scope because we see that this scenario is very important to utilize SCGs with higher frequencies in real deployment. In this contribution, open issues in each scenario are discussed.

- MN/SN initiated intra-CU SCG LTM

In our understanding, in release 18, intra-CU SCG LTM without MN involvement was supported. Therefore, what we should support in release 19 are MN initiated intra-CU SCG LTM and SN initiated intra-CU SCG LTM with MN involvement. For this case, MN/SN initiated SN Modification procedure (with MN involvement) can be a baseline. Enhancement on SN Modification procedure to let SN to prepare LTM SCG configuration and enhancement on SN Modification Required/Confirm procedure to enable to SN to request intra-CU SCG LTM to MN. This would involve F1 signalings since if SCG LTM is configured with MN format, source DU at MN needs to CSI-report configuration. However, this will not involve any enhancement on F1.

- SN initiated inter-CU SCG LTM

For this case, SN initiated conditional SN Change procedure can be a base line. Also, enhancement on SN Change Required/Confirm procedure to trigger inter-SN SCG LTM preparation, enhancement on SN Addition procedure to let candidate SN(s) to prepare LTM SCG configurations and enhancement on SN Modification procedure to notify updated SCG LTM configuration to source SN.

- Intra and inter CU MCG LTM with SCG

This case includes intra/inter CU MCG LTM with SCG change/addition. For intra/inter-CU MCG LTM with SCG change, Inter-gNB-DU Conditional Handover and Conditional Handover with Secondary Node procedures can be a baseline. Handover Request procedure and LTM Configuration Update procedure from non-DC LTM can be reused, and enhancement for SN Addition/Modification procedure is needed to enable to let SNs to prepare LTM SCG configurations. Here, this procedure is always initiated by MN.

Proposal 1:	To support MN initiated intra-CU SCG LTM and SN initiated intra-CU SCG LTM with MN involvement, following enhancements are needed:

- SN Modification procedure to let SN to prepare LTM SCG configuration

- SN Modification Required/Confirm procedure to enable to SN to request intra-CU SCG LTM to MN

Proposal 2:	To support SN initiated inter-CU SCG LTM, following enhancements are needed:

- SN Change Required/Confirm procedure to trigger inter-SN SCG LTM preparation

- SN Addition procedure to let candidate SN(s) to prepare LTM SCG configuration(s)

- SN Modification procedure to notify updated SCG LTM configuration to source SN

Proposal 3:	To support intra/inter-CU MCG LTM with SCG, following enhancement is needed:

- SN Addition/Modification procedure to enable to let SN(s) to prepare LTM SCG configurations (these enhancement would be same as for other DC cases)

SN security key handling

SN security key handling in the Inter-CU SCG LTM can support the subsequent LTM by reusing the solution supported in the subsequent CPAC. That is, the list of sk counter is pre-configured for the UE in the LTM preparation phase. Also, the source MN generates the list for all candidate SNs and forwards it to all candidate SNs.

SN security key handling in the Inter-CU MCG LTM cannot use the solution in the subsequnent CPAC when the MN changes (i.e., when the KgNB changes). This is because when the KgNB changes, the KSN must also change, so unless a list of future KgNBs is known in advance, it is not possible to generate configuration for KSN changes in advance. In other words, how to refresh the SN security key depends on how security key handling is performed in the Inter-CU LTM non-DC case, and this point is still under discussion, so that it should be suspended.

Proposal 4:	For inter-CU SCG LTM without MCG change, SN security key refresh mechanism for subsequent CPAC can be reused.

Proposal 5:	For inter-CU MCG LTM without SCG change, SN security key refresh mechanism would involve gNB security key refresh, and this is under discussion on other WG and not finished, so this point should be suspended.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	To support MN initiated intra-CU SCG LTM and SN initiated intra-CU SCG LTM with MN involvement, following enhancements are needed:

- SN Modification procedure to let SN to prepare LTM SCG configuration

- SN Modification Required/Confirm procedure to enable to SN to request intra-CU SCG LTM to MN

Proposal 2:	To support SN initiated inter-CU SCG LTM, following enhancements are needed:

- SN Change Required/Confirm procedure to trigger inter-SN SCG LTM preparation

- SN Addition procedure to let candidate SN(s) to prepare LTM SCG configuration(s)

- SN Modification procedure to notify updated SCG LTM configuration to source SN

Proposal 3:	To support intra/inter-CU MCG LTM with SCG, following enhancement is needed:

- SN Addition/Modification procedure to enable to let SN(s) to prepare LTM SCG configurations (these enhancement would be same as for other DC cases)

Proposal 4:	For inter-CU SCG LTM without MCG change, SN security key refresh mechanism for subsequent CPAC can be reused.

Proposal 5:	For inter-CU MCG LTM without SCG change, SN security key refresh mechanism would involve gNB security key refresh, and this is under discussion on other WG and not finished, so this point should be suspended.

References

- RP-242356, WID “NR mobility enhancements Phase 4”

- R3-250008, “RAN2 agreements on Inter-CU LTM and Conditional LTM”, RAN2

- R3-247111, “LS on RAN2 agreements for inter-CU LTM”, RAN2