3GPP TSG-RAN WG3 Meeting 127	R3-250547

Athens, Greece, Feb. 17th – Feb. 21st, 2025

Agenda Item:	12.2

Source: 	Jio Platforms (JPL)

Title:	Discussion on WAB enhancements

Document for:	Discussion

# Introduction

A release 19 work item “New WID on additional topological enhancements for NR”[1] was approved in RAN#105.

# 2. Discussion

### 2.1 Multi-hop Prevention

Multi-hop WAB topologies should be prevented through optimized procedures to ensure efficient network operation, minimize latency, and prevent unintended handovers between WAB nodes. Allowing multi-hop WAB topologies could lead to inefficient routing, increased backhaul traffic, and potential degradation of service quality. Therefore, specific mechanisms need to be enforced at both the RAN and network levels to avoid these issues.

- Proposal 1-1: WAB-gNB should use dedicated frequencies and/or Physical Cell Identifiers (PCIs) to prevent WAB-MTs from selecting WAB-gNB cells. By designating specific PCI ranges or frequency bands for WAB-gNBs, operators can ensure clear separation between access and backhaul nodes. This method helps prevent interference and unintentional reselection of WAB-gNBs by WAB-MTs, ensuring a strict hierarchical connectivity model.

- Proposal 1-2: The WAB-gNB should validate the S-NSSAI provided by the WAB-MT in the RRCSetupComplete message and reject connections if the S-NSSAI does not match the backhaul slice. By incorporating a dedicated Single Network Slice Selection Assistance Information (S-NSSAI), WAB-MTs can be explicitly restricted from attaching to WAB-gNBs that are primarily used for backhaul. This approach aligns with existing 5G network slicing principles, ensuring that unauthorized WAB-MTs do not establish connectivity with WAB-gNBs that are designated exclusively for backhaul operations.

- Proposal 1-3: A new indicator in System Information Block (SIB) should be introduced to explicitly prevent WAB-MTs from selecting WAB-gNBs. This indicator should be broadcast within the SIB messages of WAB-gNBs to signal to WAB-MTs that the cells are not intended for direct UE attachment. The implementation of this mechanism requires minimal modification to existing SIB structures and can be dynamically controlled by the Operations, Administration, and Maintenance (OAM) system. This will allow network operators to adjust WAB restrictions based on deployment needs and topology changes dynamically.

- Proposal 1-4: The WAB-gNB should reject Handover (HO) preparation requests that include an S-NSSAI used for backhauling. This ensures that WAB-MTs do not mistakenly attempt to handover to a WAB-gNB intended for backhaul. The handover rejection can be implemented by incorporating filtering mechanisms within the Handover Request and Handover Command messages exchanged between the source and target gNBs. When an S-NSSAI used for backhaul is detected in an incoming HO request, the target WAB-gNB should immediately reject the request, preventing unintended topology formations.

2.2 Xn and NG Connection Management

2.2.1 Xn Connection Management

The Xn interface plays a crucial role in WAB deployments, enabling inter-gNB coordination and facilitating seamless handovers between WAB-gNBs and BH-RAN nodes. The efficient management of Xn connections ensures that WAB nodes maintain appropriate interconnectivity without unintended configurations that could compromise network stability.

- Proposal 2-1: WAB cell-specific information should be included in Xn Setup Request messages to prevent unintended Xn connections between WAB-gNBs.

- The Xn Setup Request message should include an identifier or flag indicating whether the originating gNB is a WAB-gNB.

- This mechanism prevents unnecessary peer-to-peer Xn connections between different WAB-gNBs, which could lead to inefficiencies and suboptimal routing within the network.

- By enforcing these restrictions, network operators can ensure that Xn connections align with the predefined topology constraints for WAB deployments.

Proposal 2-2: The Xn Removal Request should include a cause value indicating the reason for Xn disconnection.

- When an Xn connection is removed, the initiating node should specify a cause value in the Xn Removal Request message.

- This helps in diagnostic analysis and ensures that WAB-gNBs, BH-RAN nodes, and network controllers understand the disconnection rationale.

2.2.2 NG Connection Management

The NG interface facilitates communication between WAB-gNBs and the 5G Core Network (5GC), handling signaling and user-plane data exchange. Proper management of NG connections is essential for maintaining service continuity, security, and optimized resource allocation within a WAB deployment.

- Proposal 2-3: Introduce a "WAB-gNB" indication in the NG SETUP REQUEST message.

- The NG SETUP REQUEST message is a crucial step in establishing a control plane connection between a WAB-gNB and the Access and Mobility Management Function (AMF).

- A WAB-specific indicator should be included in this message to:

- Differentiate WAB-gNBs from traditional gNBs.

- Allow the AMF to apply WAB-specific policies, optimizing session management and security procedures.

- Enable the 5GC to track and manage WAB deployments distinctly from standard NR infrastructure.

2.3 Mobility Handling

Mobility handling in Wireless Access Backhaul (WAB) scenarios is a crucial aspect of ensuring seamless connectivity, minimal service disruption, and optimized handovers between WAB-gNBs and Backhaul RAN (BH-RAN) nodes. The mobility procedures should be designed to handle transitions efficiently, ensuring that UEs connected to a WAB-gNB do not experience service degradation when the WAB-gNB itself undergoes mobility changes.

Since a WAB-gNB is inherently mobile in certain deployment scenarios (e.g., vehicle-mounted WAB node, its movement necessitates dynamic adjustments in backhaul connectivity, UE anchor points, and AMF associations. As a result, WAB-specific mobility mechanisms must be implemented to facilitate handover coordination, AMF change, and backhaul topology updates.

### Proposal 3-1: Prioritization of the Two Logical WAB-gNB Solution

One of the most efficient approaches to handle WAB-gNB mobility is to adopt the two logical WAB-gNB solution. This solution ensures that:

- A WAB-gNB maintains a logical separation between its own movement and the movement of the UEs it serves.

- Mobility procedures are executed in a manner that minimizes disruptions to ongoing UE sessions.

Figure 1WAB Mobility Handling

# Conclusion

- Proposal 1-1: WAB-gNBs should use dedicated frequency bands and/or reserved Physical Cell Identifier (PCI) ranges to prevent WAB-MTs from selecting WAB-gNB cells.

- Proposal 1-2: The WAB-gNB should validate the S-NSSAI provided by the WAB-MT in the RRCSetupComplete message and reject connections if the S-NSSAI does not match the backhaul slice.

- Proposal 1-3: Introduce a new SIB indicator to prevent WAB-MTs from selecting WAB-gNBs, dynamically controlled by OAM.

- Proposal 1-4: WAB-gNB should reject HO preparation requests that include an S-NSSAI used for backhauling to prevent unintended topology formations.

- Proposal 2-1: WAB cell-specific information should be included in Xn Setup Request messages to prevent unintended Xn connections between WAB-gNBs.

- Proposal 2-2: The Xn Removal Request should include a cause value indicating the reason for Xn disconnection to aid in diagnostics and troubleshooting.

- Proposal 2-3: Introduce a "WAB-gNB" indication in the NG SETUP REQUEST message to differentiate WAB-gNBs from traditional gNBs and enable WAB-specific policies.

- Proposal 3-1: Prioritize the two logical WAB-gNB solution to maintain a logical separation between WAB-gNB mobility and UE movement.