3GPP TSG-RAN WG3 Meeting #127	R3-250549

Athens, Greece, February 17th – 21st 2025

Agenda Item:	17.3

Source:	Ericsson, Deutsche Telekom, China Unicom

Title:	Xn impact of On-demand SIB1 for UEs in idle/inactive mode

Document for:	Other

# Introduction

A work item on “Enhancements of network energy savings for NR” was agreed in [1]. The Work Item Description (WID) was revised in [2]. According to the revised WID, the objectives of the work item are the following:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

In this contribution, we discuss issues and solutions regarding Objective 2 of the work item (see highlighted text above), focusing on the impact on the Xn interface.

# Discussion

In the following, we discuss the main principles of the inter NG-RAN node signaling to enable on-demand SIB1 (OD-SIB1) transmission, considering non-split architecture.

## General scenario

Figure 1 illustrates an example deployment scenario for OD-SIB1 transmission, where the “NES Cell” of gNB1 is meant to operate in OD-SIB1 transmission mode and the “Cell A” of gNB2 provides the UL Wake-Up Signal (WUS) configuration (that is needed to request SIB1 transmission in the NES Cell) to UEs. The scenario corresponds to Case 2 as defined by RAN1, assumed for normative work according to the WID.

Figure 1: An example deployment scenario for OD-SIB1 transmission (corresponding to Case 2).

We assume that, before the NES Cell operates in OD-SIB1 transmission mode, gNB1 provides the UL WUS configuration of its NES Cell to gNB2 in step 1. In step 2, gNB2 provides the UL WUS configuration of the NES Cell to UEs camping on (or connected to) Cell A. In step 3, when needed, a UE (in RRC Idle/Inactive mode) sends the UL WUS to the NES Cell and, in response, receives SIB1 from the NES Cell in step 4.

## Previous agreements

At the RAN3#125 meeting, the following was agreed [3]:

Case 2 can be supported with the below details:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

Moreover, at the RAN3#125-bis meeting, the following agreements were captured [4]:

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

Finally, at the RAN3#126 meeting, the following agreements related to the new procedures were captured [5]:

The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

The procedure uses non UE-associated signaling.

Cell A stores the UL WUS configuration information after it has received it.

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The procedure uses non UE-associated signalling.

Hence, RAN3 has agreed to introduce a new Class 1 procedure for signaling the UL WUS configuration from NES-Cell gNB to Cell-A gNB, and that the Cell-A gNB shall provide feedback to the NES-Cell gNB, e.g., accept/reject the UL WUS configuration provisioning. In addition, it was agreed that the Cell-A gNB can stop providing the UL WUS configuration and that the Cell-A gNB shall signal to the NES-Cell gNB about this interruption using a new Class 2 procedure.

## Further discussion on the new procedures

### Design of the new Class 1 procedure as start/stop request

Despite recent agreements on the new Class 1 XnAP procedure, its details are still to be specified.

As agreed, the new Class 1 procedure over Xn will carry the UL WUS configuration information. The UL WUS configuration information is exchanged over Xn only when the NES-Cell gNB intends to put the NES Cell into OD-SIB1 operation and cells of another gNB need to assist.

We propose to design the procedure as a start/stop request, sent by the NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The Cell-A gNB can accept or reject the “start” request. The Cell-A gNB cannot reject the “stop” request, which is similar to, e.g., the PDU session resource release procedure (where the “release” will never fail). One way to distinguish accept and reject is to introduce different response messages over Xn, e.g., a “response” message and a “failure” message, as assumed in the TP below. Another way is to define a dedicated IE in the response message. We favor the first option.

RAN3 to agree the design of the new Class 1 procedure as a start/stop request, sent by a NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The “start” request can be accepted or rejected by the Cell-A gNB. The “stop” request is always successful.

Some companies prefer to use the new Class 1 XnAP procedure to signal the UL WUS configuration information without triggering the start of the provisioning at the Cell-A gNB. A second message from the NES-Cell gNB, without the UL WUS configuration information, would be needed to start the provisioning. This approach could be beneficial if the NES-Cell gNB activates and deactivates the OD-SIB1 operation frequently, since the UL WUS configuration information is only sent once. However, we do not think that NES Cells would go into and out of OD-SIB1 operation with such high frequency as to merit the increased complexity of the procedure.

Another aspect worth considering is how to ensure that the initially provided UL WUS configuration is still available at the Cell-A gNB (e.g., the requirements on the Cell-A gNB to store it) and is valid (e.g., the complexity of potential updates of an outdated UL WUS configuration). We thus see a significant increase in complexity with no clear benefit for a procedure working in this way. It is better that the Cell-A gNB starts broadcasting the UL WUS configuration as soon as possible after accepting the request.

RAN3 to agree that the Cell-A gNB starts broadcasting/providing the UL WUS configuration in its SIB as soon as possible after accepting the “start” request of the new Class 1 procedure.

Another detail to consider is that, since the Cell-A gNB can have many cells and cannot judge itself which of its cell(s) are supposed to act as Cell A, as discussed in [6], the NES-Cell gNB shall further indicate in its “start” request to the Cell-A gNB which cell(s) of the Cell-A gNB should assist in providing the UL WUS configuration.

RAN3 to agree that, in the new Class 1 procedure, the NES-Cell gNB indicates to the Cell-A gNB which cell(s) of the Cell-A gNB should act as Cell A (assisting in providing the UL WUS configuration).

We believe, it is sufficient and most appropriate, that the Cell-A gNB responds with “accept” if, and only if, it can provide the UL WUS configuration in all the cells requested (by the NES-Cell gNB) to be Cell A, and “reject” otherwise. The Cell-A gNB cannot judge if broadcasting the UL WUS configuration in a subset of the requested cells will provide good Cell-A coverage for the NES Cell to be in OD-SIB1 mode; thus, the procedure should fail in this situation.

The Cell-A gNB may optionally inform the NES-Cell gNB which cells were responsible for the procedure failing. In this way, the NES-Cell gNB may learn about the unavailability of the failed cells and, potentially, may be able to find a different set of Cell-A cells to assist the NES Cell.

RAN3 to agree that the new Class 1 procedure fails if the Cell-A gNB cannot provide the UL WUS configuration in all the requested cells. The Cell-A gNB may optionally indicate in the failure message which cells were responsible for the procedure failing.

### Design of the new Class 2 procedure to signal the feedback regarding the UL WUS configuration provisioning

It is crucial that the Cell-A gNB provides feedback to the NES-Cell gNB about the Cell A’s provisioning status. This is to ensure that the NES Cell is still accessible for idle/inactive UEs when the NES Cell is in (or about to enter) OD-SIB1 operation mode. But it is up to the NES-Cell gNB to determine when the NES Cell actually goes into OD-SIB1 operation mode.

Indeed, RAN3 has already agreed to introduce a new Class 2 XnAP procedure so the Cell-A gNB can provide feedback to the NES-Cell gNB that it stops the UL WUS configuration provisioning in its SIB. The case of a Cell-A gNB providing feedback to the NES-Cell gNB that it starts the UL WUS configuration provisioning requires further discussion.

As specified in TS 38.331 (see IE DownlinkConfigCommonSIB), the SI modification period, expressed in number of radio frames, is: m = modificationPeriodCoeff * defaultPagingCycle. The modificationPeriodCoeff can take values between 2 and 16, and the defaultPagingCycle can take values between 32 and 256 radio frames. This means that the SI modification period m can be up to 4096 radio frames (≈ 41 seconds), and it should not be optimized only for the sake of OD-SIB1 transmission. For the defaultPagingCycle, for example, large values are common – they improve the UE energy efficiency, and the configuration of large values should therefore definitely be assumed.

The modification period for system information can be up to 41 seconds.

The new Class 1 procedure for UL WUS configuration provision should not be kept open for long, certainly not as long as 41 seconds or more (or until the SI update has reached all UEs in Cell A and the UL WUS configuration provisioning in Cell A is ensured). To avoid unfavorable protocol design, the response to the UL WUS configuration provision request should be sent promptly from the Cell-A gNB to the NES-Cell gNB, indicating that the Cell-A gNB will perform (or not) the UL WUS configuration provisioning.

The response to a request for an UL WUS configuration provisioning should be signaled promptly by the Cell-A gNB to the NES-Cell gNB in the Class 1 procedure.

The NES Cell should not go to OD-SIB1 mode immediately after a positive response in the Class 1 UL WUS configuration provision request procedure. As explained above, up to 41 seconds can pass until the SI has been updated by the Cell-A gNB. Without knowing that the broadcasting of the UL WUS configuration has been started by at least one Cell-A gNB, it is not ensured that the NES Cell can continuously serve UEs.

The NES Cell should not go into OD-SIB1 mode without knowing that the UL WUS configuration has been broadcasted by at least one Cell-A gNB.

Therefore, the agreed new Class 2 procedure should be used by the Cell-A gNB to signal any changes in its UL WUS configuration provisioning status to the NES-Cell gNB. That means, the new Class 2 procedure (not the Class 1 procedure) should also be used by the Cell-A gNB to signal that it has started the UL WUS configuration broadcasting.

RAN3 to agree the new Class 2 procedure is used by the Cell-A gNB to signal any changes in its UL WUS configuration provisioning status to the NES-Cell gNB, including that it has started the UL WUS configuration broadcasting.

A final point regarding the new Class 2 procedure relates to the highlighted part in the agreement below:

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

It is of paramount importance that there is no significant time gap between the moment the Cell-A gNB stops broadcasting the UL WUS configuration in its SIB and the moment the NES Cell reverts to periodic SIB1 transmission (if needed); otherwise, the NES Cell might not be accessible. For this reason, the Cell-A gNB should signal that it stops the UL WUS configuration provisioning before (or simultaneously with) stopping the UL WUS configuration broadcasting. This will give the NES-Cell gNB enough time to revert the NES Cell to periodic SIB1 broadcasting if needed.

RAN3 to clarify that the Cell-A gNB should signal that it stops the UL WUS configuration provisioning before (or simultaneously with) stopping the UL WUS configuration broadcasting.

### Outline of the signaling flow for the two new procedures

We now summarize the signaling steps and the interaction of the two XnAP procedures described above. Figure 2 shows how the procedures can be used by a NES-Cell gNB (gNB1) for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission. The first few steps are as follows:

- In a step 1, gNB1 generates an UL WUS configuration.

- In a step 2, gNB1 sends to gNB2 an UL WUS configuration provision request message comprising the UL WUS configuration and an indication of cell(s) of gNB2 in which gNB1 requests the UL WUS configuration to be provided. In response, gNB1 receives from gNB2 an UL WUS configuration provision response message with an accept/reject indication for the request.

- In case of accept, gNB2 starts providing the UL WUS configuration in requested cell(s) in a step 3, however subject to the SI modification period, as discussed before.

- In a step 4, gNB2 sends to gNB1 an UL WUS configuration provision status message indicating that it indeed has started providing the UL WUS configuration in the requested cell(s).

- In a step 5, gNB1 starts operating the NES Cell in OD-SIB1 transmission mode.

Figure 2: An example of how new procedures can be used for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission.

If the UL WUS configuration provisioning at gNB2 is temporarily interrupted at some point, as in step 6 in Figure 2, gNB2 sends to gNB1 an UL WUS configuration provision status message indicating that it is not providing the UL WUS configuration in the requested cell(s). After receiving such a message (i.e., Cell A to not provide the UL WUS configuration anymore), in a step 7, gNB1 may switch the NES Cell back to periodic SIB1 transmission mode, e.g., if the loss of Cell A(s) at gNB2 means lack of Cell-A coverage for the NES Cell (meaning no Cell-A redundancy).

Figure 3 shows how the proposed procedures can be used to deactivate OD-SIB1 transmission and revert the NES Cell to periodic SIB1 transmission (until further notice). If gNB1 decides to stop using OD-SIB1 transmission in the NES Cell for a longer period, it can inform gNB2 that the provisioning of the UL WUS configuration is no longer needed. Thus, it can send to gNB2 an UL WUS configuration provision request message with a corresponding indication, to relieve gNB2 from the additional provisioning of the UL WUS configuration to UEs at times when not needed. Unnecessary provisioning of an UL WUS configuration means both an unnecessary allocation of radio resources as well as a continuous increase of energy consumption by Cell-A gNB.

Figure 3: An example of how new procedures can be used to deactivate OD-SIB1 transmission and to revert to periodic SIB1 transmission (until further notice).

RAN3 to agree to capture in a Stage-2 specification the signaling flow for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission as depicted in Figure 2.

RAN3 to agree to capture in a Stage-2 specification the signaling flow for deactivating OD-SIB1 transmission and reverting to periodic SIB1 transmission as depicted in Figure 3.

A TP reflecting the above proposals is provided in Appendix A.

RAN3 to agree on the TP for BL CR for TS 38.423 provided in Appendix A.

# Conclusion

Based on the discussion in the previous sections we make the following observations:

Observation 1	The modification period for system information can be up to 41 seconds.

Observation 2	The response to a request for an UL WUS configuration provisioning should be signaled promptly by the Cell-A gNB to the NES-Cell gNB in the Class 1 procedure.

Observation 3	The NES Cell should not go into OD-SIB1 mode without knowing that the UL WUS configuration has been broadcasted by at least one Cell-A gNB.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN3 to agree the design of the new Class 1 procedure as a start/stop request, sent by a NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The “start” request can be accepted or rejected by the Cell-A gNB. The “stop” request is always successful.

Proposal 2	RAN3 to agree that the Cell-A gNB starts broadcasting/providing the UL WUS configuration in its SIB as soon as possible after accepting the “start” request of the new Class 1 procedure.

Proposal 3	RAN3 to agree that, in the new Class 1 procedure, the NES-Cell gNB indicates to the Cell-A gNB which cell(s) of the Cell-A gNB should act as Cell A (assisting in providing the UL WUS configuration).

Proposal 4	RAN3 to agree that the new Class 1 procedure fails if the Cell-A gNB cannot provide the UL WUS configuration in all the requested cells. The Cell-A gNB may optionally indicate in the failure message which cells were responsible for the procedure failing.

Proposal 5	RAN3 to agree the new Class 2 procedure is used by the Cell-A gNB to signal any changes in its UL WUS configuration provisioning status to the NES-Cell gNB, including that it has started the UL WUS configuration broadcasting.

Proposal 6	RAN3 to clarify that the Cell-A gNB should signal that it stops the UL WUS configuration provisioning before (or simultaneously with) stopping the UL WUS configuration broadcasting.

Proposal 7	RAN3 to agree to capture in a Stage-2 specification the signaling flow for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission as depicted in Figure 2.

Proposal 8	RAN3 to agree to capture in a Stage-2 specification the signaling flow for deactivating OD-SIB1 transmission and reverting to periodic SIB1 transmission as depicted in Figure 3.

Proposal 9	RAN3 to agree on the TP for BL CR for TS 38.423 provided in Appendix A.

# References

RP-234065	New WID: Enhancements of network energy savings for NR

RP-242354	Revised WID: Enhancements of network energy savings for NR

R3-244708	Summary of offline disc at RAN3#125, Ericsson (Moderator)

R3-245737	Summary of offline disc at RAN3#125-bis, Ericsson (Moderator)

R3-247806	Summary of offline disc at RAN3#126, Ericsson (Moderator)

R3-245498	Xn impact of On-demand SIB1 for UEs in idle/inactive mode, Ericsson

Text Proposal for BL CR for TS 38.423

*************** Start of TP ***************

### 8.x.x	UL WUS Configuration Provision (FFS)

#### 8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to an NG-RAN node2 and to request NG-RAN node2 to transmit the provided UL WUS configuration information in one or more of its cells.

The procedure is also used to enable NG-RAN node1 to request NG-RAN node2 to discontinue transmission of the provided UL WUS configuration information.

The procedure uses non UE-associated signaling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

The NG-RAN node1 initiates the procedure by sending the UL WUS CONFIGURATION PROVISION REQUEST message to the peer NG-RAN node2 to start the provisioning or to stop the provisioning of UL WUS configuration information in one or more cells of the NG-RAN node2. Upon receipt of the message, the NG-RAN node2:

-	shall initiate the requested provisioning according to the parameters given in the request in case the Request Type IE is set to "start"; or

-	shall terminate the indicated (previously admitted) provisioning in case the Request Type IE is set to "stop".

If the Request Type IE is set to "start" in the UL WUS CONFIGURATION PROVISION REQUEST message, the UL WUS Configuration IE and the Assisting Cells List IE shall be included. The NG-RAN node2 shall, if supported, provide the UL WUS configuration information included in the UL WUS CONFIGURATION PROVISION REQUEST message in all the cells indicated in the UL WUS CONFIGURATION PROVISION REQUEST message and shall respond with the UL WUS CONFIGURATION PROVISION RESPONSE message.

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the NG-RAN node2 cannot provide the UL WUS configuration information included in the UL WUS CONFIGURATION PROVISION REQUEST message in all the cells indicated in the UL WUS CONFIGURATION PROVISION REQUEST message, it shall respond with the UL WUS CONFIGURATION PROVISION FAILURE message with an appropriate cause value. The NG-RAN node2 may include the Assisting Cells Failed List IE in the UL WUS CONFIGURATION PROVISION FAILURE message with the list of cells which could not provide the UL WUS configuration information.

#### 8.x.x.4	Abnormal Conditions

Void.

### 8.x.y	UL WUS Configuration Transmission Status Update (FFS)

#### 8.x.y.1	General

This procedure is initiated by an NG-RAN node to inform a neighbouring NG-RAN node about status changes concerning an admitted UL WUS configuration provision.

It is also used to indicate that the UL WUS configuration provisioning is ongoing or interrupted.

The procedure uses non UE-associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update, successful operation

The NG-RAN node2 shall indicate the status of the admitted UL WUS configuration provision in the UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

*************** Next change ***************

#### 9.x.x.x	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request provisioning of an UL WUS configuration in one or more cells.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Request Type | M |  | ENUMERATED(start, stop, …) | Type of request concerning the provisioning of the UL WUS Configuration. | YES | reject |
| UL WUS Configuration | O |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331. | YES | reject |
| Assisting Cells List |  | 0..1 |  | List of cells of the NG-RAN node2 requested to provide the UL WUS configuration. | YES | reject |
| >Assisting Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Indicates the NR CGI at the NG-RAN node2 | – |  |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

#### 9.x.x.y	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration will be provided in the requested cells.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.x.x.z	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration cannot be provided by the NG-RAN node2.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Assisting Cells Failed List |  | 0..1 |  | List of cells of the NG-RAN node2 which cannot provide the UL WUS configuration. | YES | ignore |
| >Assisting Cells Failed Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Indicates the NR CGI at the NG-RAN node2 | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

#### 9.x.x.t	UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report a status change concerning an admitted UL WUS configuration provision.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Provision Status | M |  | ENUMERATED (providing, not providing, …) | Indicates the provision status of the UL WUS configuration in the assisting cells. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

*************** End of TP ***************