3GPP TSG-RAN WG3 #127	R3-250551

# Athens, Greece, 17th – 21st February 2025

Agenda Item:	17.2

Source:	Ericsson

Title: 	On-demand SSB SCell operation and other Energy Saving topics

Document for:	Discussion and Approval

# 1	Background

The revised WID for “Enhancements of network energy savings for NR” [1] in Rel-19 lists the following objectives:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

This paper discusses the “on-demand SSB SCell operation” topic (Objective 1).

# 2	RAN1&2 agreements on Objective 1

## RAN1: On-demand SSB for SCell operation

### RAN1#116

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

### RAN1#116bis

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not.

  - For on-demand SSB on the cell, downselect between the following alternatives

    - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not.

    - Alt-2: On-demand SSB is limited to non-cell-defining SSB.

      - FFS: Further limitations to on-demand SSB

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

    - FFS further details on L1 and/or L3 measurement

Agreement

The following agreement from RAN1#116 is modified (in red)

- For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

Agreement

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

### RAN1#117

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

  - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

    - This DCI signaling does not provide SCell activation/deactivation.

    - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

  - FFS: Scenarios where the above signalings are applicable

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

Agreement

- At least support L1 measurement based on on-demand SSB

  - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

    - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Agreement

For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

  - The SSB time domain positions of on-demand SSB burst are configured by gNB.

- FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

- Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

- FFS: Whether the value of T is predefined or indicated/configured by gNB

- FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE.

  - Sub-carrier spacing of the on-demand SSB

  - Physical Cell ID of the on-demand SSB

  - Location of on-demand SSB burst

  - Downlink transmit power of on-demand SSB

  - FFS: Other parameters

  - FFS: Whether each of above parameters is configured/indicated explicitly or not

### RAN1#118

Agreement

- Update the previous RAN1 agreement as follows.

  - At least support L1 measurement based on on-demand SSB

    - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

      - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

      - The support of LTM is a separate discussion point

Agreement

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Agreement

For a cell supporting on-demand SSB SCell operation, at least the following is supported

- On-demand SSB on the cell is not located on synchronization raster.

- On-demand SSB on the cell is non-cell-defining SSB

FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

Agreement

Support L3 measurement based on on-demand SSB

- Further work on L3 measurement is up to RAN2/RAN4

Agreement

LS to RAN2 for on-demand SSB SCell operation is agreed. Final LS in R1-2407438.

Agreement

The previous RAN1 agreement made in RAN1#117 is revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

  - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - FFS: Whether the value of T is predefined or indicated/configured by gNB

  - (Working assumption) T=T_min

  - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

LS on timeline for On-demand SSB operation on SCell is agreed in R1-2407565.

### RAN1#118bis

Agreement

- For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options.

  - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

  - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - Option 3: Configuration/indication of the duration of on-demand SSB transmission window

  - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell

  - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

  - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed

  - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI

  - FFS: Each option is applicable to which Cases or Scenarios

  - FFS: Details related to each of the above options

Agreement

- For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1.

  - Sub-carrier spacing of the on-demand SSB

    - FFS if this can be absent

  - Physical Cell ID of the on-demand SSB

  - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index

  - Downlink transmit power of on-demand SSB

  - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

Agreement

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB.

  - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB

  - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB

  - FFS: Whether OD-SSB and always on SSB have same beam or not

Conclusion

No consensus on the support of on-demand SSB SCell operation triggered by UE.

Agreement

The previous RAN1 agreement is partly confirmed and further revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

      - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - (Working assumption) T=T_min

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases.

- Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB

- Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

### RAN1#119

Agreement

Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1:

- The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms.

- The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB.

- RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB.

Further update to be made based on RAN1#119 progress.

Agreement

Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1:

- The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Agreement

Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1:

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Agreement

- For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2.

  - For Case #1 (i.e., No always-on SSB on the cell),

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives.

    - Alt A: Same as for Case #1

    - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

Agreement

- New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.

Agreement

Down-select at least one of the following alternatives.

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

- Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster.

Down-select at least one of the following alternatives.

- Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration.

- Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

Agreement

Response to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?) of Obj.1:

- RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst

- RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

## RAN2: On-demand SSB for SCell operation

### RAN2#125bis

- No discussion in RAN2 this meeting

### RAN2#126

- No discussion in RAN2 this meeting

### RAN2#127

- Scenarios on OD-SSB:

  - RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

- OD-SSB transmission indication:

  - RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

  - New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

- Measurement on OD-SSB:

  - Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

  - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

  - RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

R2-2407414: Proposal 3: RAN2 WG to discuss the issue of false measurement report triggering due to no SSB transmission when on-demand SSB is deactivated.

- RAN2 does not handle this issue in OD-SSB

  - RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

### RAN2#127bis

- OD-SSB transmission indication:

  - No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

  - Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

  - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

### RAN2#128

- Not discussed

# 3	Discussion

## On-demand SSB SCell operation

The concept of SSB-less SCell operation for intra-band CA was introduced in Rel-15. As an enhancement to that, SSB-less SCell operation for inter-band CA was introduced in Rel-18. TS 38.300 defines an SSB-less SCell as follows:

For an intra-band or inter-band CA SCell, a UE may obtain timing reference and AGC source from another serving cell in case the UE is not provided with SSB nor SMTC configuration for this SCell, as described in TS 38.331 [12].

The introduction of SSB-less SCell operation for intra-band and inter-band CA in Rel-15 and Rel-18, respectively, caused no RAN3 impact.

Observation 1: SSB-less SCell operation for intra/inter-band CA has no RAN3 impact.

On-demand SSB SCell operation is a new SCell operation mode that relies on neither periodic SSB transmission nor SSB transmission of another serving cell (like SSB-less SCell operation). Instead, SSB transmission in the respective SCell is triggered when needed, e.g., upon SCell activation for a UE. On-demand SSB transmission is indicated via RRC or MAC CE based signalling and on-demand SSB related parameters are preconfigured via RRC based signalling.

Observation 2: On-demand SSB SCell operation is a new SCell operation mode where SSB transmission in the respective SCell is triggered when needed and indicated to the UE via RRC or MAC CE based signalling. On-demand SSB related parameters are preconfigured via RRC based signalling.

On-demand SSB transmission is tightly connected to SCell operation. It is, for example, started upon SCell activation and stopped upon SCell deactivation or upon SCell activation completion. These operations are under the control of (and observable by) the gNB-DU. On-demand SSB transmission enables energy-efficient SCell operation, as it enables operating SCells with sparse or no always-on SSB in cases where SSB-less SCell operation is not possible, while also ensuring a good CA performance.

Observation 3: On-demand SSB transmission is tightly connected to SCell operation at the gNB-DU. It enables energy-efficient SCell operation (with sparse or no always-on SSB) in cases where SSB-less SCell operation is not possible.

Discussion on potential Xn impact

Up until now, on-demand SSB transmission on a cell (an SCell) is only agreed for scenarios where the SCell is configured to a UE, referred to as Scenario #2 and Scenario #3 in RAN1 (see agreements from RAN1#116 and RAN1#116bis).

Consequently, on-demand SSB transmission on a cell (an SCell) is not visible to UEs for which the cell is not configured as an SCell and cannot impact them, thus cannot benefit them.

Moreover, in CA operation, all serving cells of a UE belong to one gNB, more specifically, one gNB-DU.

Observation 4: On-demand SSB transmission in a cell of a gNB(-DU) may be visible to and impact only UEs served by that cell/gNB(-DU) as SCell.

Consequently, there is no need to inform a neighbour node about on-demand SSB transmission or request a neighbour node to perform on-demand SSB transmission. We conclude that on-demand SSB transmission has no Xn impact.

Proposal 1: RAN3 to conclude that On-demand SSB transmission has no Xn impact.

Discussion on potential F1 impact

On-demand SSB SCell operation is a hybrid of SCell operation with periodic SSB transmission and SSB-less SCell operation. Instead of no having SSB transmissions, SSB transmissions are triggered when needed.

Since on-demand SSB transmission is tightly connected to SCell operation at the gNB-DU (see Observation 3), on-demand SSB transmission must be triggered by the gNB-DU. This is certainly true for MAC CE based signalling but also for RRC based signalling. Note that, at RAN3#126, the following agreement has been captured:

For RRC based signalling, the DU can decide activation/deactivation state of OD-SSB configuration per SCell and convey it to the CU included in the RRC container. There is no RAN3 impact.

Since on-demand SSB transmission in a cell of a gNB-DU is only visible to UEs that are served by that cell/gNB-DU (see Observation 4), there is no obvious need to inform the gNB-CU about on-demand SSB transmission.

Observation 5: On-demand SSB transmission must be triggered by the gNB-DU, and there is no need to inform the gNB-CU.

Moreover, already in legacy, SSB characteristics like SSB frequency, subcarrier spacing, periodicity, time offset, transmit power, etc., are decided by the gNB-DU, e.g., based on OAM configuration. We think the same can apply to on-demand SSB, which, as described above, should be gNB-DU-controlled.

In previous RAN3 meetings, it has been discussed whether the gNB-CU needs to give a permission (to the gNB-DU) for performing on-demand SSB transmission, e.g., in form of an allowed cell list. We believe this is not necessary. Firstly, this was not necessary for SSB-less SCell operation, so it is not obvious why it would be necessary for on-demand SSB SCell operation. Secondly, it is not obvious how (on which basis) the gNB-CU decides to give permission or not. Besides, the gNB-DU can already decide the SSB configuration of a cell and should not be limited to transmit additional SSBs to ensure a good CA performance. We thus propose not to introduce on the F1 interface a permission (allowed cell list) for on-demand SSB SCell operation (from gNB-CU to gNB-DU).

Proposal 2: Not to introduce on the F1 interface a permission (allowed cell list) for on-demand SSB SCell operation.

We think that the on-demand SSB related configuration is best included in the gNB-DU-owned part of the UE configuration (in cell config). As such, the legacy F1 signalling, e.g., the existing UE context management procedures, would suffice to enable the gNB-DU to configure the on-demand SSB related parameters and indicate the on-demand SSB transmission to the UE, like the de-/activation state of the on-demand SSB configuration (agreement above from RAN2).

Proposal 3: Introduce the on-demand SSB related configuration in the gNB-DU-owned part of the UE configuration, but it is up to RAN2 to discuss and decide.

## Energy saving indication from CN to RAN

At this meeting, RAN3 received from SA2 an LS in R3-250024 (S2-2413034) related to “KI#2: UE subscription and policy control for energy efficiency and energy saving.”. The CR 5739 to TS 23.501 attached to the LS indicates that, subscription data for a UE may include an Energy Saving Indicator. It is FFS whether AMF can forward this information to RAN. It is also FFS whether there can be multiple values of the indicator.

5.X.Y Subscription information aspects The Subscription data for a UE may include an Energy Saving indicator that the UE is subject to network energy saving operation. The AMF receives the Energy Saving indicator in the UE Subscription data. The AMF forwards the Energy Saving indicator to the NG-RAN and to the PCF. Editor’s Note: The indicator is expected to select an energy saving strategy for the UE in the system. Whether there are multiple values of the indicator is FFS. Editor’s Note: Whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN is FFS. Editor’s Note: Whether the Energy Saving indicator is also stored in the UDR is FFS. 5.X.Z Policy control aspects During AM policy association establishment procedure, the AMF sends to the PCF the Energy Saving indicator as described in clause 5.x.y. Based on the Energy Saving indicator the PCF may make policy decisions reusing the existing parameters based on the operator policy. Editor’s Note: A notification for the energy related information, if any, should not be provided from the UDM/UDR. Whether the notification for the energy related information from the EIF is provided to the PCF and what are the notification details, are FFS.

According to the CR text, the Energy Saving indicator can be part of user subscription data. It is our understanding, that if this information is present, a user has agreed to use network energy saving operation for the UE. For example, the user accepts some kind of performance degradation (or that performances may not exceed certain levels) in exchange of some energy saving gain (at the network, or at the UE, or both).

Observation 6: If subscription data for a UE includes the Energy Saving indicator, the user subscribed to network energy saving operation.

First of all, we think it would be beneficial for RAN to become aware of this information. This is because with this information, the RAN is formally allowed to reduce its energy consumption when it serves the UE.

Observation 7: It is beneficial for RAN to be aware of user agreement to use network energy saving operation.

Of course, different network implementations can support different energy saving operations, and the same implementation may adopt different strategies to achieve energy savings. Just to make an example, the same network implementation can use different strategies to achieve the same energy saving, depending on the number of UEs served at a given time, the services being used, whether in a cell there are more UEs at cell centre or at cell edge, etc.

Observation 8: Different network implementations can support different energy saving operations.

Observation 9: Different network implementations can achieve the same energy saving in different ways.

We observe that, as part of rel-19 Network Energy Saving Enhancement WI, RAN3 is already discussing mechanisms related to network energy saving operations, such as support for on-demand SSB SCell operation, and support for on-demand SIB1 for UEs. Also, in rel-18, network energy saving mechanisms have already been defined in RAN specifications for CHO (in case the source cell is using a network energy saving solution, the UE may use NES-specific CHO event for executing CHO to a candidate cell).

Observation 10: Different network implementations can achieve the same energy saving in different ways. RAN WGs have already defined or are working on solutions for RAN network energy saving.

The Energy Saving indicator under discussion in SA2 does not relate to the methods that already exist or are being discussed in RAN3. Taking the above into account, on one hand, we think that it could be beneficial for RAN to receive information about user agreement to use network energy saving operation, so that a RAN implementation is enabled to take this information into account if it wants to. On the other hand, there should not be any direct implications on RAN behaviour, including the mechanisms already defined (or being defined) in RAN for network energy saving. To this end, we propose that if AMF provides to RAN information on whether a user subscribed to network energy saving operation, this is a simple value (e.g., a Boolean), and it is left to RAN implementation whether, when and how to use it.

Proposal 4: NG-RAN can receive from AMF a single value (e.g., a Boolean) to indicate whether the user subscribed to network energy saving operation.

Based on the presented discussion, we propose a reply LS in R3-250387.

Proposal 5: Agree to the reply LS in R3-250387.

# 4	Proposals

Proposal 1: RAN3 to conclude that On-demand SSB transmission has no Xn impact.

Proposal 2: Not to introduce on the F1 interface a permission (allowed cell list) for on-demand SSB SCell operation.

Proposal 3: Introduce the on-demand SSB related configuration in the gNB-DU-owned part of the UE configuration, but it is up to RAN2 to discuss and decide.

Proposal 4: NG-RAN can receive from AMF a single value (e.g., a Boolean) to indicate whether the user subscribed to network energy saving operation.

Proposal 5: Agree to the reply LS in R3-250387.

# 5	Reference

[1]	RP-242354	Revised WID: Enhancements of network energy savings, RAN#105, Melbourne, Australia, September 2024

[2]	RP-242354	R3-250024 LS on energy saving indication from CN to RAN