3GPP TSG-RAN WG3 #127	R3-250552

# Athens, Greece, 17th – 21st February 2025

Agenda Item:	17.4

Source:	Ericsson

Title: 	Adaptation of common signal/channel transmissions

Document for:	Discussion and Approval

# 1	Background

The revised WID for “Enhancements of network energy savings for NR” [1] in Rel-19 lists the following objectives:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

This paper discusses the “adaptation of common signal/channel transmissions” related topics (Objective 3).

# 2	RAN1&2 agreements for Objective 3

  - RAN1: Adaptation of common channels/signals (AI 9.5.3)

    - RAN1#116

Agreement

For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study

- Adaptation of SSB burst periodicity

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration

- Adapting the transmitted number of SSBs within a SSB burst

- Cell DTX for SSB adaptation

- Whether to support new SSB burst periodicity value(s)

- Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst)

  - New compact SSB burst(s)

  - Adapting the position of SSBs within a SSB burst

- Other mechanisms/combinations are not precluded

Agreement

For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study

- Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

- For the additional PRACH resources,

  - Adaptation of PRACH resource periodicity/PRACH occasion

  - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle

  - Adaptation based on extending cell DRX operation for PRACH

  - Concentrating ROs in time domain

- Other options are not precluded

Agreement

For adaptation of paging,

- Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings

- Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

Agreement

For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following:

- Applicability to UE in idle/inactive and/or connected mode

- Applicability to PCell and/or SCell(s)

Agreement

For the adaptation mechanisms of SSB in time-domain, study further following mechanisms:

- Adaptation mechanism indicated or configured by gNB without UE trigger

- Adaptation triggered by UE (if any)

FFS: Details of associated signaling/indication/configuration

Agreement

For the adaptation mechanisms of PRACH in time-domain

- Support at least PRACH adaptation provided by gNB without UE trigger

  - FFS: PRACH adaptation with UE trigger

  - Note: UE trigger means UE requests adaptation of PRACH

- Study at least the following,

  - Dynamic signaling and/or semi-static signaling of PRACH adaptation

  - Adaptation of PRACH transmission according to certain condition

  - Applicability to idle/inactive and/or connected mode UEs

  - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

    - RAN1#116bis

Agreement

For indication of adaptation of SSB in time-domain,

- Support at least SSB adaptation provided by gNB without UE trigger

Agreement

For adaptation of PRACH in time-domain, support at least the following:

- Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

  - Configuration of additional PRACH resources is provided by semi-static signalling

    - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs

  - FFS: adaptation mechanism for additional PRACH resources

  - Note: No change to the existing PRACH configuration tables in 38.211

Agreement

For adaptation of PRACH in time-domain, support the following:

- SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any)

  - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs

  - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources

  - FFS: SSB-RO mapping for the additional PRACH resources

Agreement

Support adaptation mechanisms of PRACH in time-domain for following:

- UE in idle/inactive mode

- UE in connected mode

Agreement

Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s):

- For cell with both legacy UEs and Rel-19 NES-capable UEs

  - Rel-19 NES-capable UE’s PCell (Connected mode)

    - Study from the following options:

      - Option A1: adaptation for CD-SSB

      - Option A2: adaptation for SSB that is not CD-SSB

      - Option A3: adaptation for SSB not on sync raster

  - Rel-19 NES-capable UE’s SCell

    - Study from the following options:

      - Option B1: adaptation for CD-SSB

      - Option B2: adaptation for SSB that is not CD-SSB

      - Option B3: adaptation for SSB not on sync raster

  - FFS: Rel-19 NES-capable UE in idle/inactive mode

- Note: Impact to idle/inactive UEs shall be minimized

Agreement

For adaptation of PRACH in spatial domain,

- Study possibility of scenarios with non-uniform distribution of UEs in different beams

    - Note 6: Companies are encouraged to provide details on how they map UEs to different beams

- Study network energy savings gain achieved by non-uniform PRACH resource allocation across SSBs for scenarios with non-uniform distribution of UEs in different beams (if any),

    - Assume the following framework for network energy evaluation in FR1 and companies to report at least the below settings used in the evaluation/simulation

      - 20ms SSB period

      - 30kHz SCS, DDDSU TDD pattern

      - Setting A: SIB1 period (20ms/40ms/160ms)

      - Setting B1: Cell load (Empty/low/medium)

      - Setting B2: Traffic model

      - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

      - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

      - Setting E1: PRACH configurations

        - (legacy) PRACH resources according to the following PRACH configuration for all transmitted SSBs

      - Case A1-1: PRACH configuration #5 (20ms)

      - Case A1-2: PRACH configuration #17 (10ms)

      - Case A2-1: PRACH configuration #0 (160ms)

        - (time-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration for all transmitted SSBs

      - Case B1: PRACH configuration #17 (10ms)

      - Case B2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed time domain adaptation mechanism

        - (spatial-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration

      - Case C1: PRACH configuration #17 (10ms)

      - Case C2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed spatial domain adaptation mechanism, including details of non-uniform PRACH resource allocation across SSBs

      - Setting F: Cat 1/Cat 2 BS as defined in TR38.864

      - Setting G1: Number of SSB beams: 4,8 SSBs in a SSB burst with SSB pattern case C

      - Note 1: Baseline to compare is Case C1 vs Case B1/A1-1/A1-2, Case C2 vs Case B2/A2-1

      - Note 2: It is up to company to report the SSB-RO mapping ratio and FDMed RO number, etc

      - Note 3: Other PRACH configuration index with different PRACH format other than format 0 is not precluded

      - Note 4: Other SSB/SIB1/RACH periodicity/PRACH resource/configuration assumptions are not precluded (up to companies to report)

    - Other frameworks for network energy evaluation are not precluded, e.g. including for FR2

    - RAN1#117

Agreement

For the study of adaptation of PRACH in spatial domain, following network energy savings gains were reported by sources based on the evaluation framework agreed in RAN1#116bis:

- Two sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs A1-1, zero load [R1-2404409, R1-2405107]

  - -4% ~ -45%

- Seven sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1/A1-2, zero load [R1-2404225, R1-2404185, R1-2404334, R1-2404123, R1-2404562, R1-2405107, R1-2405163]

  - 0% ~ 31%

  - Note: Five sources assumed that case B1 has same PRACH resources as case A1-2. Remaining two sources evaluated only A1-2.

  - Note: Three sources showed NES gains 0% ~ 10% [R1-2404225, R1-2404185, R1-2404334]

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 1.0%~8.8%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes.

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -48.41%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, CAT1 BS power model, for case C1 vs A1-2, zero load [R1-2404626]

  - 4.59%~38.04%

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Four sources showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1/A1-2, zero load [R1-2404562, R1-2404225, R1-2403943, R1-2404626]

  - 0% ~ 3.5%

  - Note: Three sources assumed that case B1 has same PRACH resources as case A1-2. One source evaluated only A1-2.

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 0%~0.2%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -1.19%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Two sources showed following NES gain for TDD, CAT1 or CAT2 BS power model, case C2 vs B2, zero load [R1-2403943, R1-2405107]

  - Less than 0.2%

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs A1-2 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 13.7%/8.7%/4.9%/2.6% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs B1 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 8.03%/5.1%/3.06%/1.74% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, C1 vs B1/A1-2, different loads [R1-2404562]

  - 16%/4.78% for light/medium cell load for CAT1 BS power model

  - 0.65%/0.29% for light/medium cell load for CAT2 BS power model

- One source showed following NES gain for TDD, C1 vs B1, different loads [R1-2404626]

  - -18.57%~0%/-2.52%~0% for low /medium cell load for CAT1 BS power model

  - -0.81%~0%/-0.42%~0% for low /medium cell load for CAT2 BS power model

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, C1 vs A1-2, different loads [R1-2404626]

  - 3.67%~19.88%/2.29%~5.22% for low /medium cell load for CAT1 BS power model

  - 0.67%~1.75%/0.39%~0.91% for low /medium cell load for CAT2 BS power model

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed NES gain for FDD, C1 vs B1, zero load [R1-2404464]

  - 1.4%~7% for CAT1 BS power model

  - 0%~0.3% for CAT2 BS power model

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed NES gain for FR2, CAT1 BS power model, spatial domain adaptation of PRACH configuration index 75 vs a time domain adaptation of PRACH configuration index 75, zero load [R1-2405163]

  - 4%~7%

- Note 1: About possibility of scenarios with non-uniform distribution of UEs in different beams

  - Several companies indicated (and three companies showed data/analysis) that there can be scenarios with non-uniform distribution of UEs in different beams.

  - Several companies mentioned that for non-uniform UE distribution, it can be addressed by gNB implementation e.g. by adjusting SSB beamwidth, etc. Several companies also mentioned that it is not clear how gNB can predict the distribution of UEs in different beams, especially for Idle/Inactive UEs.

- Note 2: Most sources that showed the NES gains (if any) for adaptation of PRACH in spatial domain compared to A1-2/B1 observed that the gain would be due to reduction in the number of overall ROs in time domain in their evaluations. Most of these companies only accounted for ROs in time domain.

- Note 3: The evaluation results assumed the non-uniform distribution of UE is static during the evaluation time period.

Conclusion

There is no consensus in RAN1 on the support of PRACH adaptation in spatial domain

Agreement

For adaptation of PRACH in time-domain, support at least the following case(s)

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains

- FFS: whether additional conditions are needed to support the above cases

- FFS: Additional case whether full/partial overlap in both time and frequency is allowed

- Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources

Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For adaptation of SSB in time-domain, Option 1 is supported

- Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)

- Note: Using Option 2 to realize Option 1 is not precluded

  - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active]

    - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities

- FFS: Details including applicable scenarios

- FFS: Support of Cell DTX for connected mode UEs for SSB

Agreement

For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least:

- a PRACH configuration index

- FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources

Study further the following

- When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource,

  - Additional parameter(s) for determining the additional PRACH resources e.g.

    - Scaled/adjusted PRACH configuration period

    - Additional timing offset

    - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Muting/masking ROs

- When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource

  - Additional mechanisms (if any) for determining the additional PRACH resources e.g.

    - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

- Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

Agreement

For the adaptation mechanism for additional PRACH resources, study further the following:

- Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation

- Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

  - Strive to re-use existing DCI format(s)

- Option 3: Adaptation of PRACH transmission according to predefined condition(s)

  - FFS: details

- Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation

  - Strive to re-use existing DCI format(s)

- Option 5: Enhanced cell DRX

    - RAN1#118

Agreement

For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 1-1: Scaled/adjusted PRACH configuration period

    - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Opt 1-3: Muting/masking ROs

    - Opt 1-4: additional timing offset(s)

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources,

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

    - Opt 2-2: Additional timing offset(s)

- FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

- FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset)

Agreement

Extend the RAN1#117 agreement on SSB-RO mapping rule for additional PRACH resources to Case 1

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

RAN1#117 Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For SSB-RO mapping rule for additional PRACH resources for Case 2.

- Extend the RAN1#117 and RAN1#118 agreements on SSB-RO mapping

Agreement

For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE),

- At least DCI based adaptation is supported. No introduction of new DCI format.

Agreement

For adaptation mechanism(s) of SSB in time-domain,

- For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported

  - FFS: Adaptation for SSB that is not CD-SSB is supported (A2)

  - FFS: Adaptation for SSB not on sync raster is supported (A3)

- For Rel-19 NES-capable UE’s SCell

  - Adaptation of SSB configured for the SCell is supported for the following cases

    - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell

    - Adaptation for SSB that is not CD-SSB on sync raster (B2’)

    - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Agreement

For DCI-based adaptation for additional PRACH resources,

- Select from the following DCI format(s) to carry the adaptation indication.

  - DCI format 1_0

  - DCI format 2_7

  - DCI format 2_9

- FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options

- Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period

- Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period

- Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

- Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated.

  - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available.

  - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period

  - FFS: details

    - RAN1#118bis

Agreement

For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources.

Agreement

For adaptation of PRACH in time-domain, support both of the following

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

- FFS: Additional details

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

Agreement

For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different

- FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement).

- Discuss further following options for signaling

- Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources.

  - msg1-FrequencyStart at least for 4-step RACH

  - FFS: other applicable legacy frequency domain parameter(s)

- Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources

  - Note: Offset to legacy frequency domain parameter(s) is a new parameter

  - FFS: applicable legacy frequency domain parameter(s)

Conclusion

There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).

- No spec impact

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

    - RAN1#119

Agreement

Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?):

- RAN1 agreed that at least SSB burst periodicity is adapted.

- There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options.

Agreement

Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?):

- The frequency location is same before and after SSB adaptation.

Agreement

Reply to Q3(What is the spatial relation before and after SSB adaptation?):

- There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation.

- At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

Further update to be made based on RAN1#119 progress if any.

Agreement

At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16]

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

    - Alt 2-6: Network configured time period

Conclusion

There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

Working Assumption

For DCI-based adaptation for additional PRACH resources,

- Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI

- Option 1: Use reserved bits in the DCI format

  - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format

- Option 2: Use Bits 5-8 within the Short Message (from upper layers)

  - Note: Availability should be confirmed by checking with RAN2.

- Option 3: Use bits available for both Option 1 and Option 2

FFS: Payload size for adaptation for additional PRACH resources

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - [DCI payload size = 1 bit]

  - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources

  - FFS: details

Agreement

Confirm the following working assumption from RAN1#118bis.

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

## RAN2: Adaptation of common channels/signals

### RAN2#125bis

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

  - Option 1: Prevent the access of legacy UE via barring;

  - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

### RAN2#126

RACH adaptation in spatial domain

- RAN2 counts on RAN1 evaluation and conclusion.

### RAN2#127

Agreements on paging adaptation

- Option-a) is about the bundling of PF for R19 NES UEs.

- R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.

### RAN2#127bis

Agreements on paging adaptation

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

### RAN2#128

Agreements on paging adaptation

- Introduce value for Ns=8. FFS on 16.

- Introduce value for N= T/32. FFS on T/64, T/128, T/256.

# 3	Discussion

### 3.1	Adaptation of SSB transmission

On the topic of adaptation of SSB transmission, RAN1 has concluded the following (see Section 2):

Conclusion

There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

Conclusion

There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

Consequently, adaptation of SSB transmission is only for Rel-19 NES-capable UE’s SCell(s), and only for RRC_CONNECTED state.

Observation 1: Adaptation of SSB transmission is only for Rel-19 NES-capable UE’s SCell(s).

A cell is an SCell of a UE if the cell is configured as SCell at the UE. Consequently, SSB adaptation on such a cell is not visible to UEs for which the cell is not (configured as) an SCell and cannot impact them.

Moreover, in CA operation, all serving cells of a UE belong to one gNB, more specifically, one gNB-DU.

Considering the above, we observe that adaptation of SSB transmission in a cell of a gNB may be visible to (and impact, e.g., benefit) only UEs served by that cell/gNB, and more specifically, only UEs for which that cell is an SCell.

Observation 2: Adaptation of SSB transmission in a cell of a gNB may be visible to and impact only (Rel-19 NES-capable) UEs served by that cell/gNB (as an SCell).

Consequently, there is no need to inform a neighbour node about SSB adaptation or request a neighbour node to perform some form of SSB adaptation. We conclude that SSB adaptation has no Xn specification impact.

Proposal 1: RAN3 to conclude that adaptation of SSB transmission has no Xn specification impact.

In legacy, the SSB transmission characteristics like ARFCN frequency, subcarrier spacing, periodicity, time offset, transmit power, etc., are decided by the gNB-DU, e.g., based on OAM configuration. Subsequently, they are indirectly communicated to the gNB-CU (via the F1 interface) through the Measurement Timing Configuration IE, which is a mandatory part of the Served Cell Information IE (see TS 38.473). Note that, the SSB configuration comprising such information can also be directly requested by the gNB-CU using the TRP Information Exchange procedure.

This information, comprised in the Measurement Timing Configuration IE, is used by the gNB-CU, e.g., when configuring UEs for measurements. The information includes the SSB Measurement Timing Configuration (SMTC) information, which is configured to the UEs for performing cell measurements, specifically, so that the UEs know when SSB transmissions occur and hence when to measure.

Observation 3: SMTC information is exchanged between gNB-DU and gNB-CU and is configured to the UEs for performing cell measurements.

This means that SSBs must be transmitted (at least) according to the indicated SMTC information.

Consequently, adaptation of SSB transmissions in a cell may only lead to more SSB transmissions than indicated in the SMTC information. The additional SSBs are only visible to UEs that have the cell as an SCell and that are capable of and configured with the SSB adaptation feature. They are not visible to other UEs (see Observation 2).

SSB adaptation cannot lead to less SSB transmissions than indicated in the SMTC information. Otherwise, other UEs are impacted, e.g., when performing cell measurements, which would contradict the conclusions drawn in RAN1.

Observation 4: SSBs must be transmitted according to the indicated SMTC information. Additional SSB transmissions according to SSB adaptation are possible, but they are only visible to UEs that are configured with the feature and do not impact other UEs.

Considering the above, we believe that SSB adaptation, including the associated SSB transmission characteristics, can be decided by the gNB-DU, like in legacy. We think that the SSB adaptation related configuration is best included in the gNB-DU-owned part of the UE configuration (in cell config).

Proposal 2: Introduce the SSB adaptation related configuration in the gNB-DU-owned part of the UE configuration, but it is up to RAN2 to discuss and decide.

As such, the legacy F1 signalling, e.g., the existing UE context management procedures, would suffice to enable the gNB-DU to apply SSB adaptation. Moreover, the existing SMTC information signaling may not need to be enhanced.

So far, we see no obvious F1 specification impact. However, RAN2 does not have any agreements on this topic yet. We thus propose to wait for further progress in RAN1/2.

Conclusion 1: Until further agreements in RAN1/2, SSB adaptation has no F1 specification impact.

Proposal 3: For adaptation of SSB transmission, wait for further progress in RAN1/2.

### 3.2	Adaptation of PRACH

According to various RAN1 agreements (see Section 2), adaptation of PRACH in time domain can be summarized as follows:

Additional PRACH resources (in addition to the PRACH resources available to legacy UEs) can be provided to Rel-19 NES-capable UEs via semi-static signalling. This is simply an additional PRACH configuration. Moreover, the additional PRACH resources can be indicated as available or not available via DCI-based signalling, thus dynamically adapted.

Observation 5: Adaptation of PRACH in time domain relies on semi-statically configured additional PRACH resources that can be dynamically de-/activated.

To avoid excessive inter-cell interference on PRACH, it must be assured that intra-frequency neighbour cells use different PRACH resources in time, frequency, or code domain, e.g., by using different root sequences for deriving preamble sequences.

To automate this, RAN3 has introduced the exchange of information on configured PRACH resources per cell over F1 and Xn. A gNB-DU can share this information (a list of PRACH configurations) to its gNB-CU via F1, which can forward the information to neighbour gNB-CUs via Xn, which can in turn distribute the information to their gNB-DUs via F1. With knowledge of the PRACH configurations of cells of neighbour gNB-DUs, a gNB-DU can choose the PRACH configurations of its cells accordingly.

In current RAN3 specifications, gNB-DUs can, indirectly via their gNB-CU(s), exchange up to 16 PRACH configurations per cell. Hence the additional PRACH configuration that is used for (dynamic) adaptation of PRACH can be coordinated in the same way. The existing signalling is still futureproof and can be reused. We currently see no need to discuss new signalling.

Observation 6: The existing signalling for coordinating PRACH resources among gNBs/gNB-DUs can be reused for coordinating dynamically adaptable PRACH resources in Rel-19.

We believe that, to this point, adaptation of PRACH has no Xn or F1 specification impact. However, RAN1 is still discussing this topic, and RAN2 has not even started the discussion.

Conclusion 2: Until further agreements in RAN1/2, adaptation of PRACH has no Xn or F1 specification impact.

Proposal 4: For adaptation of PRACH, wait for further progress in RAN1/2.

### 3.3	Adaptation of paging occasions

Adaptation of paging occasions has primarily been discussed in RAN2. According to the existing agreements (see Section 2), RAN2 will introduce a new Rel-19 paging configuration and has agreed that Rel-19 UEs only monitor paging occasions (POs) according to that paging configuration, instead of the legacy paging configuration.

### RAN2#127bis

Agreements on paging adaptation

<<< Further agreements are omitted >>>
Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

However, when a gNB is tasked to page a UE, upon receiving a PAGING message from the 5GC in case of CN paging, or a RAN PAGING message from a neighbour gNB in case of RAN paging, it needs to know whether the UE monitors the new or the legacy POs, which depends on whether the UE supports the new paging scheme. Otherwise, the gNB needs to page the UE in both new and legacy POs. But that is not a suitable option, as it will lead to excessive paging, and therefore a waste of radio resources and energy.

Observation 7: When a gNB is tasked to page a UE, it needs to know whether the UE supports the new paging scheme or not and, thus, monitors the new POs or the legacy POs.

Proposal 5: Discuss solutions that enable a gNB to know whether a UE, that is to be paged, supports adaptation of paging occasions.

As defined in TS 38.413 and TS 38.423, the gNB can already receive paging-related UE radio capability info from the 5GC in the PAGING message, or from a neighbour gNB in the RAN PAGING message, respectively. More precisely, those messages can contain the UE Radio Capability for Paging IE, which in turn includes an UERadioPagingInformation message (defined in TS 38.331) as an OCTET STRING. The content of the UERadioPagingInformation message is defined in RAN2, also based on input from RAN3.

To summarize: In legacy, a gNB that is instructed to page a UE receives paging-related UE radio capability information at the same time, and the details of the paging-related UE radio capability information are defined in TS 38.331 (RRC specification).

Observation 8: In legacy, a gNB that is instructed to page a UE receives paging-related UE radio capability information at the same time, and the details of the paging-related UE radio capability information are defined in TS 38.331.

The existing signalling can be extended to solve the outlined issue. One simple option would be to add, to the UERadioPagingInformation message, an indication of whether the concerned UE supports adaptation of paging occasions. We propose RAN3 to consider the addition of such indication.

Proposal 6: To add an indication of whether the concerned UE supports adaptation of paging occasions to the UERadioPagingInformation message (which is defined in TS 38.331).

# 4	Proposals

Observation 1: Adaptation of SSB transmission is only for Rel-19 NES-capable UE’s SCell(s).

Observation 2: Adaptation of SSB transmission in a cell of a gNB may be visible to and impact only (Rel-19 NES-capable) UEs served by that cell/gNB (as an SCell).

Observation 3: SMTC information is exchanged between gNB-DU and gNB-CU and is configured to the UEs for performing cell measurements.

Observation 4: SSBs must be transmitted according to the indicated SMTC information. Additional SSB transmissions according to SSB adaptation are possible, but they are only visible to UEs that are configured with the feature and do not impact other UEs.

Observation 5: Adaptation of PRACH in time domain relies on semi-statically configured additional PRACH resources that can be dynamically de-/activated.

Observation 6: The existing signalling for coordinating PRACH resources among gNBs/gNB-DUs can be reused for coordinating dynamically adaptable PRACH resources in Rel-19.

Observation 7: When a gNB is tasked to page a UE, it needs to know whether the UE supports the new paging scheme or not and, thus, monitors the new POs or the legacy POs.

Observation 8: In legacy, a gNB that is instructed to page a UE receives paging-related UE radio capability information at the same time, and the details of the paging-related UE radio capability information are defined in TS 38.331.

Proposal 1: RAN3 to conclude that adaptation of SSB transmission has no Xn specification impact.

Proposal 2: Introduce the SSB adaptation related configuration in the gNB-DU-owned part of the UE configuration, but it is up to RAN2 to discuss and decide.

Proposal 3: For adaptation of SSB transmission, wait for further progress in RAN1/2.

Proposal 4: For adaptation of PRACH, wait for further progress in RAN1/2.

Proposal 5: Discuss solutions that enable a gNB to know whether a UE, that is to be paged, supports adaptation of paging occasions.

Proposal 6: To add an indication of whether the concerned UE supports adaptation of paging occasions to the UERadioPagingInformation message (which is defined in TS 38.331).

# 5	Reference

[1]	RP-242354	Revised WID: Enhancements of network energy savings, RAN#105, Melbourne, Australia, September 2024