3GPP TSG-RAN WG3 Meeting #127	R3-250553

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.1

Source:	Nokia

Title:	Emergency Services and Inactive eDRX

WID/SID:	NR_newRAT-Core, NR_redcap-Core

Document for:	Discussion and Decision

# 1	Introduction

At this meeting RAN3 received the LS in [1] from RAN2 reporting an issue when eDRX feature is combined with RRC_INACTIVE feature.

The LS states:

In RAN2#128, companies discussed a scenario where a UE is configured with a RAN eDRX cycle in RRC_INACTIVE state when the UE has a PDU session associated with emergency services. RAN2 notice the below description in TS 23.501 which states that an Idle eDRX cycle should not be used by the UE when it has a PDU session associated with emergency services. However, it is unclear whether it is possible for such UE to be released to RRC_INACTIVE state with a RAN eDRX cycle and if it is so how UE behavior may be.

Based on current specification, a RAN eDRX cycle can be configured by a RAN node when releasing the UE to RRC_INACTIVE state only if the UE is configured with an Idle eDRX cycle by CN.

RAN2 assumes that RAN node should be aware whether a UE has a PDU session associated with emergency services so that it can avoid releasing such UE to RRC_INACTIVE state with a RAN eDRX cycle, or even if it is so, there should be means for the RAN node and the UE to not use the configured RAN eDRX cycle. Regarding how RAN node can be aware of such PDU session, RAN2 understands that CN can already indicate this (e.g. via ARP value) but there is no requirement for CN to do this, so, there is case that RAN node may not be aware of this.

RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

# 2	Actions

To SA2, CT1 group

ACTION: 	RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

However, even though the action is directed to SA2/CT1, this issue seems primarily a RAN3 issue since the gNB is in charge of deciding whether/when to send a UE in RRC INACTIVE state.

This is why this paper investigates this issue from RAN3 standpoint.

# 2	Discussion

Even though the action is directed to SA2/CT1, this issue seems primarily a RAN3 issue since the gNB is in charge of deciding whether/when to send a UE in RRC INACTIVE state.

- Requirements

The main question is whether it is acceptable to permanently disable the extended eDRX for voice centric UEs which may need to be called back in a reasonable time (extended eDRX can go up to 3 hours…).

Permanently disabling eDRX would drain the battery faster. This may be seen acceptable for smartphones which are less probe to use this extended eDRX mode, but may not be acceptable for other type of devices

Indeed, certain mobile devices capable of handling emergency calls may be configured to use eDRX, for example:
•  Senior Medical Alert System - simplified devices worn by a person on a neck strap or in the form of a key ring, enabling periodic reporting of geographical position in normal operation mode with long update periods and immediate automatic or manual establishment of emergency calls in the event of danger.
•  Emergency buttons - simplified cellular devices, permanently attached to elements of building infrastructure or to public transport vehicles (including autonomous vehicles) equipped with a ‘red button’ used to initiate emergency calls.
Such devices may use eDRX to support delay tolerant MT communication for remote management and status monitoring while ensuring long battery life.
Automatic activation of emergency calls and activation by ‘red button’ are mentioned in 3GPP TS 22.101 V19.0.0 Clause 10.1.

Due to the above requirements, solutions must be found to only disable temporarily and not permanently extended eDRX whenever a PS callback is expected.

Besides, it is acknowledged that the duration of this temporary disabling is a regulation matter.

- Solutions

After acknowledging the basic requirement above, solutions can be looked at. In fact, two types of solutions may be thought of:

Solutions Type 1: emergency PDU session maintained activated

As long as the UE is in RRC connected state and the emergency PDU session is activated, the gNB has the PDU session context and therefore the ARP of QCI 5 which can inform about the emergency. Therefore, the gNB should refrain from sending the UE to eDRX RRC_INACTIVE state.

However, even though this looks common sense for gNB, the caveat of this type of solutions is that there is strictly speaking no specification text in 3GPP standards currently that specify gNB to behave like this.

Another issue is that even if gNB is configure to disable eDRX inactive while emergency PDU session is activated, such solution would assume keeping the UE is RRC connected until the callback time, and this time can be long depending on regulation. Typical example given by RFC 7090 clause 1 is 30 minutes. See extract below:

Solutions Type 2: emergency session may be deactivated as per TS 23.501 clause 5.6.8

For example, the following scenario could happen:

- UE has an emergency PDU session and another PDU session 1.

- The emergency PDU session is deactivated after the call originated by the end user.

- In the meantime, the PDU session 1 inactivity timer expires and the gNB, which is only aware of the PDU session 1 at that time (it has no more context for the emergency PDU session) which is inactive; therefore, the gNB can send the UE to RRC inactive with inactive eDRX.

- the emergency call back comes from 5GC and it is delayed by inactive eDRX.

This scenario could happen because of TS 23.501 section 5.6.8 which does not forbid SMF to deactivate emergency PDU sessions. Even if SA2 specification would change now, there can be SMFs on field which behave as such.

Solution 1: SMF-only solution

SMF behavior could be changed to never deactivate an emergency PDU session. However, this would not necessarily mean that the emergency PDU session is activated as soon as UE is RRC connected and that we are back to case 1.

For example, the following scenario could happen:

- UE has an emergency PDU session and possibly another PDU session 1.

- The emergency PDU session is not deactivated after the call originated by the end user, at least until for psap callback.

- In the meantime, the UE goes idle (e.g. gNB release UE to idle due to connection lost). The gNB loses all contexts.

- UE reconnects and sends service request for an internet connection PDU session 2. The gNB is only aware of the PDU session 2 context.

- Due to inactivity of the PDU session 2, the gNB sends the UE to inactive eDRX.

- the emergency call back comes from 5GC and it is delayed by inactive eDRX.

Observation 1: SMF-only solution does not solve the problem.

Solution 2: SMF + AMF solution: always-on like solution

Currently, the AMF is aware whether there is an ongoing emergency PDU session: this can be seen from the note 7 in TS 23.502 clause 4.2.2.2:

NOTE 7:	The new AMF can determine if a PDU Session is used for emergency service by checking whether the DNN matches the emergency DNN.

This information is stored in UE context because we can see that it is transferred later on to the SMF during mobility in TS 23.502 clause 4.9.1.3.2 step 3:

3.	[Conditional] S-AMF to T-AMF: Namf_Communication_CreateUEContext Request (N2 Information (Target ID, Source to Target transparent container, SM N2 information list, PDU Session IDs), UE context information (SUPI, Service area restriction, Allowed NSSAI for each Access Type and Partially Allowed NSSAI if available, Tracing Requirements, LTE M Indication, the list of PDU Session IDs along with the corresponding SMF information and the corresponding S-NSSAI(s), PCF ID(s), DNN, UE Radio Capability ID and UE Radio Capability Information, N2 Notify URI). If the subscription information includes Tracing Requirements, the old AMF provides the target AMF with Tracing Requirements.

Observation 2: AMF is aware when an ongoing PDU session is an emergency PDU session

In order to complement the above SMF-only solution, we could have a change in AMF to behave like always-on for this emergency PDU session. This is to say that whenever the UE gets RRC connected and AMF receives a service request, if the AMF has knowledge of an emergency PDU session (see observation 2), the AMF asks SMF to reactivate it.

The required change for AMF would be to modify TS 23.502 clause 4.2.3.2 (UE triggered service request) because currently AMF ask SMF to reactivate PDU sessions only based on UE trigger as can be seen at step 4:

The Nsmf_PDUSession_UpdateSMContext Request is invoked:

-	If the UE identifies List Of PDU Sessions To Be Activated in the Service Request message;

Observation 3: AMF+SMF solution can work to fall back to “activated “ solution type 1, but this will still remain dependent on gNB behavior as for type 1.

Solution 3: AMF-only solution: disable inactive eDRX in the gNB

As shown before, the AMF is aware when there is an ongoing emergency PDU session.

AMF may not be aware if this PDU session is deactivated by SMF or not, but a simple rule can be that AMF disables the inactive eDRX mode in the gNB as soon as/as long as there is an ongoing emergency PDU session, be it deactivated or not.

For this no signaling is needed because the AMF can already enable/disable the inactive eDRX in the gNB by sending UE Context setup/modification request which includes or omits the NR eDRX Information IE.

Observation 4: AMF-only solution does not require new signaling.

Another advantage of the AMF-only solution is that it does not depend on gNB implementation since the gNB cannot send the UE to inactive eDRX if the NR eDRX Information IE is not present.

More precisely, the gNB can be configured by O&M per PLMN whether the AMF(s) in the network implement the new behavior or not as this support information is rather static. Then:

- By default the gNB follows current behavior for inactivity which is based on presence of NR eDRX information received from AMF, no change.

- If some AMF(s) have (still) not implemented the new behavior, the gNB is configured to never use the inactive eDRX.

Observation 5: no impact or dependency foreseen on gNB for the AMF-only solution.

# 3	Conclusion and Proposals

This paper has explained why the issue brought up by RAN2 of disabling inactive eDRX in some specific scenario involves RAN3.

It has identified some possible solutions and has made the following observations and conclusions:

Observation 1: SMF-only solution does not solve the problem.

Observation 2: AMF is aware when an ongoing PDU session is an emergency PDU session

Observation 3: AMF+SMF solution can work to fall back to “activated “ solution type 1, but this will still remain dependent on gNB behavior as for type 1.

Observation 4: AMF-only solution does not require new signaling not any coordination.

Observation 5: no impact or dependency foreseen on gNB for the AMF-only solution.

Therefore, we propose to answer the received LS despite RAN3 is originally in copy and provide a RAN3 solution to the problem along the following two proposals:

Proposal 1: agree the solution 3 which has the least impacts.

Proposal 2: agree the corresponding draft CRs in [2], [3] and the LS in [4] to inform SA2/RAN2.

# 4	References

- R3-250006, LS on Emergency Services and eDRX

- R3-250554, Correction of emergency services and inactive eDRX, R17 TS38.300

- R3-250555, Correction of emergency services and inactive eDRX, R18 TS 38.300

- R3-250556, Reply LS on emergency services and eDRX