3GPP TSG-RAN WG3 Meeting #127	R3-250569

Athens, Greece, 17 – 21 February, 2025

Agenda item:	11.2

Source:	Nokia, Jio Platforms (JPL)

Title:	AI/ML Slicing Open Issues

Document for:	Discussion and Decision

# 1	Introduction

In earlier meetings we have taken several agreements towards defining Stage 3 signalling details related to the AI/ML Slicing use case. Specifically, in RAN3 #125bis we agreed the following:

Enhance the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface.

Introduce Slice To Report List for Data Collection IE including PLMN ID and S-NSSAI in DATA COLLECTION REQUEST message.

Also, in RAN3 #126, we made the following agreements related to predicted resource status per slice:

First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status with list of Slice in the DATA COLLECTION REQUEST message.

Introduce the new bit for Predicted slice available capacity in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message.

Introduce a Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message to report the requested predicted slice available capacity to the requesting node.

Update the Semantics Description of Predicted Radio Resource Status IE in the DATA COLLECTION UPDATE message to indicate that this IE also includes the Slice Radio Resource Status List IE.

In addition, even though we extensively discussed UE Performance on a per slice granularity we could not reach consensus on whether UE Performance is measured on a per S-NSSAI granularity or per QoS Flow group granularity. However, granularity to be provided in UE performance configuration (request) was agreed as follows:

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

In this contribution we provide our views regarding remaining open aspects such as:

- the granularity of the reporting of UE Performance feedback that is reported by a target node to a source node

- remaining aspects for slice level load reporting.

# 2	Discussion

## 2.1	Slice Level Load Reporting

In RAN3 #125-bis we captured the following open point regarding the failure case of predicted radio resource status per slice and predicted slice available capacity:

The failure of the predicted radio resource status per slice and predicted slice available capacity should be indicated to the sending node? Per cell per slice?

Predicted radio resource status per slice and predicted slice available capacity are requested per cell. We think that in case there is a failure to be reported in the  DATA COLLECTION RESPONSE message it should also be indicated per cell. A bit already exists in Cell Measurement Failed Report Characteristics IE to capture the failure case for predicted radio resource status that may include slice-specific granularity. Thus, we think that another bit can be introduced in the Cell Measurement Failed Report Characteristics IE in the DATA COLLECTION RESPONSE message also for the predicted slice available capacity.

Proposal 1: Introduce a new bit in the Cell Measurement Failed Report Characteristics IE in the DATA COLLECTION RESPONSE message to indicate that Predicted Slice Available Capacity measurement cannot be initiated.

However, since partial reporting is supported in Data Collection Reporting procedures, it is possible that a given measurement failed on a certain slice while the reporting procedure is still successful. The slices over which predicted load per slice (Predicted Radio Resource Status per Slice and Predicted Slice Available Capacity) measurement failed are important information to the receiving node. Therefore, we propose to introduce failure information per slice in the DATA COLLECTION RESPONSE.

Proposal 2: Introduce failure indication per slice in DATA COLLECTION RESPONSE message for the predicted load measurements per slice (Predicted Radio Resource Status per Slice and Predicted Slice Available Capacity).

In the last meeting we agreed to introduce Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message. We chose to reuse the existing Slice Available Capacity IE for encoding of the reported predictions. However, one issue that arises with this encoding is the lack in the reported value of the Cell Capacity Class Value IE which is used to scale the slice capacity with respect to the total capacity of the cell in case multiple slices are available for a cell.

One simple way to address this issue without standards impacts is to request Slice Available Capacity (using legacy Resource Status Procedures) every time Predicted Slice Available Capacity (using Data Collection Reporting procedures) is requested. Since in legacy Resource Status Reporting procedures Slice Available Capacity is always reported together with Composite Available Capacity the reported Cell Capacity Class Value will provide the available resources of the cell in the uplink and downlink directions to be used for the Predicted Slice Available Capacity.

Observation 1: One way to determine the Cell Capacity Class Value used to scale the cell capacity of the Predicted Slice Available Capacity IE is to also request Slice Available Capacity using legacy Resource Status Reporting Procedures (which is reported together with the Composite Available Capacity).

Even though this is a feasible solution it requires to always trigger both Data Collection Reporting and Resource Status Reporting procedures in order to deduce the used Cell Capacity Class Value required for the scaling of the reported Predicted Slice Available Capacity. This is not an efficient solution.

Observation 2: Triggering both Data Collection Reporting and Resource Status Reporting procedures in order to determine the reported Predicted Slice Available Capacity is inefficient.

Other more efficient approaches exist but would require to make some changes into the agreements from the previous meeting. One approach is to append in the reporting of Predicted Slice Available Capacity the Predicted Composite Available Capacity, in order to imitate legacy behavior. This would provide the needed information to the receiving node to determine the scaling factor used in the calculation of the Predicted Slice Available Capacity. One problem with this approach is that we need to introduce a new IE (Predicted Composite Available Capacity IE) that contains also unnecessary information since in practice the only information needed to interpret Predicted Slice Available Capacity is the Cell Capacity Class Value.

Observation 3: Reporting Predicted Composite Available Capacity together with Predicted Slice Available Capacity would lead to generating and sending unnecessary information since the only information needed to interpret Predicted Slice Available Capacity is the Cell Capacity Class Value.

For this reason, we think that a better approach would be to introduce a new IE for Predicted Slice Available Capacity and within this IE introduce the missing Cell Capacity Class Value IEs hence making the new IE self-contained.

Proposal 3: Define a new self-contained IE for Predicted Slice Available Capacity including Cell Capacity Class Value Downlink IE and Cell Capacity Class Value Uplink IE.

## 2.2	Slice Level UE Performance

We have agreed to support finer granularity of UE performance feedback in handover scenario to determine UE performance in the target cell for a certain slice. However, so far there haven’t been any stage 3 details agreed. One aspect that remains open is how source triggers slice-level UE Performance feedback from a target node. In DATA COLLECTION REQUEST message we have introduced 4 bits in the Report Characteristics for Data Collection IE to request UE Performance measurements from a target node (i.e., Average Throughput UL, Average Throughput DL, Average Packet Delay and Average Packet Loss). We think that these bits can be reused to request slice-level UE Performance from a target node by enhancing the UE Performance Collection Configuration IE with an indication that Slice Level UE Performance Reporting is requested.

Proposal 4: Introduce Slice Level UE Performance Reporting IE to indicate that slice level UE Performance Reporting is requested from a target node when a UE Performance-specific bit in Report Characteristics IE is set.

A UE requested to report slice-level UE Performance, reports UE Performance for all its active slices. A question that arises is on the behaviour when a UE performance cannot be calculated per slice at the target node. In this case UE Performance is reported according to the UE-level UE Performance.

Proposal 5: If UE at the target node has no active slices, UE-level UE Performance is reported from target node to source node.

Another open aspect pertains to the granularity of slice-level UE Performance. In RAN3 #125 we took an agreement to introduce finer granularity of UE Performance to support slice UE Performance, with the exact granularity to be defined in work item phase. In addition, in RAN3 #125bis there was a strong support by companies to support that UE performance is measured per PDU session WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node. In the lack of consensus in the last meeting, we agreed that a source node can request UE Performance per S-NSSAI granularity. In what follows, we provide our analysis with respect to whether PDU Session level UE Performance feedback is sufficient or not. Towards this end, we list the scenarios of “slice” usage in the RAN below:

- Scenario #1: RAN offers different services, e.g., eMBB, URLLC as different slices: In this case, a separate PDU Session is configured for a given service/slice e.g., eMBB or URLLC. Also, QoS Flow characteristics of QoS Flows in a given PDU Session associated with a slice are typically the same or similar. For example, PDU Session #1 may be configured for eMBB Slice with n QoS Flows, e.g., QoS Flow #1_1, QoS Flow #1_2,….QoS Flow #1_n (with same or similar QoS Characteristics). PDU Session #2 may be configured as a URLLC Slice with n QoS Flows QoS Flow #2_1, QoS Flow #2_2,….QoS Flow #2_n (same or similar QoS Characteristics).  In this scenario, UE Performance measurement at PDU Session level is accurate since all the QoS Flows within it are expected to have the same performance.

Observation 4: If a separate PDU Session is configured for a given service/slice, then UE Performance measurement at PDU Session level can accurately capture UE Performance since all QoS Flows within the PDU session have the same performance.

- Scenario #2: RAN slices are used as a mechanism to partition radio resources: In this case, RAN resources can be partitioned e.g. as GBR slice and non-GBR slice. A GBR slice may offer a set of GBR services such as voice/video/streaming. This means that the QoS Flow characteristics of QoS Flows (voice/video/streaming) in a given PDU Session associated with a slice (GBR slice) may be different. For example, PDU Session #1 (associated with GBR Slice) may have QoS Flow 1_1 associated with 5QI2 (i.e. 5QI=2, video) and also QoS Flow 1_2 associated with 5QI3 (gaming). Given that 5QI2 and 5QI3 have different QoS Characteristics in terms of packet delay, packet loss etc, it wouldn’t be accurate to measure UE performance measurement at PDU Session level since the measurements will be aggregated across different types of QoS Flows.

Observation 5: If the QoS Flow characteristics within a given PDU Session associated with a slice are different, measuring UE Performance at PDU Session level is not sufficient.

Note that, as another possibility the different PDU Sessions can be created and associated with same S-NSSAI (GBR). For example, PDU Session #1 is created with all QoS Flows of 5QI2 and PDU Session #2 is created with all QoS Flows of 5QI3. Both PDU Sessions #1 and #2 are associated with a GBR Slice and therefore measuring UE Performance at PDU Session level would not be an issue.

The details whether a new PDU Session is created or whether a new QoS Flow within an existing PDU Session is used is up to (Core) network implementation.

Note that for the case where the QoS Flow characteristics within a given PDU Session are different, the source node will be aware of this. Thus, if the source node decides to request PDU Session level UE Performance, it would be under the assumption that it is still interested in the reported UE Performance even though it may be inaccurate. As another alternative, the source node can ignore those PDU Sessions from UE Performance measurement.

In any case, all QoS flows that are mapped to the same DRB are also part of the same PDU session. For all scenarios of interest, QoS flows carried by a given DRB will all have the same UE performance. However, any aggregate of UE performance measurements at UE level can’t be used for the slicing use case when the UE is served by more than one slice. Aggregation of UE performance measurements for multiple PDU sessions belonging to the same slice would be theoretically possible, but would require additional aggregation effort and corresponding uncertainty in terms of interpretation of the aggregated result.

According to common understanding in RAN3, UE level performance is derived as the average value of per DRB level measurements of the UE. This aggregation mechanism is intended to apply for each of the components that constitutes UE performance: throughput, packet loss, packet delay. Here, we analyse it for the case of throughput measurements.

As per the common understanding cited above, throughput aggregated at UE level would be obtained by averaging of the measured DRB level throughput for all DRBs allocated for the UE (separately for UL and DL). If simple average of the measured DRB-level throughput is used, where each DRB of the UE is given the same weight, the aggregated throughput per UE may be expressed as follows:

ThpUE= DRBsThpDRBnDRBs												(1)

where ThpUE corresponds to the UE throughput, ThpDRB corresponds to the measured throughput of each DRB in the UE, and nDRB is the overall number of DRBs.

However, throughput measurements in SA5 specification use a different approach. In the ‘Average DL UE throughput in gNB’ measurement specified in TS 28.552, the total data volume is obtained as the sum of the data volumes transmitted in a set of bursts divided by the sum of the durations of these bursts (TS 28.552 clause 5.1.1.3.1):

If UEsThpTimeDl>0, UEsThpVolDlUEsThpTimeDl×1000 [kbit/s]					(2)

If UEsThpTimeDl=0, 0 [kbit/s]

(2) doesn’t explicitly indicate the set of bursts over which the sums ThpVolDl and ThpTimeDl are obtained. However it is clear from (2) that the measurement may span over multiple UEs, and measurement names indicate that there is flexibility in terms of granularity, e.g. it is possible to provide sub-counters per slice (S-NSSAI) and per QCI for this throughput measurement involving multiple DRBs allocated for more than one UE. No explicit mention is made whether the considered bursts can or cannot overlap in time, but no such constraint seems to apply taking into account e.g. that the measurement covers bursts from multiple UEs. The set of observed UEs for the measurement is only hinted to via an example in TS 28.552: “One usage of this measurement is for performance assurance within integrity area (user plane connection quality)”.

Observation 6: Throughput measurement defined in TS 28.552, clause 5.1.1.3.1 aggregates throughput of multiple DRBs and UEs by taking the sum of the data volumes transmitted in a set of bursts divided by the sum of the durations of these bursts.

TS 28.552 defines 5G performance measurements, while RAN3’s current work concerns performance measurements at UE level. SA5 has described corresponding measurements in TS 28.558 applicable to a single UE, derived from the performance measurements in TS 28.552. In this context, the measurement (2) was adapted for the purpose of obtaining the DL throughput measurement for single UE documented in TS 28.558 clause 6.3.1.4.1. This measurement has kept the same name ‘Average DL UE throughput in gNB’ as its TS 28.552 counterpart, and has essentially the same description as the measurement in TS 28.552. However it is clarified that the measurement in TS 28.558 corresponds to the DL M5 measurement for MDT (TS 37.320). The measurement definition is the same as (2) with the only difference that the sum over UEs (UEs ) has been removed from the nominator and the denominator:

If ThpTimeDl>0, ThpVolDlThpTimeDl×1000 [kbit/s]									(3)

If ThpTimeDl=0, 0 [kbit/s]

Same as for (2), also (3) provides flexibility of measurement granularity by avoiding hardcoding of the set of bursts over which the measurement is taken. The described granularities are the DRB (measurement name DRB.UEThpDl), the QoS flow (DRB.UEThpDl.QoS) and the slice (DRB.UEThpDl.SNSSAI). Because the measurement is defined for a single UE, it becomes straight-forward to report measurement results to the TCE per DRB, but (3) also allows for aggregation at e.g. PDU session level or slice level.

It can also be noticed that the “Average DL UE buffered Throughput per DRB” measurement defined in TS 28.552 clause 5.1.1.3.7, done at the gNB-CU-UP, has the same characteristics as (3):

ThroughputVolumeThroughputTime [kbits/s]	(4)

Observation 7: UE-level throughput measurements defined in TS 28.558 clause 6.3.1.4.1 and TS 28.552 clause 5.1.1.3.7 allow for results to be reported per DRB to the TCE or to be aggregated e.g. at PDU session level or slice level.

Based on what we have shown above, UE throughput of a PDU session may be reported over Xn according to 3 options:

- Option 1: Reporting of PDU session level UE throughput obtained by aggregation of per DRB level measurements according to the method in (1)

- Option 2: Reporting of PDU session level UE throughput obtained by aggregation of per DRB level measurements according to the method in (3) or (4)

- Option 3: Reporting of UE throughput for each DRB (or group of QoS flows) of the PDU session. The receiver of the information may then perform aggregation according to its preferred method, if needed

For down-selection between these options, we first show via a simple illustration some practical differences between option 1 and 2. For this we use the simplified scenario of a PDU session composed of 2 DRBs, where each DRB will have a single data burst during the measurement period with different burst data volumes (DV) and burst durations as exemplified in the following table:

|  | DRB1 |  |  | DRB2 |  |  | Aggregated throughput option 1 [Mbit/s] | Aggregated throughput option 2 [Mbit/s] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Burst DV [kbit] | Burst duration [ms] | Burst tput [Mbit/s] | Burst DV [kbit] | Burst duration [ms] | Burst tput [Mbit/s] |  |  |
| Example 1 | 100 | 10 | 10 | 400 | 40 | 10 | 10 | 10 |
| Example 2 | 100 | 10 | 10 | 100 | 400 | 0.25 | 5.13 | 0.49 |
| Example 3 | 200 | 40 | 5 | 400 | 20 | 20 | 12.5 | 10 |

In example 1 the burst durations are different (10 ms and 40 ms respectively), however the burst throughput are identical and hence method 1 and method 3 give the same result (10 Mbit/s). In example 2 we used the same data volume for both bursts but introduce a significantly longer burst duration for DRB2 (400 ms) compared to DRB1 (10 ms). The non-weighed average based on option 1 gives an aggregate throughput of 5.13 Mbit/s, while option 2 gives a weight to each of the DRBs corresponding to the burst duration and hence only 0,49 Mbit/s aggregated burst throughput, relatively close to the throughput of DRB2 of 0.25 Mbit/s. A first observation is that these examples show the importance of clarifying in the standard which aggregation method is used if aggregated throughput values are signalled over network interfaces.

Observation 8: If aggregated UE throughput values are signalled over network interfaces, it is important to indicate in the specification which aggregation method is used.

When it comes to which of the aggregation methods that seems the most suitable, we notice that a DRB that has a less stringent requirement of latency (i.e. it has a higher packet delay budget), and hence longer burst periods, is given stronger weight by the aggregation algorithm based on option 2 than a DRB with stricter latency requirement. From this point of view, we believe that option 1 provides better fairness between DRBs with different QoS requirements.

Observation 9: Option 1, by avoiding weighed aggregation, provides better fairness between DRBs with different QoS requirements than option 2.

For Rel-19 work, the simplest way still seems to be to go for option 3 which is also the option used for the MDT M5 (UE throughput) measurement. With this option the UE throughput is reported separately for each DRB. However, due to potential remapping of DRBs in the target node, it seems appropriate to report the UE throughput per group of QoS flows corresponding to the DRB in the reporting (target) node.

Proposal 6: For the Rel-19 slicing use case, report the UE throughput per group of QoS flows over network interfaces.

This approach is applicable also to packet delay measurements and packet loss measurements. Therefore, in order to support the stage 3 work for the slicing use case, we propose the following regarding UE performance reporting:

Proposal 7: Upon request of slice level UE Performance Feedback from the source, the target node reports UE performance for all PDU sessions associated to one or more requested S-NSSAIs.

# 3	Conclusion

In this paper we make the following observation and proposals:

Proposal 1: Introduce a new bit in the Cell Measurement Failed Report Characteristics IE in the DATA COLLECTION RESPONSE message to indicate that Predicted Slice Available Capacity measurement cannot be initiated.

Proposal 2: Introduce failure indication per slice in DATA COLLECTION RESPONSE message for the predicted load measurements per slice (Predicted Radio Resource Status per Slice and Predicted Slice Available Capacity).

Observation 1: One way to determine the Cell Capacity Class Value used to scale the cell capacity of the Predicted Slice Available Capacity IE is to also request Slice Available Capacity using legacy Resource Status Reporting Procedures (which is reported together with the Composite Available Capacity).

Observation 2: Triggering both Data Collection Reporting and Resource Status Reporting procedures in order to determine the reported Predicted Slice Available Capacity is inefficient.

Observation 3: Reporting Predicted Composite Available Capacity together with Predicted Slice Available Capacity would lead to generating and sending unnecessary information since the only information needed to interpret Predicted Slice Available Capacity is the Cell Capacity Class Value.

Proposal 3: Define a new self-contained IE for Predicted Slice Available Capacity including Cell Capacity Class Value Downlink IE and Cell Capacity Class Value Uplink IE.

Proposal 4: Introduce Slice Level UE Performance Reporting IE to indicate that slice level UE Performance Reporting is requested from a target node when a UE Performance-specific bit in Report Characteristics IE is set.

Proposal 5: If UE at the target node has no active slices, UE-level UE Performance is reported from target node to source node.

Observation 4: If a separate PDU Session is configured for a given service/slice, then UE Performance measurement at PDU Session level can accurately capture UE Performance since all QoS Flows within the PDU session have the same performance.

Observation 5: If the QoS Flow characteristics within a given PDU Session associated with a slice are different, measuring UE Performance at PDU Session level is not sufficient.

Observation 6: Throughput measurement defined in TS 28.552, clause 5.1.1.3.1 aggregates throughput of multiple DRBs and UEs by taking the sum of the data volumes transmitted in a set of bursts divided by the sum of the durations of these bursts.

Observation 7: UE-level throughput measurements defined in TS 28.558 clause 6.3.1.4.1 and TS 28.552 clause 5.1.1.3.7 allow for results to be reported per DRB to the TCE or to be aggregated e.g. at PDU session level or slice level.

Observation 8: If aggregated UE throughput values are signalled over network interfaces, it is important to indicate in the specification which aggregation method is used.

Observation 9: Option 1, by avoiding weighed aggregation, provides better fairness between DRBs with different QoS requirements than option 2.

Proposal 6: For the Rel-19 slicing use case, report the UE throughput per group of QoS flows over network interfaces.

Proposal 7: Upon request of slice level UE Performance Feedback from the source, the target node reports UE performance for all PDU sessions associated to one or more requested S-NSSAIs.