3GPP TSG-RAN WG3 Meeting #127	R3-250578

Athens, Greece, 17 – 21 February, 2025

Agenda item:	20.2

Source:	Nokia

Title:	(TP to TS 38.300,TS 38.305,TS 38.455) AI/ML Positioning Case 3a

Document for:	Text Proposal

# 1	Introduction

In RAN3 #126, we made the following agreements:

Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

There exist several open aspects that we would like to address in this meeting.

FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1).

FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used.

FFS if LMF should be aware that the reported measurements are AI/ML generated.

For the sake of completeness to the current status we provide next, earlier RAN3 agreements. In RAN3 #125bis, we agreed the following:

The existing LCS framework is re-used for AI/ML positioning.

Also, for case 3a the following was agreed:

The AI/ML model (inference and training functions) is at the gNB.

LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

The details of the measurements are still pending and subject to further discussion in RAN1.

In this contribution, we provide our views with respect to different considerations pertaining to Case 3a. We also provide two Stage 2 TPs, one to TS 38.300 and another one to TS 38.305, as well as a TP to TS 38.455, in the annexes of this paper.

# 2	Discussion

## 2.1 	AI/ML Assisted Positioning (Case 3a)

RAN3 has agreed that for Case 3a AI/ML Model Training and AI/ML Model Inference functions are located at the gNB. Another possible option for the AI/ML Model Training is OAM, if OAM receives training data (measurements, ground truth label and related data) so that an AI/ML Model can also be trained offline. This would be in line with the Rel-18 framework of NG-RAN AI/ML described in TS 38.300 where AI/ML Model Training may be located in OAM or in the gNB. In addition, this would be in line with the TR 38.843 where the following is mentioned: “For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.”. This means that OAM may be a termination point receiving training data and hence may also train the AI/ML model of a gNB.

Proposal 1: Besides the gNB, AI/ML Model Training can also be located in OAM.

Note that if AI/ML Model Training is in OAM we can also reuse the Model Deployment/Update functionality that SA5 has introduced to support the deployment of the AI/ML Model from OAM to RAN.

Several aspects remain open for AI/ML assisted positioning (case 3a) that we address next.

### 2.1.1 UE Selection

We have agreed that the AI/ML model (inference and training functions) is at the gNB. A question that arises is who selects the UEs for the data collection, namely the LMF or the gNB. We identify the following options:

- gNB selects the best UEs for training its gNB-side model. The best UEs may be UEs for which there does not exist training data available at the LMF.

- LMF indicates to a gNB a list of UEs from which the gNB can select for its data collection needs. Those are UEs for which necessarily exist training data (part B) available at the LMF.

Since data is collected for training of a model located at a gNB, we believe that only the gNB, knowing the details of its own AI/ML Model, can determine the right UEs for the data collection. Those can be UEs in certain locations, transmitting UL SRS with certain characteristics, UEs that meet criteria to avoid bias in the model training process, etc. On the other hand, LMF can select UEs that are authorized to provide measurements. However, it is unclear if those UEs are necessarily the “right” UEs for training a gNB-side model. The reason is that those UEs may be selected in a biased way and may not ensure diversity in the UE selection, e.g., may not represent UEs chosen over different areas in a gNB and having the desirable UE capabilities or radio conditions. Of course, it cannot be also excluded that some of those UEs are suitable.  The advantages and disadvantages of the above options are summarized below:

|  | Advantages | Disadvantages |
| --- | --- | --- |
| gNB in charge of UE selection | -gNB knows best which UEs need to be selected for training its own AI/ML gNB-side model, e.g., satisfying certain SRS configurations, radio conditions, being at a certain location | -LMF may not have the requested positioning data and will need to start necessary measurements to obtain those - UEs need to be authorized to provide measurements (e.g., user consent needs to be checked) |
| LMF in charge of UE selection | -Collection of training data is optimized since LMF chooses UEs for which it already has training data available -Selected UEs are authorized to provide measurements (e.g., user consent has been checked by the CN) | -Inflexible and may lead to suboptimal training outcome (e.g., training bias) |

Among the different criteria discussed and listed in the table above, particular weight needs to be given to select a solution that enables a gNB to obtain a good dataset for its AI/ML Model Training and that can avoid training bias, which is essential to obtain a correctly working model. To this end, a gNB itself, knowing the details of its own ML Model, can determine the right UE selection to achieve these targets.

Observation 1: A gNB itself, knowing the details of its own ML Model, can determine the right UE selection to obtain a good dataset for its AI/ML Model Training and avoid training bias.

Therefore, it seems that allowing the gNB to choose the UEs has an advantage over letting LMF to do so when it comes to optimizing the training and monitoring data collection related to an AI/ML Model at the gNB. This leads to the conclusion that the right UE selection can only be determined by the gNB. A gNB may determine a list of suitable UEs based on identified requirements that the selected UEs should meet. Such requirements may comprise requirements on UE location or a preferred SRS Configuration, as per the example given above.

Proposal 2: A gNB determines UEs to be chosen for data collection based on certain requirements, e.g. with respect to a required SRS Configuration or UE location.

Of course, it cannot be precluded that an LMF also has data available that may be useful for the training of an AI/ML model at the gNB side. In this case, a gNB can opportunistically take advantage of the available LMF information for training of its own AI/ML Model.

Observation 2: LMF may also have available information that can be used by the gNB for training its own AI/ML model.

Different gNBs may execute different AI/ML Models and may have distinct implementations. Data collection methods should be able to provide all the necessary data at a training entity to support various AI/ML algorithms and implementations. Therefore, we think that both gNB-based and LMF-based UE selection methods need to be supported.

Proposal 3: Support both gNB-based and LMF-based UE selection methods.

Towards this end, RAN3 should further discuss LMF-based UE selection methods where data available at the LMF can be exploited opportunistically by a gNB for its training needs.

A gNB may decide to train an AI/ML Model by its own implementation. This can be based on internal determination of the need to improve its measurements or through a trigger from OAM that could orchestrate the AI/ML functionality in the RAN to give some examples.

For label-based training, where part B is needed to be received from LMF, LMF starts the NRPPa transaction. This, in a sense, also limits the requests for part B from gNBs to points in time when LMF decides so.

Towards this end, LMF starts an NRPPa transaction by providing to a gNB a list of UEs and the corresponding Part B information.

Proposal 4: LMF provides to a gNB a list of UEs and the corresponding part B data.

As discussed above, this list of UEs may not comprise the optimal set for AI/ML Training of a gNB side model. Thus, the LMF may further introduce an indication to the gNB to allow it to provide feedback that part B for additional specific UEs is desired.

Proposal 5: The LMF may additionally include an indication in an NRPPa message to the gNB to allow the gNB to provide feedback that part B for additional specific UEs is desired.

We think that this message by the LMF can be sent in a Class 2 procedure.

Proposal 6: Introduce a new LMF-initiated Class 2 procedure to enable the LMF to provide a gNB with a list of UEs and corresponding part B data along with an indication whether the gNB is allowed to request part B for additional UEs.

Subsequently, a gNB may request part B for additional UEs, if it deems necessary. One of the open points from the last meeting is which protocol is used to request part B from LMF, namely NGAP or NRPPa. We think that since part B pertains to positioning information it is more appropriate to use NRPPa.

Proposal 7: Introduce a new gNB-initiated Class 2 procedure to enable the gNB to provide feedback that Part B for additional specific UEs is desired.

There is a benefit to design the protocol through two Class 2 procedures in that an NRPPa transaction does not need to be delayed, waiting for data to become available at either the LMF or at the gNB side.

In the last meeting we made a WA which we believe we can confirm in this meeting.

Proposal 8: RAN3 to confirm the WA:

WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

A design principle for the AI/ML for NG-RAN work has been that the procedures used for the support of AI/ML for NG-RAN needed to be use case and data type agnostic. We believe that this is an important design principle which should also be applied to the AI/ML assisted positioning with gNB side AI/ML model.

Proposal 9: The new NRPPa signalling procedures used for the exchange of information between a gNB and an LMF to support AI/ML assisted Positioning with gNB side AI/ML model, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

We provide a TP to TS 38.455 in annex of this paper.

### 2.1.2	Correlation of Part A and Part B

RAN1 has made the following agreement with respect to Part A and Part B:

RAN1 #116bis

Agreement

For training data collection of AI/ML based positioning, the collected data sample can include the following components:

Part A:

- channel measurement

- quality indicator of channel measurement

- time stamp of channel measurement

Part B:

- ground truth label (or its approximation)

- quality indicator of label

- time stamp of label

Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification.

Note: contents in Part A and Part B may or may not be generated by different entities.

Note: Part A and/or Part B, and their contents may or may not apply for each case

FFS: detailed definition of channel measurement

RAN1 #117

Agreement

For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: ​

• for a same UE (PRU or Non-PRU UE), and ​

• for a same location associated with Part B.​

Note: the association can be discussed

In addition, one of the agreements of the previous RAN3 meeting is that the LMF provides ground truth label and related data (i.e., part B information) to NG-RAN.

LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

This information will be used by a gNB to train its model and to perform model performance monitoring, and the gNB will not be able to perform model training without it, in case of label-based training.

According to RAN1 discussions, part B may comprise ground truth label (or its approximation), a quality indicator of label and a time stamp of label. This label may be, e.g., in case of ToA measurement a distance, but there is currently no RAN1 agreement about part B including actual UE location.

Observation 3: According to RAN1 agreement, Part A and Part B are from the same location and from the same UE, but there is no assumption that in order to associate the two parts UE location is known at the gNB.

A gNB receiving part B from the LMF and having calculated part A itself will need to correlate the two quantities for training its AI/ML model. According to RAN1 agreements, for training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are:

- For a same UE (PRU or Non-PRU UE), and ​

- For a same location associated with Part B

RAN3 also agreed that LMF provides the ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN while NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). From the above, it follows that a data sample containing both Part A (from gNB) and Part B (from the LMF) must be for the same UE and for the same location, but RAN1 has made no assumption on how to associate the two parts. We think that from RAN3 perspective, correlation of Part A and Part B through location is not necessary.

Observation 4: From RAN3 point of view, correlation of Part A and Part B through location may not be necessary.

Instead, we suggest that this correlation is done using timestamp information.

Proposal 10: For a given UE, an NG-RAN node correlates part A and part B using timestamp information.

### 2.1.3	Reporting of Intermediate Features to LMF

RAN1 made the following agreements with respect to the intermediate features:

RAN3 #116

Agreement

For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215.

- Note: details of the report are pending further discussion.

RAN1 #118bis

Agreement

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

RAN3 #119

Conclusion

For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator,

- LOS/NLOS indicator can’t be reported independently from other measurements

According to legacy positioning methods, an LMF requests measurements from a gNB and uses those measurements for the calculation of a UE positioning estimate. The LMF’s positioning algorithm does not depend on the method that gNB used to determine those measurements, while at the same time it is up to gNB implementation whether and how to perform those measurements.

Observation 5: For Case 3a, the gNB, by its implementation, will use its best available method to determine the timing information measurements requested by the LMF.

Looking at the TRP Measurement Result IE in TS 38.455, we observe that the timing information that a gNB can provide through AI/ML corresponds to measurements that can be reported already by a gNB to the LMF. From LMF point of view, there is no difference between measurements that are derived by AI/ML Model Inference or by legacy measurements, measured by the gNB/TRP. Thus, it should be transparent to the LMF what methods (AI/ML or legacy) the gNB uses to provide the requested measurements to the LMF. The gNB should respond to the LMF request by choosing the best measurement it has available by its implementation, either by deploying AI/ML or legacy procedures.

Proposal 11: gNB decides whether the measurements reported to LMF are outcomes of legacy measurements or produced by AI/ML operations.

Therefore, existing methods can be used to configure reporting by a gNB in Case 3a.

Proposal 12: Reuse existing NRPPa methods to configure gNB to report LoS/NLoS indication and timing information, such as UL RTOA and gNB Rx-Tx Time Difference.

# 3	General Principles

Case 3a assumes the existence of an AI/ML Model in the gNB. We, therefore, see useful to agree for this case some high-level principles from TR 37.817 and from the AI/ML for NG-RAN work, that were considered valid during Rel-18 AI/ML for NG-RAN normative phase and are also used for the Rel-19 work that is ongoing.

Specifically, even though in Case 3a we provide support so that a gNB may train an AI/ML Model we should follow the same assumption as in NG-RAN AI/ML that AI/ML Models and Algorithms are not in scope.

Proposal 13: Details of AI/ML Models and Algorithms for AI/ML positioning case 3a are implementation specific and out of scope.

Another principle that we followed in Rel-18 NG-RAN AI/ML work is that an AI/ML Model used for Model Inference needs to have completed all the steps of Life Cycle Management, meaning AI/ML Model Training and AI/ML Testing.  Since in Case 3a, a gNB can provide the LMF with inferred measurements we thinking that the same principle should be valid for the AI/ML Air Interface work.

Proposal 14: An AI/ML model used in a Model Inference function has to be initially trained, validated and tested before deployment.

Finally, another fundamental principle for Data Collection is related to respecting user data privacy and anonymization and it shall be valid for this work item also.

Proposal 15: User data privacy and anonymization need to be respected during AI/ML operation.

In the last meeting, we confirmed a working assumption that NG-RAN node performs monitoring metric calculation for its own model. Model Monitoring in TR 38.843 corresponds to Model Performance monitoring in RAN3 discussions, which is the process enabling the monitoring of the performance of an AI/ML model. The outcome of Model Performance Monitoring is provided by the Model Inference to the Model Training entity through Model Performance Feedback.

Proposal 16: Use the term Model Performance monitoring to refer to Model Monitoring for better alignment with RAN3 terminology.

Similarly to the RAN3 NG-RAN AI/ML framework, the process of Model Performance Monitoring and Model Performance Feedback is implementation specific and out of RAN3 scope.

Proposal 17: Model Performance Feedback is implementation-specific and out of RAN3 scope.

We introduce some of the above proposals in the stage 2 TPs in the annexes.

# 4	Conclusion

In this paper we make the following observation and proposals:

Proposal 1: Besides the gNB, AI/ML Model Training can also be located in OAM.

Observation 1: A gNB itself, knowing the details of its own ML Model, can determine the right UE selection to obtain a good dataset for its AI/ML Model Training and avoid training bias.

Proposal 2: A gNB determines UEs to be chosen for data collection based on certain requirements, e.g. with respect to a required SRS Configuration or UE location.

Observation 2: LMF may also have available information that can be used by the gNB for training its own AI/ML model.

Proposal 3: Support both gNB-based and LMF-based UE selection methods.

Proposal 4: LMF provides to a gNB a list of UEs and the corresponding part B data.

Proposal 5: The LMF may additionally include an indication in an NRPPa message to the gNB to allow the gNB to provide feedback that part B for additional specific UEs is desired.

Proposal 6: Introduce a new LMF-initiated Class 2 procedure to enable the LMF to provide a gNB with a list of UEs and corresponding part B data along with an indication whether the gNB is allowed to request part B for additional UEs.

Proposal 7: Introduce a new gNB-initiated Class 2 procedure to enable the gNB to provide feedback that Part B for additional specific UEs is desired.

Proposal 8: RAN3 to confirm the WA:

WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

Proposal 9: The new NRPPa signalling procedures used for the exchange of information between a gNB and an LMF to support AI/ML assisted Positioning with gNB side AI/ML model, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

Observation 3: According to RAN1 agreement, Part A and Part B are from the same location and from the same UE, but there is no assumption that in order to associate the two parts UE location is known at the gNB.

Observation 4: From RAN3 point of view, correlation of Part A and Part B through location may not be necessary.

Proposal 10: For a given UE, an NG-RAN node correlates part A and part B using timestamp information.

Observation 5: For Case 3a, the gNB, by its implementation, will use its best available method to determine the timing information measurements requested by the LMF.

Proposal 11: gNB decides whether the measurements reported to LMF are outcomes of legacy measurements or produced by AI/ML operations.

Proposal 12: Reuse existing NRPPa methods to configure gNB to report LoS/NLoS indication and timing information, such as UL RTOA and gNB Rx-Tx Time Difference.

Proposal 13: Details of AI/ML Models and Algorithms for AI/ML positioning case 3a are implementation specific and out of scope.

Proposal 14: An AI/ML model used in a Model Inference function has to be initially trained, validated and tested before deployment.

Proposal 15: User data privacy and anonymization need to be respected during AI/ML operation.

Proposal 16: Use the term Model Performance monitoring to refer to Model Monitoring for better alignment with RAN3 terminology.

Proposal 17: Model Performance Feedback is implementation-specific and out of RAN3 scope.

# Annex A -TP to TS 38.300

## <<< start of changes >>>

## 16.20	Support of AI/ML for NG-RAN

### 16.20.1	General

Support of AI/ML for NG-RAN, as a RAN function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization, AI/ML assisted positioning with gNB-side AI/ML model.

### 16.20.2	Principles

Support of AI/ML for NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

Signalling procedures used for the exchange of information to support AI/ML for NG-RAN, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

AI/ML algorithms and models are out of 3GPP scope. Model-specific performance information, e.g. model performance indicators specified in clause 6 of TS 28.105 [64], is not exchanged over NG-RAN interfaces in TS 38.401 [4].

Support of AI/ML for NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [64]. An AI/ML Model needs to be trained, validated and tested before deployment for AI/ML Model Inference.

AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [64].

### 16.20.3	Data Collection and Reporting

The following information can be configured to be reported by an NG-RAN node:

-	Predicted resource status information;

-	UE performance feedback;

-	Measured UE trajectory;

-	Energy Cost (EC).

The collection and reporting are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover.

Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Procedures for Data Collection and Reporting for AI/ML assisted positioning with gNB-side AI/ML model are described in TS 38.305 [42].

## <<< end of changes >>>

# Annex B -TP to TS 38.305

## <<< start of changes >>>

## x	Support of AI/ML Positioning

## x.1	AI/ML assisted Positioning with gNB side AI/ML model

### x.1.1	General

NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements by the gNB for the purpose of training of the corresponding gNB side AI/ML model and to enable the NG-RAN to provide output of Model Inference e.g., intermediate parameters to the LMF.

### x.1.2	Positioning procedures (name FFS)

The following information can be configured to be reported by an NG-RAN node to an LMF:

-	Positioning intermediate parameters (e.g. LoS indicator, NLoS indicator, timing information).

The following information can be configured to be reported by an LMF to an NG-RAN node:

- Ground truth label, quality indicator of label, time stamp of label.

NRPPa procedures are used for the configuration and reporting of this information.

<<< end of changes >>>

# Annex C -TP to TS 38.455

## <<< start of changes >>>

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-	Measurement Preconfiguration Information Transfer. This function allows the LMF to request the NG-RAN node to preconfigure and activate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer. This function allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

-     Data Collection Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of AI/ML data collection.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Preconfiguration Information Transfer | Measurement Preconfiguration Measurement Activation |
| Area-specific SRS Information Transfer | SRS Information Reservation Notification |
| Data Collection Transfer | a) Data Collection Report b) Data Collection Feedback |

# 8	NRPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Data Collection Report | DATA COLLECTION REPORT |
| Data Collection Feedback | DATA COLLECTION FEEDBACK |

## <<< next change >>>

## 8.X	Data Collection Transfer

### 8.X.1	Data Collection Report

#### 8.X.1.1	General

The Data Collection Report procedure allows the LMF to provide the NG-RAN node with positioning information for one or more UEs e.g., to support AI/ML assisted positioning with gNB side AI/ML model. This procedure applies only if the NG-RAN node is a gNB.

#### 8.X.1.2	Successful Operation

Figure 8.X.1.2.1: Data Collection Report procedure. Successful operation.

The LMF initiates the procedure by sending a DATA COLLECTION REPORT message to the NG-RAN node, indicating positioning information for a list of UEs.

If the Additional UE Selection Allowed IE is contained in the DATA COLLECTION REPORT message, the NG-RAN node may consider the NG-RAN node is allowed to provide feedback to request positioning information for additional UEs.

#### 8.X.1.3	Unsuccessful Operation

Not Applicable.

#### 8.X.1.4	Abnormal Conditions

Void.

## <<< next change >>>

### 8.X.2	Data Collection Feedback

#### 8.X.2.1	General

The Data Collection Feedback procedure allows the NG-RAN node to provide feedback to LMF that positioning information from for one or more UEs is needed e.g., to support AI/ML assisted positioning with gNB side AI/ML model. This procedure applies only if the NG-RAN node is a gNB.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2.1: Data Collection Feedback procedure. Successful operation.

The NG-RAN node initiates the procedure by sending a DATA COLLECTION FEEDBACK message to the LMF to indicate a list of UEs for which positioning information is needed.

#### 8.X.2.3	Unsuccessful Operation

Not Applicable.

#### 8.X.1.4	Abnormal Conditions

## Void.

## <<< next change >>>

### 9.1.X	Messages for Data Collection Transfer Procedures

#### 9.1.X.1	DATA COLLECTION REPORT

This message is sent by the LMF to indicate positioning data for a list of UEs to the NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Label Collection List |  | 1 |  |  | YES | reject |
| >Label Collection List Item |  | 1..< maxnoDataItems> |  |  | - |  |
| >>UE ID | M |  | FFS |  | - |  |
| >>Label Information | M |  | FFS |  | - |  |
| Additional UE Selection Allowed | O |  | ENUMERATED(true, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoDataItems | Maximum no. of label data that can be provided by LMF. Value is FFS. |

#### 9.1.X.2	DATA COLLECTION FEEDBACK

This message is sent by the NG-RAN node to indicate that it needs positioning data from the LMF for additional UEs.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Data Request List |  | 1 |  |  | YES | reject |
| >Data Request List Item |  | 1..< maxnoDataItems> |  |  | - |  |
| >>UE ID | M |  | FFS |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoDataItems | Maximum no. of label data that can be provided by LMF. Value is FFS. |

<<< end of changes >>>