3GPP TSG-RAN WG3 Meeting #127	R3-250580

Athens, Greece, 17–21 February 2025

Title:	Discussions for AI/ML assisted NR-DC

Source:	Jio Platforms (JPL)

Agenda item:	11.4

Document Type:	Discussion and agreement

# Introduction

At the last RAN3#126 meeting minutes, what was agreed and reported for the AI/ML assisted mobility optimization for NR-DC follows:

- Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side.

- Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

- Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information?

In this document, we further explore the implications of the aforementioned agreements and we propose some basic ideas for the NR-DC AI/ML aided mobility optimization.

# General Discussion

The integration of Artificial Intelligence (AI) and Machine Learning (ML) into Next-Generation Radio Access Networks (NG-RAN) is grounded in the findings of the Rel-18 study item 3GPP TR37.817). Enhancements in data collection and signaling support across existing network interfaces are outlined in the Rel-18 AI/ML for NG-RAN Work Item, focusing on three key use cases: network energy efficiency, load balancing, and mobility optimization. To enable these use cases through AI/ML capabilities, input data from neighboring NG-RAN nodes is essential. Specific configurations for AI/ML data collection and reporting are communicated over the Xn interface using the Data Collection Reporting Initiation procedure, while the actual reporting is executed through the Data Collection Reporting procedure, as per 3GPP TS 38.423 figure 8.4.13.2-1 “Data Collection Reporting Initiation, successful operation”:

And TS 38.423 figure 8.4.14.2-1 “Data Collection Reporting, successful operation”:

## Data Collection NR-DC Mobility Optimization Decisions

Following the RAN3#126 meeting minutes “Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side” and for the “Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context”, for NR-DC use cases it was then agreed to reuse the Data Collection Reporting Initiation and Data Collection Reporting procedure to support MN requests to neighbors SNs gnBs for the proper acquisition of the UE performance. However in these messages there is no IE to include the UE Performance data collection framework to facilitate the MN to make a decision on the proper PSCell out of the SCG group for SN addition. To facilitate this procedure the Data Collection ID IE is proposed to be included in the XnAP S-NODE ADDITION REQUEST message, as per TS 38.423 Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation:

Proposal 1:  For NR-DC AI/ML assisted mobility optimization, MN sends the S-NODE ADDITION REQUEST including the Data Collection ID IE for UE performance acquisition.

After SN addition is successful in NR-DC mobility scenario, SN node should terminate the collection of UE performance, as being requested by the MN node in a prior step. Moreover for any other scenario of M-NG-RAN NODE MODIFICATION, S-NG-RAN NODE MODIFICATION, S-NG-RAN NODE CHANGE or S-NG-RAN NODE RELEASE (M-NG-RAN or S-NG-RAN node initiated), the SN takes should also terminate the collection of UE performance requested by the MN.

Proposal 2:  NR-DC AI/ML assisted mobility optimization, SN should terminate the collection of UE performance in SN successful addition, Handover and/or any scenario of SN modification, change or release.

## Predicted PSCell ID in Mobility Optimization Decisions for NR-DC

Following the RAN3#126 meeting minutes “Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information?” for NR-DC, during a SN Addition or Change and in order to assist the MN for a proper decision making action MN should communicate to the SN an non-mandatory (optional) proposal on the PSCell for a certain UE service or bearer based on the UE performance data collection data-set, which is inherently tied to a specific PSCell. Of course the SN gNB would consider that proposal in the final decision of the PSCell out of the SCG group in a non-restrictive way.

Proposal 3:  In NR-DC mobility optimization the MN gNB would propose to the SN node an optional information on the predicted PSCell out of the SCG group.

# Conclusions

From the analysis presented in this document, we have formulated the following proposals:

Proposal 1:  For NR-DC AI/ML assisted mobility optimization, MN sends the S-NODE ADDITION REQUEST including the Data Collection ID IE for UE performance acquisition.

Proposal 2:  NR-DC AI/ML assisted mobility optimization, SN should terminate the collection of UE performance in SN successful addition, Handover and/or any scenario of SN modification, change or release.

Proposal 3:  In NR-DC mobility optimization the MN gNB would propose to the SN node an optional information on the predicted PSCell out of the SCG group.