3GPP TSG RAN WG3 Meeting #127	R3-250581

Athens, Greece,  17 – 21 February, 2025

Title: 	Further Enhancements on AIML support for CCO

Source: 	Jio Platforms JPL

Agenda item:	11.3

Document Type:	Discussion and decision

# 1. Introduction

During last RAN3#126 meeting discussions on timing information follow:

Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message.

The following question arises in the content:

Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1

In this Discussion contribution, we provide further discussion related the aforementioned open issue to AI/ML based Coverage and Capacity Optimization (CCO).

# 2. Discussion

## gNB CU Future CCO state per Content

Referring to Figure 1, gNB2 CU will receive a predicted CCO issue from gNB1 CU, corresponding to two distinct coverage holes, which are likely associated with different SSB beams. If the future CCO issue report only includes information about the affected cells, gNB2 CU may have limited insight into which specific SSB beams are impacted. This lack of detailed information could make it challenging for gNB2 CU to accurately interpret the reported issue and identify the relevant SSB beams. As a result, it may be compelled to rely on exhaustive brute-force algorithms to deduce the corresponding SSB beams, if feasible.

Figure 1: Example of two coverage holes in one neighbor cell CCO issue scenario

Proposal 1: Any forthcoming CCO issue communicated over the Xn interface to an individual or a cluster of neighboring gNB CUs should, among other details, encompass both the associated cells and their respective SSB beams.

The 3GPP standard remains AI/ML model agnostic, placing the responsibility for energy-efficient AI/ML implementation on system designers. Key decisions, such as whether to train models in OAM or directly within the gNB, influence energy consumption, requiring optimal dimensioning to minimize resource usage. While 3GPP’s signaling enhancements for CCO predictions primarily focus on performance, sustainability considerations should also be integrated. The existing split-architecture CCO framework does not selectively target specific cells for CCO issue detection, as gNB-CU already oversees overall cell performance, failure cases, and degraded UE experiences for KPI reporting. However, given the rising emphasis on sustainability, energy-efficient CCO prediction mechanisms could potentially complement existing legacy CCO detection methods.

The gNB-DU should have the capability to inform the gNB-CU about which cells can undergo coverage adjustments if a CCO issue is anticipated. This adaptability may be influenced by SSB beams in the antenna characteristics. Providing this information to the gNB-CU would prevent unnecessary computational overhead and energy consumption for predicting CCO issues in cells where no corrective action is feasible. Additionally, RAN3 could explore leveraging this data for legacy CCO detection, enhancing overall energy and coverage efficiency.

Proposal 2: The gNB-CU to be informed about the specific cells where the gNB-DU can adjust the coverage state (i.e. SSB beams), allowing it to focus CCO issue prediction exclusively on those relevant cells.

To enhance optimization, the gNB-DU can inform the gNB-CU about available coverage configurations, which may vary in flexibility across different cells. Some cells might offer a broad range of alternative configurations, while others have limited options. Additionally, the gNB-DU could indicate whether these configurations are suitable for addressing coverage gaps or cell-edge capacity issues. By incorporating this information, the gNB-CU can optimize data collection, model training, and inference, ensuring computational and energy resources align with actionable corrective measures.

In legacy CCO, an AI/ML-predicted CCO issue is linked to a list of affected neighbor cells and corresponding SSB beams, which the gNB-CU typically relays to the gNB-DU via the F1 interface. Extending this mechanism to future CCO scenarios received from neighboring cells is a logical progression. If the gNB-CU forwards future CCO data via F1 but excludes the relevant SSB beams, the gNB-DU would struggle to identify the affected beams, requiring complex brute-force algorithms due to the absence of AI/ML-based intelligence.

Proposal 3: gNB-DU informs the gNB-CU about available coverage configurations and their suitability for resolving coverage or capacity issues with energy efficiency, including affected SSB beams to enhance efficiency and reduce computational complexity.

# 3. Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Proposal 1: Any forthcoming CCO issue communicated over the Xn interface to an individual or a cluster of neighboring gNB CUs should, among other details, encompass both the associated cells and their respective SSB beams.

Proposal 2: The gNB-CU to be informed about the specific cells where the gNB-DU can adjust the coverage state (i.e. SSB beams), allowing it to focus CCO issue prediction exclusively on those relevant cells.

Proposal 3: gNB-DU informs the gNB-CU about available coverage configurations and their suitability for resolving coverage or capacity issues with energy efficiency, including affected SSB beams to enhance efficiency and reduce computational complexity.

# References

[1] 3GPP TR 38.743 V2.0.0 (2024-08)