3GPP TSG-RAN WG3 Meeting #127	R3-250588

Athens, GR, 17-21 Feb, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.415 | CR | 0050 | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Paging Policy Indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Paging Policy Indicator (PPI) is used by the NG-RAN for paging policy differentiation associated with the DL packet, and its value range is between 0 and 7. It is said that the details can be referred to TS 23.501. However, the section 5.4.3.2 Paging Policy Differentiation or the section 5.8.5.4 QoS Enforcement Rule in TS 23.501 does not give the exact priority order. Given that the PPI is used for the NG-RAN to apply the paging policy for the RRC inactive UE, it is very important to specify the exact priority order. The simple way is to specify that the PPI values are ordered in decreasing order of priority, i.e., with 0 as the highest priority and 7 as the lowest priority, which is very similar to the Paging Priority IE in the NGAP Paging message |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To clarify that PPI values are ordered in decreasing order of priority, i.e., with 0 as the highest priority and 7 as the lowest priority. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only impacts the Paging Policy Indicator (PPI) for the PDU Session user plane protocol. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The priority order remains unclear for the Paging Policy Indicator. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 5.5.3.6	Paging Policy Presence (PPP)

Description: This parameter indicates the presence of the Paging Policy Indicator (PPI).

Value range: {0= Paging Policy Indicator not present, 1= Paging Policy Indicator present}.

Field length: 1 bit.

#### 5.5.3.7	Paging Policy Indicator (PPI)

Description: When present, the Paging Policy Indicator is used for paging policy differentiation (see details in 3GPP TS 23.501 [5]). This field applies to PDU sessions of IP type.

Value range: {0..23-1}. Values are ordered in decreasing order of priority, i.e., with 0 as the highest priority and 7 as the lowest priority.

Field length: 3 bits.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Change Ends