3GPP RAN WG3 Meeting #127	R3-250596

# Athen, Greece, 17th – 21st Feb 2025

Source:	CATT

Title:	(TP for TS 38.423) Introducing essentially per-DRB UE performance result for DC

Agenda Item:	11.4

Document for:	Discussion and decision

- Introduction

This document is a TP based on R3-250595.

- Text Proposal for TS 38.423

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

#### 9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  | – |  |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  | – |  |
| Average Packet Delay | O |  | 9.2.3.187 |  | – |  |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  | – |  |
| UE Performance DRB List |  | 0..1 |  |  | YES | ignore |
| >UE Performance DRB Item |  | 1 .. <maxnoofDRBs> |  | Each item is identified by at least one of the DBR ID IE or the QoS Flow List IE combined with the PDU Session ID IE. | – |  |
| >>DRB ID | O |  | 9.2.3.33 | Permitted to be used as the sole ID only for bearers configured with Xn-U tunnel(s) in DC scenario. | – |  |
| >>PDU Session ID | O |  | 9.2.3.18 |  | – |  |
| >>QoS Flow List | O |  | 9.2.1.4a |  | – |  |
| >>Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  | – |  |
| >>Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  | – |  |
| >>Average Packet Delay | O |  | 9.2.3.187 |  | – |  |
| >>Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

id-UE-PerformanceDRB-List,

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

FROM XnAP-Constants

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

#### -- U

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

UEPerformance ::= SEQUENCE {

dL-UE-AverageThroughput						BitRate					OPTIONAL,

uL-UE-AverageThroughput						BitRate					OPTIONAL,

uE-AveragePacketDelay						AveragePacketDelay		OPTIONAL,

uE-AveragePacketLossDL						PacketLossRate			OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEPerformance-ExtIEs} } OPTIONAL,

...

}

UEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-UE-PerformanceDRB-List					CRITICALITY ignore	EXTENSION UE-PerformanceDRB-List							PRESENCE optional },

...

}

UE-PerformanceDRB-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF UE-PerformanceDRB-Item

UE-PerformanceDRB-Item ::= SEQUENCE {

drb-ID										DRB-ID										OPTIONAL,

pDUSessionID								PDUSession-ID								OPTIONAL,

qoS-Flow-List								QoSFlows-List								OPTIONAL,

dL-UE-AverageThroughput						BitRate										OPTIONAL,

uL-UE-AverageThroughput						BitRate										OPTIONAL,

uE-AveragePacketDelay						AveragePacketDelay							OPTIONAL,

uE-AveragePacketLossDL						PacketLossRate								OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {UE-PerformanceDRB-Item-ExtIEs} } OPTIONAL,

...

}

UE-PerformanceDRB-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

9.3.7	Constant definitions

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

id-UE-PerformanceDRB-List										ProcedureCode ::=

//////////////////////////////////////////////////////////////////end//////////////////////////////////////////////////////////////////