3GPP RAN WG3 Meeting #127	R3-250597

# Athens, Greece, 17th – 21st February 2025

Source:	CATT

Title:	AI/ML in split gNBs

Agenda Item:	11.4

Document for:	Discussion and decision

- Introduction

This document is mainly a revised one based on R3-247472. The only difference is:

- Adding figure to emphasise that F1-CP interaction with gNB-DU is anyway needed for the delay metrics.

- Discussion

  - Exchanging UE performance

The IE “UE Performance” is a group of per-UE metric, and may contain three types of measurement results: Average UE throughput, Average Packet Delay, and Average Packet Loss DL. In DC scenario the data for a UE may be split into more than one node, so the MN has to collect both the UE performance measured in the MN itself and the one measured in the SN in order to generate the final “UE Performance” provided toward e.g. the source node.

The question is which node collects these three types of UE performance result in split scenarios—once this is decided the case for multiple bearer types in DC is decided as well. Our preference on this issue is following the legacy mechanism introduced for SON/MDT.

It was ever proposed that gNB-DUs can send the delay of its part toward the gNB-CU-UPs through the F1-U. It is true, but it cannot satisfy the requirement in AI/ML:

- In AI/ML the requesting node usually indicates the periodicity or time window of measurement, so as to provide proper training input (e.g. as an input of “reward”). But the method to report delay over F1-U does not include a configurable periodicity or time window.

- In AI/ML it is assumed that every node may reject data collection requests due to various reasons, and upon such reject further action may be taken, e.g. issuing a reject further back (gNB-DU rejects gNB-CU and then gNB-CU rejects the originating gNB), altering the parameter of request or cancelling requests to other nodes which are deemed to be useless. But the method to report delay over F1-U does not include an outcome IE.

Figure 1: F1-CP interaction with gNB-DU is anyway needed for the delay metrics.

Observation 1: The mechanics of delivering delay over F1-U cannot be configured with reporting periodicity and is without any explicit acceptance or reject indication, therefore not suitable for AI/ML purpose.

The cases on throughput and packet loss are simple:

- Both downlink throughput and uplink throughput are measured by the gNB-DU as defined in Section 6.3.1.4.1 and Section 6.3.1.4.2 of TS 28.558 respectively. There is indeed an “Average DL UE buffered Throughput per DRB” metric defined in Section 5.1.1.3.7 of TS 28.552, but it is claimed that “For very bursty traffic with burst duration within the interval of one DDDS the measured values can be unprecise and cannot be compared across vendors.”

- Downlink packet loss is measured by the gNB-DU as defined in Section 4.2.1.5.1 of TS 38.314.

Observation 2: The “Average DL UE buffered Throughput per DRB” collected by the gNB-CU-UP is described as “can be unprecise and cannot be compared across vendors” in Section 5.1.1.3.7 of TS 28.552, therefore is not preferred for AI/ML purpose.

The case is quite complex on packet delay as shown in Section 4.2.1.2.1 of TS 38.314. In the downlink direction it is defined as the sum of 4 items:

- Average delay DL air-interface in Section 6.3.1.1.1 of TS 28.558, wrongly defined as measured by the gNB-CU-UP but should actually be measured by the gNB-DU (as in Section 5.1.3.1.1 of TS 28.552);

- Average delay in RLC sublayer, measured by the gNB-DU as defined in Section 6.3.1.1.2 of TS 28.558, wrongly defined as measured by the gNB-CU-UP but should actually be measured by the gNB-DU (as in Section 5.1.3.3.3 of TS 28.552);

- Average delay on F1-U, measured by the gNB-CU-UP as defined in Section 6.3.1.1.3 of TS 28.558;

- Average delay DL in CU-UP, measured by the gNB-CU-UP as defined in Section 6.3.1.1.4 of TS 28.558.

In the uplink direction it is defined as the sum of 5 items:

- UL PDCP packet average delay, measured by the UE as defined in Section 4.3.1.1 of TS 38.314 (it is wrongly defined as measured by the gNB-CU-UP in TS 28.558);

- Average over-the-air interface packet delay, measured by the gNB-DU as defined in Section 4.2.1.2.2 of TS 38.314;

- Average RLC packet delay, measured by the gNB-DU as defined in Section 4.2.1.2.3 of TS 38.314;

- Average delay on F1-U, measured by the gNB-CU-UP as defined in Section 6.3.1.1.3 of TS 28.558;

- Average PDCP re-ordering delay, measured by the gNB-CU-UP as defined in Section 4.2.1.2.4 of TS 38.314.

Nevertheless, it is not necessary to report every metric separately. The delay measured in gNB-DU can be combined into one single metric, and so can the delay measured in gNB-CU-UP.

As the summary, there are 5 metrics collected in the gNB-DU whereas 2 metrics collected in the gNB-CU-UP. The remaining one is collected by the UE, which is reported toward the gNB-CU-CP directly and thus has no impact on either E1AP or F1AP.

Proposal 1: The following 5 metrics for UE performance is collected in the gNB-DU (aligned with legacy SON/MDT, and the case in DC should be aligned with this):
-	DL throughput;
-	UL throughput;
-	DL packet loss;
-	Average DL delay air-interface plus Average DL delay in gNB-DU;
-	Average UL delay air-interface plus Average UL delay in gNB-DU.

Proposal 2: The following 2 metrics for UE performance is collected in the gNB-CU-UP (aligned with legacy SON/MDT, and the case in DC should be aligned with this):
-	Average delay F1-U plus Average DL delay in gNB-CU-UP;
-	Average delay F1-U plus Average UL delay in gNB-CU-UP.

  - Introducing new metrics

We observe one type of information which is not introduced in the Rel-18 WI but essential in split scenario: the UE traffic volume. It was ever mentioned in the discussion for the Energy Saving use case, as the “Energy Efficiency” is literally the data volume divided by the energy consumption.

Companies found it hard to describe traffic between neighbouring gNBs, but this is not the case within one gNB: the source gNB-CU-CP knows everything from the QoS requirement toward the UE radio status (i.e. RSRP etc), with the only exception is the data rate for each flow.

Therefore we propose enhancing E1AP so that the gNB-CU-UP can provide the data rate for each flow toward the gNB-CU-CP. (The model for energy saving should surely reside in the gNB-CU-CP.)

Proposal 3: To enhance E1AP so that the gNB-CU-UP can provide the data rate for each QoS flow toward the gNB-CU-CP, in order to cover the use case of energy saving (in which the gNB acts as the source node).

  - Message to use

For F1AP, we agreed at last meeting that the data collection procedures should be introduced. For E1AP, there are also many metrics needs to be delivered, many of which the same as the metrics included in XnAP Data Collection Update messages. Therefore we propose introducing similar procedures for alignment.

For measurement objects related to one single UE, a measurement ID pair for reference is added into UE-associated messages, such as F1AP UE Context Setup Request messages, E1AP Bearer Context Setup Request messages.

Proposal 4: To introduce new data collection procedures into E1AP, similar to the Rel-18 one introduced into XnAP. For measurement objects related to one single UE, a measurement ID pair is added into UE-associated messages for reference.

  - New node is configured sometime during the feedback collection window

After a handover take place, the target node may need to collect the feedback upon the prior request from the source node. It is possible that a new node is added to serve the UE while it is still right within the feedback collection window:

- The UE further experiences an intra-CU inter-DU handover (or PSCell change), thus a new gNB-DU (or even gNB-CU-UP) starts to serve the UE, and some performance needs to be collected by the new gNB-DU (or gNB-CU-UP).

- The target node further configures an SCG for the UE, and some performance needs to be collected by the SN or secondary gNB-DU.

- The UE further experience an SN change, and some performance needs to be collected by the new SN.

Generally speaking, the existing data collection mechanism can be reused for such case, except the length of collection time duration: the duration is now a reduced value (the original duration minus the time length between the two RRC reconfigurations), rather than the original one.

The source node expects the feedback one “collection time duration” after the handover toward the target node take places (after which the source node may delete the UE context for data collection), so the target node must collect all information and send it back to the source node in time. Therefore, the new serving node must collect information within the reduced duration and send it back instantly.

Considering that the new serving node has no idea of the time length between the two RRC reconfigurations, the new serving node needs to be provided with a “remaining” collection time duration. Considering that the time length between the two RRC reconfigurations varies from one UE to another, we prefer indicating the “remaining” collection time duration within a UE-associated message, i.e. the message containing the Data Collection ID.

Proposal 5: To introduce a “remaining collection time duration” into the UE-associated message which contains the Data Collection ID, in order to cover the case that a new node is configured sometime during the original data collection window.

- Conclusion

Observation 1: The mechanics of delivering delay over F1-U cannot be configured with reporting periodicity and is without any explicit acceptance or reject indication, therefore not suitable for AI/ML purpose.

Observation 2: The “Average DL UE buffered Throughput per DRB” collected by the gNB-CU-UP is described as “can be unprecise and cannot be compared across vendors” in Section 5.1.1.3.7 of TS 28.552, therefore is not preferred for AI/ML purpose.

Proposal 1: The following 5 metrics for UE performance is collected in the gNB-DU (aligned with legacy SON/MDT, and the case in DC should be aligned with this):
-	DL throughput;
-	UL throughput;
-	DL packet loss;
-	Average DL delay air-interface plus Average DL delay in gNB-DU;
-	Average UL delay air-interface plus Average UL delay in gNB-DU.

Proposal 2: The following 2 metrics for UE performance is collected in the gNB-CU-UP (aligned with legacy SON/MDT, and the case in DC should be aligned with this):
-	Average delay F1-U plus Average DL delay in gNB-CU-UP;
-	Average delay F1-U plus Average UL delay in gNB-CU-UP.

Proposal 3: To enhance E1AP so that the gNB-CU-UP can provide the data rate for each QoS flow toward the gNB-CU-CP, in order to cover the use case of energy saving (in which the gNB acts as the source node).

Proposal 4: To introduce new data collection procedures into E1AP, similar to the Rel-18 one introduced into XnAP. For measurement objects related to one single UE, a measurement ID pair is added into UE-associated messages for reference.

Proposal 5: To introduce a “remaining collection time duration” into the UE-associated message which contains the Data Collection ID, in order to cover the case that a new node is configured sometime during the original data collection window.

We also draft two TPs for introducing AI/ML function and data-collection-related messages into E1 and F1 as in the annexes.

- Reference

[1] R3-24xxxx; Enhancement of UE performance metrics; CATT.

- Text Proposal for TS 37.480

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

# 5	Functions of the E1 interface

## 5.1	General

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 5.1.x	AI/ML support function

This function is used to support AI/ML for NG-RAN, including initiation of data collection and reporting of collected data.

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

# 6	Procedures of the E1 interface

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

## 6.y	AI/ML support procedures

The following procedures are used to initiate data collection and report collected data to support, e.g., AI/ML for NG-RAN:

-	Data Collection Reporting Initiation

-	Data Collection Reporting

///////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////

- Text Proposal for TS 38.470

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

## 5.2	F1-C functions

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 5.2.x	AI/ML support function

This function is used to support AI/ML for NG-RAN, including initiation of data collection and reporting of collected data.

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

## 6.1	Control plane procedures

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 6.1.y	AI/ML support procedures

The following procedures are used to initiate data collection and report collected data to support, e.g., AI/ML for NG-RAN:

-	Data Collection Reporting Initiation

-	Data Collection Reporting

///////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////