3GPP RAN WG3 Meeting #127	R3-250598

# Athens, Greece, 17th – 21st February 2025

Source:	CATT

Title:	Potential RAN3 impacts of LP-WUS

Agenda Item:	18.2

Document for:	Discussion and decision

- Introduction

This document is mainly a revised one based on R3-247473. Some main differences include:

- Removing the part discussing UE capability, which partly contradicts with a WA last meeting.

- Removing the part discussing System Information, which was concluded last meeting.

- Adding a part to analyse the problem of missing UE capability mentioned in the Reply LS from SA2.

- Discussion

  - UE paging subgroup ID

One LS [1] was received from RAN2, stating that:

1) The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups. 2) RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. 3) RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. 4) For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. 5) For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

In addition, RAN1 has assumed that:

Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 32.

For CN-based subgrouping, RAN2’s assumptions are consistent with RAN3’s current progress. Considering that RAN1 has decided the maximum number of total subgroups and for PEI the maximum number of total subgroups is identical to the maximum number of CN-assigned subgroups, we can now assume directly that the maximum number of CN-assigned subgroups is 32 and generates BL CRs for NGAP and F1AP capturing it accordingly. The case on XnAP was captured as pending RAN2 last meeting, thus we do not propose any BL CR for XnAP now.

Proposal 1: Introduce an “LP-WUS assistance information” IE into the NGAP PAGING message, the “Core Network Assistance Information for RRC INACTIVE” structure in NGAP, and the F1AP PAGING message. The “LP-WUS assistance information” IE includes a “CN Subgroup ID” IE with type “INTEGER (0..31, …)”.

For UE-ID-based subgrouping, the case is relatively complex. RAN2 agreed that the formula used in calculating UE-ID-based subgroup for LP-WUS is similar with the one for PEI, but this may mean the “Extended UE Identity Index Value” provided in the “Core Network Assistance Information for RRC INACTIVE” structure—which is 16-bit, used in RAN-initiated paging—may not be enough:

- Up to 10 bits in the UE ID are used in calculating POs (if no short eDRX), and up to 3bits in the UE ID are used in calculating the subgroup of PEI, which shall never overlaps with the prior 10 bits according to the formula. There may be only 3 bits left. If 3GPP agreed that the bits used in calculating the subgroup of LP-WUS shall neither overlap with the ones used in calculating the subgroup for PEI, there is no way for the gNB to determine the UE’s subgroup (requiring 5 bits) by using the remaining 3 bits.

- Up to 12 bits in the UE ID are used in calculating POs if the UE is configured with short eDRX, so there may be only 4 bits left. If 3GPP agreed that LP-WUS can be used together with short eDRX, there is no way for the gNB to determine the UE’s subgroup (requiring 5 bits) by using the remaining 4 bits.

For the most extreme case that all of short eDRX, PEI and LP-WUS exists, the total number of the required bits is 20.

Observation 1: The current 16-bit “Extended UE Identity Index Value” delivered in the “Core Network Assistance Information for RRC INACTIVE” structure may not be enough for determining the subgroup of LP-WUS.

The simplest solution for this problem is extending—for another time—the UE Identity Index Value provided by the core network. However, this value is acquired by every RAN node when serving the UE and thus can be used as tracking ID by an attacker if the IE is too unique. We believe RAN3 shall consult SA2/SA3 before deciding to extend.

Observation 2: The “Core Network Assistance Information for RRC INACTIVE” structure is available at every gNB which serves the UE. Further extending the UE Identity Index Value provided in this structure may cause concern on privacy (enabling attackers to track UEs) and thus requires allowance from SA2/SA3.

If SA2/SA3 rejects the proposal of further extending the UE Identity Index Value, RAN2 will probably have to find other solutions, which in turn will probably cost some time. Therefore, we believe SA2/SA3 should be consulted as early as possible.

Observation 3: If SA2/SA3 rejects the proposal of further extending the UE Identity Index Value, RAN2 will probably have to find other solutions, which in turn will probably cost some time.

Proposal 2: Send an LS to SA2/SA3 (as a reply LS to the SA2’s) pointing out the problem stated in Observation 1 and asking SA2/SA3 whether it is acceptable to introduce a further extended UE ID—20-bits at least—into the “Core Network Assistance Information for RRC INACTIVE” structure.

  - Emergency service

A UE using LP-WUS will experience significant extra paging delay compared with UEs not monitoring only conventional (Rel-15) POs. This may be suboptimal for emergency service. If there is an (already established) emergency session between the UE and the network, and the UE is currently not in RRC_CONNECTED state, there may be some benefit to prevent the UE from using LP-WUS.

The case is simple at the UE side: A UE knows whether there is an established emergency session (by checking e.g. its own NAS context), and if there is, the UE may simply act as if it does not support LP-WUS for shorter paging delay.

The case in CN is simple as well: The only impact on CN is assigning the CN-assigned subgroup ID. As long as the UE does not indicate its support of LP-WUS, the CN will not assign—the same method as in PEI.

The case in RAN, however, is not so easy, since RAN is not provided with necessary information whether there is an emergency session for a UE. There are a few possible attitudes toward this issue:

- Simply ignores the drawback. The delay caused by LP-WUS is probably within 1000ms (waiting for RAN1 to decide), comparable with the delay caused by PO periodicity (and occasionally missing a couple of PO when monitoring). This may be acceptable.

- gNB begins to send Uu paging from the first available PO after it knows it shall page, rather than the first available PO after LP-WUS wake up period. Not only the UE with established emergency session but also UEs not monitoring LP-WUS due to other reason (e.g. out of enter condition) will benefit from such implementation. (Note that RAN2 has agreed that the gNB is assumed to be unaware of whether a UE monitors LP-WUS.)

- Assume that emergency session always come with the highest paging priority (an IE within NGAP paging message and/or NG-U DL packet). The gNB begins to send Uu paging from the first available PO for the highest paging priority.

- The UE updates its Uu capability (by UE Assistance Information maybe) as “not supporting LP-WUS” once an emergency session is established, fully aligned with the method in NAS. The gNB will of course act as legacy since it considers the UE not supporting LP-WUS.

- The core network indicates that a session is an emergency one, or an NG paging is triggered for an emergency session, in the corresponding NGAP messages.

The fourth attitude requires adequate design in RAN2, whereas the last attitude requires enhancement on RAN3 stage-3 specification—other attitudes are fully implementation-based and independent with each other (i.e. there would not be any extra issue if one gNB uses one implementation and another gNB uses another). We propose RAN3 to discuss this issue.

Proposal 3: To discuss the issue of emergency service handling based on the five attitudes listed above.

  - The question from SA2

The LS received from SA2 [3] pointed out a problem: the “UE Radio Capability for Paging Information” was created by RAN based on the UE radio capability provided over the Uu RRC. As a result, paging will be missed if the current camped cell supports a new feature but the node generates the “UE Radio Capability for Paging Information” does not support.

The concerned IE structures in TS 38.331 are as following:

- UE-RadioPagingInfo The IE UE-RadioPagingInfo contains UE capability information needed for paging. UE-RadioPagingInfo information element -- ASN1START -- TAG-UE-RADIOPAGINGINFO-START UE-RadioPagingInfo-r17 ::= SEQUENCE { -- R1 29-1: Paging enhancement pei-SubgroupingSupportBandList-r17 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL, ... } -- TAG-UE-RADIOPAGINGINFO-STOP -- ASN1STOP #### – UERadioPagingInformation This message is used to transfer radio paging information, covering both upload to and download from the 5GC, and between gNBs. Direction: gNB to/ from 5GC and gNB to/from gNB // skip UERadioPagingInformation-v15e0-IEs ::= SEQUENCE { dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2 ENUMERATED {supported} OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1700-IEs OPTIONAL } UERadioPagingInformation-v1700-IEs ::= SEQUENCE { ue-RadioPagingInfo-r17 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17) OPTIONAL, inactiveStatePO-Determination-r17 ENUMERATED {supported} OPTIONAL, numberOfRxRedCap-r17 ENUMERATED {one, two} OPTIONAL, halfDuplexFDD-TypeA-RedCap-r17 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1800-IEs OPTIONAL } UERadioPagingInformation-v1800-IEs ::= SEQUENCE { numberOfRxERedCap-r18 ENUMERATED {one, two} OPTIONAL, supportOf2RxXR-r18 ENUMERATED {supported} OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1840-IEs OPTIONAL } UERadioPagingInformation-v1840-IEs ::= SEQUENCE { dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18 ENUMERATED {supported} OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL }

The PEI-related information, as received by RAN, is included in a “UE-RadioPagingInfo-r17” over the Uu RRC. This IE structure should be copied and pasted—separately—into the “UERadioPagingInformation” to be provided to the CN or neighbour RAN nodes for RAN paging. It is obvious that only Rel-17 gNBs or later ones can perform such copying and pasting, i.e. Rel-16 gNBs or older ones cannot do this, resulting in missing of UE capability for paging. There is no easy solution for such problem, i.e. the problem caused by heterogeneous deployment of Rel-16 or earlier version of gNBs and Rel-17 or later version of gNBs.

However, there may not be any problem for features introduced after Rel-17, e.g. LP-WUS. Nevertheless, two conditions are needed:

- The capability for such new feature (e.g. LP-WUS) is encoded as an extension field within the “UE-RadioPagingInfo-r17” structure, rather than within some new structure such as “UE-RadioPagingInfo-r19”.

- The gNB which received the UE capability over Uu RRC always copies and pastes the entire “UE-RadioPagingInfo-r17” into the “UERadioPagingInformation”, including every ASN.1 extension within “UE-RadioPagingInfo-r17”, regardless of whether the gNB can decode the extension or not. Such handling is feasible from the aspect of ASN.1 encoding rule, but there is no specification mandating such handling.

The first condition is fully within RAN2 scope, but the second condition may be within RAN3 scope. We propose RAN3 check and confirm whether every Rel-17 gNB or later one follows the handling described in the second condition.

Proposal 4: We propose RAN3 checking and confirming whether every Rel-17 gNB or later one always copies and pastes the entire “UE-RadioPagingInfo-r17” into the “UERadioPagingInformation”, including every ASN.1 extension within “UE-RadioPagingInfo-r17”, regardless of whether the gNB can decode the extension or not, and then draft a reply LS accordingly.

- Conclusion

Proposal 1: Introduce an “LP-WUS assistance information” IE into the NGAP PAGING message, the “Core Network Assistance Information for RRC INACTIVE” structure in NGAP, and the F1AP PAGING message. The “LP-WUS assistance information” IE includes a “CN Subgroup ID” IE with type “INTEGER (0..31, …)”.

Observation 1: The current 16-bit “Extended UE Identity Index Value” delivered in the “Core Network Assistance Information for RRC INACTIVE” structure may not be enough for determining the subgroup of LP-WUS.

Observation 2: The “Core Network Assistance Information for RRC INACTIVE” structure is available at every gNB which serves the UE. Further extending the UE Identity Index Value provided in this structure may cause concern on privacy (enabling attackers to track UEs) and thus requires allowance from SA2/SA3.

Observation 3: If SA2/SA3 rejects the proposal of further extending the UE Identity Index Value, RAN2 will probably have to find other solutions, which in turn will probably cost some time.

Proposal 2: Send an LS to SA2/SA3 (as a reply LS to the SA’s) pointing out the problem stated in Observation 1 and asking SA2/SA3 whether it is acceptable to introduce a further extended UE ID—20-bits at least—into the “Core Network Assistance Information for RRC INACTIVE” structure.

Proposal 3: To discuss the issue of emergency service handling based on the five attitudes listed above.

Proposal 4: We propose RAN3 checking and confirming whether every Rel-17 gNB or later one always copies and pastes the entire “UE-RadioPagingInfo-r17” into the “UERadioPagingInformation”, including every ASN.1 extension within “UE-RadioPagingInfo-r17”, regardless of whether the gNB can decode the extension or not, and then draft a reply LS accordingly.

We draft an LS [2] according to Proposal 2.

- Reference

[1] R3-247007/R2-2409226; LS on LP-WUS subgrouping; RAN2.

[2] R3-25xxxx; [Draft] Reply to LS on LP-WUS subgrouping; CATT.

[3] R3-250021/S2-2412876; LS Reply on LP-WUS subgrouping; SA2.