3GPP TSG-RAN WG3 #127	          R3-250635

# Athens, Greece, 17th – 21th Feb 2025

Agenda item:	15.2

Source: 	Samsung

Title: 	Discussion on support of regenerative payload for IoT NTN

Document for:	Discussion and decision

Introduction

Last meeting primarily discussed the support of regenerative payload for IoT NTN, and the following agreement is achieved,

RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later.

Check progress in RAN1/2…

In this contribution, we continue to discuss the support of regenerative payload for IoT NTN.

Discussion

As indicated by the WID in RP-234077, the support of S&F satellite operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure. And a typical scenario can be illustrated as follows,

Assuming the full eNB is on board the satellite, at time instance T1, the satellite can provide the service link with the UE without the feeder link. After obtaining UL data from UE which is to be transmitted to the CN, the Satellite needs to store such UL data during T1->T2->T3. At time instance T3, when the satellite establishes the feeder link with the NTN GW, such UL data from UE can be further transmitted to the CN for further operations.

It should be noted that in TR 23.700-29, SA2 has discussed the possibility of the support of S&F satellite operation with both single satellite deployment and multiple satellite deployment. More specifically, for single satellite deployment, only one regenerative payload serves a specific UE; while for multiple satellite deployment, multiple regenerative payloads serve a specific UE. Consequently, RAN3 can also consider both scenarios with single satellite deployment and multiple satellite deployment.

According to the discussion last time, operators are quite interested in the multiple satellite deployment, depending on the fact that the regression time of some satellite will be as long as 12 hours. Although the service is non real time and delay tolerant, it is still too long for a UE to perform MO/MT data transmission with such long regression time satellite with full CN/split MME onboard. So it is reasonable to consider both single and multiple satellite deployment to support S&F operation for a UE.

However, if we only consider full CN/split MME onboard architecture, how to apply the multiple satellite deployment to such architectures is unclear. The first issue is that whether the UE should make multiple registrations to multiple satellites with MME onboard. In our opinion, in order to support the multiple satellite deployment scenario, more SA2 progress is needed.

Proposal 1: The feasibility to support multiple satellite deployment needs more progress from SA2.

With the above understanding, the next is to check whether there has to be any update on the current basic procedures, and we believe such basic procedures may include the following ones,

- (Initial) Attach procedure

- TAU procedure

- Control Plane CIoT EPS Optimisation

- User Plane CIoT EPS Optimisation

Moreover, for procedures of CP/UP CIoT EPS Optimisation, more detailed procedures including MO-EDT, MT-EDT and transmission using PUR can be further investigated.

For CP/UP CIoT EPS optimization, our understanding is that for single satellite deployment, the current call flows for CP CIoT can be reused without further enhancement.

While for the UP CIoT,

Since SA2 has concluded that the S-GW is on the ground, so for S&F operation, the step 1 is performed when the service link is available while the feeder link is unavailable, and the steps (2/)3-7 can be performed when the feeder link is available while the service link is unavailable. So the question is,

- Whether to support UP CIoT solution for S&F operation?

Proposal 2: RAN3 is kindly asked to discuss whether to support UP CIoT solution for S&F operation.

Conclusion

In this paper, we provide primary discussion on support of regenerative payload for IoT NTN. The following proposals are provided,

Proposal 1: The feasibility to support multiple satellite deployment needs more progress from SA2.

Proposal 2: RAN3 is kindly asked to discuss whether to support UP CIoT solution for S&F operation.

References