3GPP TSG-RAN WG3 #127	          R3-250638

# Athens, Greece, 17th – 21th Feb 2025

Agenda item:	22.2

Source: 	Samsung

Title: 	Discussion on multi-hop for SL relay

Document for:	Discussion and decision

Introduction

The R19 SL Relay WI begins to discuss, with the following RAN3 scope,

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE):

First Priority:

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

Second Priority in order of importance:

- Intra-gNB direct to multi-hop indirect path switching

- Intra-gNB single-hop indirect to multi-hop indirect path switching

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this contribution, we discuss the fundamental aspects to be specified.

Discussion

The discussion on R19 SL relay WI begins, and the main objective for RAN3 is to,

- Control plane procedure to support additional hops relays on top of R17 U2N relay.

- Specify the signalling support for intra-gNB service continuity scenarios for multi-hop U2N relay.

Before we dig into details, RAN2 has discussed the scenarios as well as the control plane procedures for multi-hop relay with the following agreements which are related to RAN3,

- Distinguish the “last” relay UE as the U2N relay UE directly connected to the gNB, for RAN2 discussion

- Distinguish the “intermediate” relay UEs as the other relay UEs, for RAN2 discussion

- Distinguish the “first” relay UE as the intermediate relay UE directly connected to the remote UE, for RAN2 discussion

- RAN2 intend to design mechanisms from the start of the WI that are flexible enough to be adapted to at least two additional hops in Rel-19.

- Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

  - Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

  - Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment.

  - All agreements in this WI apply to both these cases unless otherwise specified.

- The following connections are assumed as a baseline to be needed:

  - From last Relay UE perspective:

    - A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

    - A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

  - From intermediate relay UE perspective (including first relay UE):

    - A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

    - An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

    - FFS what RRC states are supported for the intermediate relay UE.

  - From U2N Remote UE perspective:

    - A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

    - An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.

According to RAN2 agreement, we can observe that,

- The terminology on relay UEs has changed.

- The mechanisms are designed to support at least two additional hops in R19 instead of at most two additional hops.

- Support the baseline procedure that when remote UE transits to RRC Connected, all the intermediate relay UEs should transit to RRC Connected in advance.

In our understanding, RAN3 will still consider the scenario to support multi-hop relay in case of CU-DU split architecture, as in R17/18.

Proposal 1: Consider CU-DU split architecture to support multi-hop relay.

Recall that in R17/18, to support the single-hop relay, over F1AP,

- For the Relay UE context, one list of Uu RLC channel (between gNB and Relay UE) and one list of PC5 RLC channel (between Relay UE and Remote UEs) are needed to be coordinated between gNB-CU and gNB-DU.

- For the Remote UE context, one list of PC5 RLC channel (between Relay UE and Remote UE) is needed to be coordinated between gNB-CU and gNB-DU.

While for R19 intermediate Relay UE, two separate lists of PC5 RLC channel (each list for one of two adjacent UEs) are needed to be coordinated between gNB-CU and gNB-DU. And the current F1AP spec is not enough for the intermediate relay UE context setup/modification.

Proposal 2: Two sets of PC5 RLC Channel to be Setup/Modified/Release list are needed for the intermediate relay UE context setup/modification.

Another topic is about the intra-gNB service continuity. According to the WID, the intra-gNB multi-hop indirect to direct/single-hop indirect path switching is with the first priority; while the intra-gNB direct/single-hop indirect to multi-hop indirect path switching is with the second priority. In our understanding, we can start with 1st priority call flows, and the following call flows are tentative ones,

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework (Inter-gNB-DU switch case)

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework (Intra-gNB-DU switch case)

Proposal 3: For intra-gNB multi-hop indirect to single-hop indirect path switching scenario, consider both inter-gNB-DU switch case and intra-gNB-DU switch case.

Conclusion

In this paper, we provide primary discussion on multi-hop relay. The following proposals are provided,

Proposal 1: Consider CU-DU split architecture to support multi-hop relay.

Proposal 2: Two sets of PC5 RLC Channel to be Setup/Modified/Release list are needed for the intermediate relay UE context setup/modification.

Proposal 3: For intra-gNB multi-hop indirect to single-hop indirect path switching scenario, consider both inter-gNB-DU switch case and intra-gNB-DU switch case.

Reference