3GPP TSG-RAN WG3 Meeting #127	R3-250650

Athens, Greece, Feb. 17th-21st, 2025

Agenda Item:	14.3

Source:	CSCN

Title:	Discussion on support of regenerative payload for NR NTN

Document for:	Discussion

1	Introduction

In the last RAN3 #126 meeting, there are some discussions as below about NG interface management and supporting of RRC_INACTIVE [1].

NG Interface management Issue: Whether to support NG suspend/Resume? Support of RRC_INACTIVE: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Any standard impact? Option 1, UE context is forwarded to the next satellites Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

In addition, SA2 and SA5 have replied their liaisons from RAN3 on OAM requirements to support regenerative payload [2][3].

In this contribution, we will further discuss the following issues on the support of regenerative payload.

Issue 1: Reply LS to SA2 and SA5 on Supported TAI list

Issue 2: Support of RRC_INACTIVE

2	Discussion

Issue 1: Reply LS to SA2 and SA5 on Supported TAI list

RAN3 has reached a consensus in the #125bis meeting on OAM based TAI list configuration and sent a LS to SA2 and SA5, then SA2 and SA5 replied their LS.

Essentially, OAM configuration is the simple solution, only the gNB and AMF are required to maintain the TAI list separately, thus reducing excessive signalling interaction on NGAP (e.g. RAN Configuration Update). In addition to configuration for gNBs, the most obvious difference in OAM based solution is that OAM requires to configure the TAI list for all gNBs that AMF serves. Of course, the cost of OAM based solution is that it will may increase the work load of network management, but the LS replied by SA5[3] also agree with OAM based solution and is ready to consider normative work in R19.

SA5 would like to inform RAN3 that SA5 Rel 19 study “SP-231733 New SID on Management Aspects of NTN Phase 2” has investigated and agreed the potential solutions of providing the information of supported TAIs for NTN gNB to AMF in TR 28.874. Further, SA5 will consider the solution in normative work in Rel-19.

Furthermore, legacy NGAP mechanism for supported TAI list can still work in NTN regenerative scenario. Therefore, further enhancements to NGAP for supported TAI list seem unnecessary in order to minimize the impacts on existing specifications. However, SA2 raises concerns in LS[2] regarding the potential impacts on the existing procedures related to paging, sequencing of OAM updates, and OAM update frequency.

- SA2 discussed whether the OAM solution would have impacts to the existing procedures related to paging and sequencing of OAM updates and RAN3 updates towards the AMF, e.g. in NGAP Setup/configuration update etc. However, it was unclear to SA2 how the OAM solution would work based on the description in the incoming LS to be able to determine whether there are impacts. - The frequency of updates by OAM was not clear to SA2 and whether the information provided to the AMF is a timetable of which TAIs the gNB will support at what time, or whether the updates are provided frequently as the satellite moves updating the AMF each time a change occurs.

If the AMF has already configured the TAI list, it can follow the existing paging procedure to page the UE via the gNBs associated with the TAI list. Therefore, the paging process itself is not expected to have any impact. However, there is a potential issue that AMF may send the paging message to an incorrect gNB if the TAI list is not update timely. Consequently, it is essential for the AMF to clearly understand which TAI list each gNB supports at any given time, in accordance with the OAM configuration.

SA2 also considers it necessary to determine what the OAM provides for gNB and AMF and how often the OAM configuration is updated. The update frequency is related to the satellite movement and ephmeris information. We consider the maintenance of TAI list is a timetable with second-level granularity, which can be updated on an hourly or daily basis. RAN3 can give a reference on the configuration at the working group level, while the final design should be determined by SA5.

Proposal 1: The timetable configured by OAM with a TAI list with granularity in seconds, and updated hourly or daily basis, can serve as a reference in RAN3. The specific configuration can be determined by SA5.

As above, We support both legacy and OAM-based solutions for the TAI list, and which one is applied can be determined by network implementation and operator’s policy.

Proposal 2: In addition to the legacy mechanism, the OAM-based solution should also be supported, and which applied subject to operator’s policy.

Issue 2: Support of RRC_INACTIVE

RNA Configuration

The gNB can assign an RNA (RAN-based Notification Area) to the RRC_INACTIVE UE via RRC Release signal (i.e. SuspendConfig - RAN-NotificationAreaInfo). The RNA in current spec can be configured in any of the following ways:

RAN-NotificationAreaInfo ::= CHOICE { cellList PLMN-RAN-AreaCellList, ran-AreaConfigList PLMN-RAN-AreaConfigList, ... } PLMN-RAN-AreaCellList ::= SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell PLMN-RAN-AreaCell ::= SEQUENCE { plmn-Identity PLMN-Identity OPTIONAL, -- Need S ran-AreaCells SEQUENCE (SIZE (1..32)) OF CellIdentity } PLMN-RAN-AreaConfigList ::= SEQUENCE (SIZE (1..maxPLMNIdentities)) OF PLMN-RAN-AreaConfig PLMN-RAN-AreaConfig ::= SEQUENCE { plmn-Identity PLMN-Identity OPTIONAL, -- Need S ran-Area SEQUENCE (SIZE (1..16)) OF RAN-AreaConfig } RAN-AreaConfig ::= SEQUENCE { trackingAreaCode TrackingAreaCode, ran-AreaCodeList SEQUENCE (SIZE (1..32)) OF RAN-AreaCode OPTIONAL -- Need R }

Option 1: Cell List

Option 2: Tracking Area List

Option 3: Tracking Area List, RAN Area List

In NTN, if option1 is configured, UEs need to trigger RNA update procedure frequently towards the gNB on board even the UE remains stationary. In order to reduce the frequency of RNA updates and reduce signaling overhead between the UEs and the gNB on board, it is better to configure a geo-fixed RNA for regenerative payload NR NTN, i.e. A list of Tracking Area Code.

Observation 1: Geo-fixed RNA configuration can reduce RNA update signaling overhead with satellite moving.

Proposal 3: RNA could be configured as a fixed geographical area (i.e. A list of Tracking Area Code) in regenerative payload NR NTN.

UE context retrieval

The UE context remains in the UE and the last serving gNB when the UE is in the RRC_INACTIVE mode. Given that it may take several hours for the gNB to reconnect with the UE, if the UE context cannot be retrieved through the satellite with ISL (Xn interface), then the NW initiates RRC Setup in legacy way. In response to the open issue of UE context retrieval in RRC_INACTIVE, several companies have proposed multiple options as follows:

Option 1, UE context is forwarded to the next satellites

Option 2, UE context is kept on a fixed node (on-ground gNB or AMF)

Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

The introduction of RRC_INACTIVE offers multiple advantages for small data transmissions, including the reduction of signalling overhead and UE power consumption, as well as the improvement of access latency in TN. Since the UE context is already saved in UE and NG-RAN, CN is not required to involve when UE state switches to RRC_CONNECTED mode. However, option2 and 3 require CN to cooperate with the gNB to retrieve UE context, which involving CN to participate in RRC_INACTIVE resume process and increases signalling overhead. In option 1, during paging the UE to the subsequent satellite, UE have to wake up when every satellite passing through, which is not conducive to UE power consumption. Pro-actively UE context transfer hop by hop in option1 and relocate the UE context to the gNB on ground in option2 have minimal impacts on existing procedures, but raises security concerns regarding UE context interactions between multiple satellites. Such optimizations may diverge from the original objectives of introducing RRC_INACTIVE mode. Therefore, we think the further enhancements for supporting RRC_INACTIVE mode is not clear and there seems to be limited benefits for regenerative payload NR NTN.

Proposal 4: Any enhancements for RRC_INACTIVE in regenerative NR NTN in R19 should be thoroughly analyzed for their benefits and costs.

3	Conclusion

In this contribution, we provide considerations on the NTN regenerative payload in RAN3, following observations and proposals are made:

Proposal 1: The timetable configured by OAM with a TAI list with granularity in seconds, and updated hourly or daily basis, can serve as a reference in RAN3. The specific configuration can be determined by SA5.

Proposal 2: In addition to the legacy mechanism, the OAM-based solution should also be supported, and which applied subject to operator’s policy.

Observation 1: Geo-fixed RNA configuration can reduce RNA update signaling overhead with satellite moving.

Proposal 3: RNA could be configured as a fixed geographical area (i.e. A list of Tracking Area Code) in regenerative payload NR NTN.

Proposal 4: Any enhancements for RRC_INACTIVE in regenerative NR NTN in R19 should be thoroughly analyzed for their benefits and costs.

4	Reference

- RAN3_126_agenda_20241122_1244-EOM

- S2-2413030, LS on reply to LS on OAM requirements to support regenerative payload

- S5-247311, LS reply on OAM requirements to support regenerative payload