3GPP TSG-RAN WG3 Meeting #127	R3-250651

Athens, Greece, 17 - 21 Feburary, 2025

Agenda item:	19.2

Source: 	Huawei, China Telecom, China Unicom

Title: 	Further discussions on SBFD related information exchange

Document for:	Discussion

1. Introduction

In the RAN3#125bis meeting, there were agreements about SBFD related information exchange, listed as follow:

RAN3 agrees to Exchange the following information over Xn interface and F1 interface: - Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP) - Measurement resource configuration - Strongest DL beam information (FFS on the details) - CLI-mitigation request (FFS on the details)

And in the RAN3#126 meeting, there were agreement about SBFD time and frequency location configuration:

The SBFD time and frequency location configuration with cell granularity in Served Cell Information NR IE shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

In this contribution, we will further discuss the RAN3 impacts, including SBFD configuration exchange, measurement procedure, and CLI mitigation procedure among gNBs.

2. Discussion

2.1	Exchange of SBFD configuration

In the previous meeting, RAN1 has achieved the following agreements about SBFD time and frequency configuration:

- SBFD subband time-domain location indication Agreement A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot SBFD subband frequency-domain location indication Agreement The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid. - RB-level granularity is supported for semi-static indication of SBFD subband frequency location. - Subject to RAN4 guidance on the size of subband/guardband, if any - FFS reference starting RB and reference SCS Agreement For cell-specific configuration of frequency locations of SBFD subbands, - Option 1: Cell-specific frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList. - For each SCS configuration, the reference starting PRB is the PRB determined by the SCS configuration and offsetToCarrier corresponding to this subcarrier spacing.

Currently, the TDD UL-DL Configuration Common NR IE and the Carrier List IE are included in Served Cell Information NR IE to indicate the time and frequency configuration of the cell.

| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the SIB1 message as defined in TS 38.331 [10] | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >>>Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |

From above, the TDD UL-DL Configuration Common NR IE is directly referred to the field tdd-UL-DL-ConfigurationCommon in TS 38.331, and the Carrier List IE is explicitly defined with relevant values. For the SBFD time configuration, RAN2 has not decided the signalling to indicate the time location of SBFD symbols, if the SBFD time configuration is extended in tdd-UL-DL-ConfigurationCommon IE in TS 38.331, the current XnAP specification can support the exchange of SBFD time configuration naturally; otherwise, new filed needs to be introduced to Served Cell Information NR IE. For frequency configuration, new fields need to be introduced to Served Cell Information NR IE. Thus, how to enable the exchange of SBFD time and frequency configuration among gNBs depends on RAN2 work.

One more thing is, since RAN2 is in CC in the LS from RAN1, maybe there is a need for RAN3 to remind RAN2 to work on the IE design of SBFD time and frequency configuration, and from RAN3 perspective, RAN1 should also provide the detailed parameters for the SBFD timing and frequency info.

Proposal 1: The detailed sub-IE design for the SBFD time and frequency configuration is pending to RAN2 and RAN3 to send an LS asking RAN1 and RAN2 to work on the detailed parameters and IE design.

Besides, the F1AP specification should also support to provide the SBFD time and frequency configuration of the served cell and obtain the SBFD time and frequency configuration of other cells. In the current F1AP specification, the TDD UL-DL Configuration Common NR IE and Carrier List IE are provided from gNB-DU to gNB-CU. Similarly, the SBFD time and frequency location configuration should also be provided from gNB-DU to gNB-CU in Served Cell Information IE, RAN3 can simply use the way of enhancement for Xn interface, the messages to be enhanced should be straight forward, i.e F1 setup and gNB-DU Configuration Update.

Proposal 2: It is proposed RAN3 to agree that, in case of split architecture, the gNB-DU provide the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE of F1AP specification.

Similarly, to make gNB-DU know the SBFD configuration of the cell of neighbour gNBs, the F1AP should also be enhanced to provide the time and frequency configuration of other cells from gNB-CU to gNB-DU, the message to be enhanced is also straight forward, since Neighbour Cell Information List is only carried via gNB-DU Configuration Update message.

Proposal 2bis: It is proposed RAN3 to agree to enhance the F1AP specification to support gNB-CU can provide the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in gNB-DU Configuration Update message.

2.2	Exchange of measurement configuration

In the LS from RAN1, RAN3 is request to support the exchange of following information:

- Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

Based on this, the measurement resource configuration is required to be exchanged. Currently, the SSB resource configuration has been already exchanged in MeasTiming field of inter-node RRC message Measurement Timing Configuration, which is used by gNB to generate the measurement gaps based on the received SMTC information. While the NZP CSI-RS resource configuration hasn’t be provided in inter-node RRC message Measurement Timing Configuration.

While for NZP CSI-RS resource configuration, it was defined in NZP-CSI-RS-ResourceSet in Measurement Timing Configuration by RAN2, but RAN2 has not included it in the inter-node message. RAN3 could either refer to NZP-CSI-RS-ResourceSet IE directly, or ask RAN2 to include this IE in the inter-node message.

Observation 1: The SSB resource configuration has been already exchanged in MeasTiming field of inter-node RRC message Measurement Timing Configuration, as defined by RAN2; while for NZP CSI-RS resource configuration, RAN2 defined this IE but not included in the inter-node RRC message.

On the other hand, whatever the solution option is to be chosen by RAN1 or RAN2, we need to further consider the concrete IE design of XnAP and F1AP. As we could see that from the IE “Served Cell Information NR”, in which there is sub-IE “Measurement Timing Configuration” which further includes the MeasurementTimingConfiguration inter-node message for the served cell, which has included the SSB resource configuration. If RAN2 agree to introduce the NZP-CSI-RS-ResourceSet in Measurement Timing Configuration, the current XnAP and F1AP specification can support the exchange of SBFD time configuration naturally; otherwise, new filed need to be introduced to Served Cell Information NR IE.

Proposal 3: RAN3 to confirm that SSB resource configuration has already been supported to be exchanged between gNBs in Served Cell Information NR IE.

Proposal 3bis: While for the NZP-CSI-RS resource configuration, it should be provided in Served Cell Information NR IE and, RAN3 to discuss the detailed sub-IE design, either asking RAN2 to work on the detailed parameters and IE design, or just refer directly to the IE NZP-CSI-RS-ResourceSet defined by RAN2.

In addition, there is another open point, i.e. whether there is need to trigger the neighbour gNB to perform CLI measurement on which cell and on exactly which signal(s) (SSB or NZP-CSI-RS), in order to be simple, we think it could be left to gNB implementation.

2.3	Exchange of measurement result and CLI-mitigation request

In the LS from RAN1, RAN3 is request to support the exchange of following information:

- Information exchange of strongest DL beam information

- Information exchange of CLI-mitigation request

Firstly, RAN3 should discuss whether the strongest DL beam information and CLI-mitigation request can be sent together. Technically, we think the above two fields can be provided together, since when the neighbour gNB gets the strongest DL beam information, it could also evaluate whether there is any CLI from other gNB or not, and decide to request CLI mitigation, thus the two parameters could be included together, but of course it is up to gNB to decide whether to include the two parameters together or not, if there is no interference, there is no need, i.e. the two parameters could be included in the same message as optional IEs.

Proposal 4: RAN3 agree to exchange the strongest DL beam information and the CLI-mitigation request as two optional IEs in the same message.

On the issue that whether to introduce the new procedure/message or enhance the existing procedure to support strongest DL beam information and CLI-mitigation request exchange. For this issue, there are three options may be used:

- Option 1: To reuse interface management procedures: NG-RAN node Configuration Update procedure

- Option 2: To reuse Resource Status Reporting procedure

- Option 3: To introduce new procedures

Among these options, we think option1 and option3 is suitable to exchange the above information. We slightly support Option3 which is clear to provide the strongest DL beam information and the CLI-mitigation request.

Proposal 4bis: It is proposed RAN3 discuss which options to support the exchange of strongest DL beam information and CLI-mitigation request, with the following options:

- Option 1: To reuse interface management procedures: NG-RAN node Configuration Update procedure

- Option 2: To reuse Resource Status Reporting procedure

- Option 3: To introduce new procedures

For the detailed IEs to indicate the strongest DL beam information for SSB and NZP CSI-RS resource, we think it can follow the description in LS from RAN1:

Agreement Send an LS to RAN3 with the following information (cc: RAN2) From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported. Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

From RAN3 point of view, the strongest beam of NZP CSI-RS Resource can be expressed to INTEGER (1..64), whose number is the order of CSI-RS resources within a set of resources for which the configuration is provided to a gNB, and the strongest beam of SSB can be expressed to INTEGER (0..63) as existing field SSB index.

Proposal 5: RAN3 agree that the strongest beam of NZP CSI-RS Resource is provided using a NZP-CSI-RS-ResourceId, i.e.  INTEGER (1..64); and the strongest beam of SSB is provided using a SSB-Index, i.e.  INTEGER (0..63).

RAN3 can further discuss the F1 impacts to provide CLI measurement result based on the progress of Xn interface, i.e. in the victim gNB side, the gNB-DU should inform its gNB-CU about the strongest beam info and possible CLI request, while in the aggressor gNB side, the gNB-CU should forward the received info to its gNB-DU.

Proposal 5bis: RAN3 to discuss the corresponding F1 impacts, i.e. in the victim gNB side, the gNB-DU should inform its gNB-CU about the strongest beam info and possible CLI mitigation request, while in the aggressor gNB side, the gNB-CU should forward the received info to its gNB-DU.

Here stage 2 TPs and stage3 TPs to TS 38.423 could be referred in [3], and stage 2&3 TPs to TS 38.473 in [4].

3. Conclusion

In this contribution, we analyzed the specification impacts of gNB-gNB mitigation. The following observation and proposals were provided:

Exchange of SBFD configuration

Proposal 1: The detailed sub-IE design for the SBFD time and frequency configuration is pending to RAN2 and RAN3 to send an LS asking RAN1 and RAN2 to work on the detailed parameters and IE design.

Proposal 2: It is proposed RAN3 to agree that, in case of split architecture, the gNB-DU provide the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE of F1AP specification.

Proposal 2bis: It is proposed RAN3 to agree to enhance the F1AP specification to support gNB-CU can provide the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in gNB-DU Configuration Update message.

Exchange of measurement configuration

Observation 1: The SSB resource configuration has been already exchanged in MeasTiming field of inter-node RRC message Measurement Timing Configuration, as defined by RAN2; while for NZP CSI-RS resource configuration, RAN2 defined this IE but not included in the inter-node RRC message.

Proposal 3: RAN3 to confirm that SSB resource configuration has already been supported to be exchanged between gNBs in Served Cell Information NR IE.

Proposal 3bis: While for the NZP-CSI-RS resource configuration, it should be provided in Served Cell Information NR IE and, RAN3 to discuss the detailed sub-IE design, either asking RAN2 to work on the detailed parameters and IE design, or just refer directly to the IE NZP-CSI-RS-ResourceSet defined by RAN2.

Exchange of memeasurement result and CLI-mitigation request

Proposal 4: RAN3 agree to exchange the strongest DL beam information and the CLI-mitigation request as two optional IEs in the same message.

Proposal 4bis: It is proposed RAN3 discuss which opionts to support the exchange of strongest DL beam information and CLI-mitigation request, with the following options:

- Option 1: To reuse interface management procedures: NG-RAN node Configuration Update procedure

- Option 2: To reuse Resource Status Reporting procedure

- Option 3: To introduce new procedures

Proposal 5: RAN3 agree that the strongest beam of NZP CSI-RS Resource is provided using an NZP-CSI-RS-ResourceId, i.e. INTEGER (1..64); and the strongest beam of SSB is provided using a SSB-Index, i.e.  INTEGER (0..63).

Proposal 5bis: RAN3 to discuss the coo corresponding F1 impacts, i.e. in the victim gNB side, the gNB-DU should inform its gNB-CU about the strongest beam info and possible CLI mitigation request, while in the aggressor gNB side, the gNB-CU should forward the received info to its gNB-DU.

4. Reference

[1]. RP-241614, WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2]. R1-2407533, LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation

[3]. R3-250652	Stage 2&3 CR on the introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[4]. R3-250653	Stage 2&3 CR on the introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

Annex C: Draft LS to RAN2 on SBFD information exchange among gNBs for CLI mitigation

Title:	LS to RAN2 on SBFD information exchange among gNBs for CLI mitigation

Release:	Release 19

Work Item:	NR_duplex_evo

Source:	TSG RAN WG3

To:	TSG RAN WG2, TSG RAN WG1

Cc:

Contact person:	Yang Xudong

yangxudong@huawei.com

# 1	Overall description

RAN3 has started to work on the SBFD information exchange among gNBs for CLI mitigation, according to the received LS in R1-2407533 from RAN1.

RAN3 has agreed that SBFD time and frequency configuration info should be exchanged between neighbour gNBs in Served Cell Information NR IE, RAN3 also reached the common understanding that RAN3 needs RAN2 to provide the concrete IE design on SBFD time and frequency configuration info, which should be based on parameters provided by RAN1.

RAN3 has also agreed that measurement resource configuration should be exchanged between neighbour gNBs in Served Cell Information NR IE, RAN3 also reached the common understanding that measurement resource configuration should be provided in inter-node RRC message Measurement Timing Configuration, while such message doesn’t include NZP-CSI-RS-ResourceSet.

With the agreement and common understanding reached in RAN3, RAN3 would like to ask RAN1 to work on the detailed parameters, and ask RAN2 to work on the IE design of SBFD time and frequency configuration info.

# 2	Actions

To TSG RAN1

ACTION: 	RAN3 respectfully asks RAN1 to provide the detailed parameters for SBFD time and frequency configuration info.

To TSG RAN2

ACTION: 	RAN3 respectfully asks RAN2 to work on the IE design of SBFD time and frequency configuration info.

# 3	Dates of next TSG RAN WG3 meetings

RAN3#127-bis	2025-04-07 - 2025-04-11		China, CN

RAN3#128	2025-05-19 - 2025-05-23	Malta, MT