3GPP TSG-RAN WG3 Meeting #127	R3-250659

Athens, Greece, 17 – 21 February, 2025

Agenda item:	8.1

Source: 	Huawei

Title: 	(TP for TS 38.415) XR available data rate report

Document for:	Approval

# 1. Introduction

This contribution is to show the potential change needed for TS 38.415 to support the XR available data rate report, according to the discussion in R3-250294.

# Annex - TP for TS 38.415

Start of Change

#### 5.5.2.2	UL PDU SESSION INFORMATION (PDU Type 1)

This frame format is defined to allow the UPF to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective UL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | QMP | DL Delay Ind. | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag | QoS Flow Identifier |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) | New IE Flag 6 | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  |  |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| UL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| UL Available Data Rate Information |  |  |  |  |  |  |  | 0 or 1 |
| DL Available Data Rate Information |  |  |  |  |  |  |  | 0 or 1 |
| Padding |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: UL PDU SESSION INFORMATION (PDU Type 1) Format

The New IE Flag in bit 6 of 2nd octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the first octet of New IE Flags Octet is present or not.

Bit 0 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the D1 UL PDCP Delay Result Ind is present (1) or not (0).

Bit 1 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Congestion Information is present (1) or not (0).

Bit 2 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Congestion Information is present (1) or not (0).

Bit 3 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Available Data Rate Information is present (1) or not (0).

Bit 4 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Available Data Rate Information is present (1) or not (0).

Next Change

#### 5.5.3.x	UL Available Data Rate Information

Description: This parameter indicates the identifier of the available data rate report thresholds defined in TS 38.413 [7]. When it is present, it means the UL available data rate is larger than or equal to the threshold corresponding to the identifier, and smaller than the next threshold if any.

Value range: {0..FFS}.

Field length: 1 octet.

#### 5.5.3.y	DL Available Data Rate Information

Description: This parameter indicates the identifier of the available data rate report thresholds defined in TS 38.413 [7]. When it is present, it means the DL available data rate is larger than or equal to the threshold corresponding to the identifier, and smaller than the next threshold if any.

Value range: {0..FFS}.

Field length: 1 octet.

End of Change