3GPP TSG-RAN WG3 Meeting #127	R3-250666

Athens, GR, Feb 17th – 21th, 2025

Agenda Item:	10.3.1

Source:	CMCC, Huawei

Title:	Discussion on SON/MDT enhancement for NTN

Document for:	Discussion

# 1 Introduction

In RAN3#126 meeting, there was discussion on SON/MDT enhancement for Intra-NTN mobility. Some agreements were achieved and some issues were captured [1] and need to be continued in next meeting:

RAN3#126:

Existing timers can be reused to identify whether RLF happens before or after T1 with 100ms accuracy, FFS if the accuracy is a problem to be solved.

WA: There is no need to report any other info in the RLF report for the location based CHO.

RAN3 assumes both geographical area and the mapped cell ID as Area Scope of logged MDT for NTN over NGAP are feasible and confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for MDT NTN.

In this contribution, we would discuss on the issues of MRO for failure scenario for intra-NTN and our initial consideration on MRO for near failure scenario.

# 2 Discussion

2.1 MRO for failure scenarios

Based on the following agreements achieved in RAN3#125 meeting [2], we can see that there are two cases (Too Late CHO Execution, Too Early CHO Execution/CHO Execution to Wrong Cell) in MRO for time-based CHO.

MRO for CHO with a time-based trigger condition:

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

- Distinguish if the RLF happens before or after T1

Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

Case 1 (Too Late CHO Execution) can be further divided into RLF happens before and after T1 threshold starts. Network could consider to move T1 threshold to an earlier time, so that UEs could start the CHO execution earlier for RLF happens before T1 threshold in Case 1. For RLF happens after T1 threshold in Case 1, it may lead to RLF happens before CHO execution that the value of Duration is set too large. Network could consider to decrease the value of the Duration in this sub-case. For the case of Too Early CHO Execution in Case 2, CHO executesbefore HOF happens or RLF happens. Network could consider to move T1 threshold to a later time to avoid UEs start the CHO execution early. For the case of CHO Execution to Wrong Cell, UE successfully executes CHO before RLF happens and the UE re-connects in a third cell. Thus, the candidate cell of CHO needs to be optimized rather than the configuration of T1 threshold and Duration.

Proposal 1: The objective of MRO for time-based CHO is to optimize configuration issues of T1 threshold, Duration or CHO candidate cell.

For location based trigger condition:

WA: There is no need to report any other info in the RLF report for the location based CHO.

There are two cases (Too late CHO Execution and Too Early CHO Execution/CHO Execution to Wrong Cell) in the scenario where a location-based trigger condition configured. To analyze potential issues, we believe it is valuable to gather information about the distance between the UE and the moving reference location in both the serving cell and the candidate cell at the time of failure. This information will help to optimize whether the configured distance thresholds may contribute to the failure.

Observation 1: For location based trigger condition, the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively when the failure occurs is helpful to analysis whether the distance thresholds configured in location-based trigger condition have the potential problem.

Proposal 2: The objective of MRO for location-based CHO is to optimize configuration issues of the distance thresholds in trigger condition.

Regarding to get the information about the distance between UE and the moving referenceLocation in serving cell and in candidate cell, we made a working assumption for location based CHO in last RAN3 meeting that there is no need to report any other info in the RLF report for the location based CHO. The common understanding is network could get to know the UE location based on the existing IE locationInfo in RLF report and derive the distance between UE and the referenceLocation in serving cell and in candidate cell respectively. However, we observe that the locationInfo IE in RLF report is optional, that means UE location information may be not available when the failure occurrs and network could not derive the distance between UE and the referenceLocation.

Observation 2: Since the IE locationInfo in RLF report is optional, UE location information may be not available when RLF occurred and network could not derive the distance between UE and the referenceLocation in serving cell and in candidate cell.

Moreover, the time of RLF occurred needs to be known by the network to derive the distance between UE and the referenceLocation in serving cell and in candidate cell based on UE location information. Network could deduce the time of RLF occurred after receiving the relevant time (e.g., timeSinceCHO-Reconfig) in RLF report.

timeSinceCHO-Reconfig This field is used to indicate the time elapsed between the initiation of the last conditional reconfiguration execution towards the target cell and the reception of the latest conditional reconfiguration for this target cell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

In NTN, the duration from network sending CHO reconfiguration to UE receiving CHO reconfiguration should be considered when identifying the time of RLF. It is noted that the propagation delay from network to UE cannot be ignored in NTN. If a UE only reports the timeSinceCHO-Reconfig IE in the RLF report, the network cannot accurately infer the time of RLF occurred based solely on the reported time. Therefore, the network is unable to derive the accurate distance between UE and the referenceLocation in serving cell and in candidate cell at the time of RLF occurs based solely on the timeSinceCHO-Reconfig IE in the RLF report. Obviously, the work assumption we made in the last meeting is not correct, we urge all companies to recheck, and some necessary information in the RLF report should be reported for the location based CHO.

Observation 3: In NTN, the duration time from network sending CHO reconfiguration to UE receiving CHO reconfiguration should be considered when identifying the time of RLF.

Observation 4: The network is unable to derive the accurate distance between UE and the referenceLocation in serving cell and in candidate cell at the time of RLF occurs based solely on the timeSinceCHO-Reconfig IE in the RLF report.

Proposal 3: The work assumption we made in the last meeting is not correct, we urge all companies to recheck, and some necessary information in the RLF report should be reported for the location based CHO.

Hence, we think the straightforward solution to get the information about the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell for location-based CHO when RLF occurs is to include them in RLF report.

Proposal 4: The straightforward solution to get the information about the distance between UE and moving referenceLocation in serving cell and in candidate cell for location-based CHO when RLF occurs is to include them in RLF report.

2.2 MRO for near failure scenarios

Based on the agreements achieved in RAN3#123bis meeting [3], the work on MRO for near failure scenarios could start after failure scenarios. Considering the progress of normative work on MRO for failure scenarios, we propose to start the work on optimization for near failure scenarios.

MRO mechanisms for intra-NTN should be addressed based on failure scenarios first, work on near failure scenarios later.

For near failure scenarios, Successful Handover Report (SHR) could be enhanced to support successful Conditional handovers in NTN to detect a sub-optimal successful CHO event. Considering the time-based and location-based trigger condition for CHO are supported in NTN, a new time-based SHR trigger and a location-based SHR trigger configured by network could be considered accordingly to trigger the SHR report when UE execute CHO successfully to a candidate cell with high risk of failure. Specifically,

- A new time-based SHR trigger for NTN could be the threshold(s) of time (e.g., in terms of percentage values) used to trigger the SHR report. For example, UE collects SHR if the ratio between the value of elapsed time since t1(t1 is configured by NW in CondEventT1) until UE execute CHO and the value of Duration (Duration is configured by NW in CondEventT1) is greater than the threshold, and makes the SHR available to the network.

- A new location-based SHR trigger for NTN could be the threshold(s) of distance used to trigger the SHR report. For example, in the case that the threshold of distance is in terms of percentage values, UE collects SHR if the ratio between the value of distance between UE and the referenceLocation/moving referenceLocation in serving cell when UE execute CHO, and the threshold configured by NW is greater than the location-based SHR trigger, and makes the SHR available to the network.

Proposal 5: Successful Handover Report (SHR) could be enhanced to support MRO for near failure scenarios in NTN.

Proposal 6: The introduction of a new time-based SHR trigger and a location-based SHR trigger could be considered for optimization of near failure scenarios in NTN.

Based on initial analysis, we think that the following information maybe helpful for optimization of CHO trigger conditions in NTN and could be logged by UE in the enhanced SHR.

- The time point that the first CHO trigger condition is satisfied both for one or multiple CHO trigger conditions are configured by network; this information helps to identify whether too late CHO may occur and optimize T1 threshold or Duration in the time-based trigger condition or distance thresholds/timeToTrigger in location-based trigger condition or the Thresholds in CHO events A3/A4/A5.

- The first CHO trigger condition and the time elapsed between the two triggered conditions, if multiple CHO trigger conditions are configured by network; this information helps to identify whether the second CHO trigger condition satisfied late that may cause too late CHO and optimize Duration in the time-based trigger condition or distance thresholds/timeToTrigger in location-based trigger condition or the Thresholds in CHO events A3/A4/A5.

- The distance between UE and the moving referenceLocation in serving cell and the distance between UE and the moving referenceLocation in candidate cell when timetotrigger configured in CHO trigger condition are fulfilled; the information could be used for location-based trigger condition to identify whether too late CHO or too early CHO may occur and optimize distance thresholds or timeToTrigger in location-based trigger condition, for the earth moving case.

- The cause value that triggers the SHR; this information helps to know which configurations of CHO trigger condition have potential problems.

Proposal 7: RAN3 to consider SHR enhancement to support optimization of near failure scenarios in NTN, e.g.,

- The time point that the first CHO trigger condition is satisfied both for one or multiple CHO trigger conditions are configured by network;

- The first CHO trigger condition and the time elapsed between the two triggered conditions, if multiple CHO trigger conditions are configured by network;

- The distance between UE and the moving referenceLocation in serving cell and the distance between UE and the moving referenceLocation in candidate cell when timetotrigger configured in CHO trigger condition are fulfilled, for the earth moving case;

- The cause value that triggers the SHR, e.g., the threshold(s) of time in time-based SHR trigger and the threshold(s) of distance in location-based SHR trigger.

# 3 Conclusion

In this paper, we discuss on MRO mechanisms for intra-NTN mobility, we have the following observations and proposals:

MRO for failure scenarios

Proposal 1: The objective of MRO for time-based CHO is to optimize configuration issues of T1 threshold, Duration or CHO candidate cell.

Observation 1: For location based trigger condition, the distance between UE and the referenceLocation/moving referenceLocation in serving cell and in candidate cell respectively when the failure occurs is helpful to analysis whether the distance thresholds configured in location-based trigger condition have the potential problem.

Proposal 2: The objective of MRO for location-based CHO is to optimize configuration issues of the distance thresholds in trigger condition.

Observation 2: Since the IE locationInfo in RLF report is optional, UE location information may be not available when RLF occurred and network could not derive the distance between UE and the referenceLocation in serving cell and in candidate cell.

Observation 3: In NTN, the duration time from network sending CHO reconfiguration to UE receiving CHO reconfiguration should be considered when identifying the time of RLF.

Observation 4: The network is unable to derive the accurate distance between UE and the referenceLocation in serving cell and in candidate cell at the time of RLF occurs based solely on the timeSinceCHO-Reconfig IE in the RLF report.

Proposal 3: The work assumption we made in the last meeting is not correct, we urge all companies to recheck, and some necessary information in the RLF report should be reported for the location based CHO.

Proposal 4: The straightforward solution to get the information about the distance between UE and moving referenceLocation in serving cell and in candidate cell for location-based CHO when RLF occurs is to include them in RLF report.

MRO for near failure scenarios

Proposal 5: Successful Handover Report (SHR) could be enhanced to support MRO for near failure scenarios in NTN.

Proposal 6: The introduction of a new time-based SHR trigger and a location-based SHR trigger could be considered for optimization of near failure scenarios in NTN.

Proposal 7: RAN3 to consider SHR enhancement to support optimization of near failure scenarios in NTN, e.g.,

- The time point that the first CHO trigger condition is satisfied both for one or multiple CHO trigger conditions are configured by network;

- The first CHO trigger condition and the time elapsed between the two triggered conditions, if multiple CHO trigger conditions are configured by network;

- The distance between UE and the moving referenceLocation in serving cell and the distance between UE and the moving referenceLocation in candidate cell when timetotrigger configured in CHO trigger condition are fulfilled, for the earth moving case;

- The cause value that triggers the SHR, e.g., the threshold(s) of time in time-based SHR trigger and the threshold(s) of distance in location-based SHR trigger.

# Reference

- RAN3_126_agenda_20241122_EOM1, Nov 2024.

- RAN3_125_agenda_20240823EOM, Aug 2024.

- RAN3_123bis_agenda_20240419_EOM, Apr 2024.