3GPP TSG-RAN WG3 #127	              R3-250667

Athens, Greece, 17th – 21th Feb 2025

Agenda Item:	18.2

Source:	CMCC

Title:	Discussion on supporting LP-WUS Indicating Paging Monitoring

Document for:	Discussion

- Introduction

Rel-19 NR LP-WUS/WUR WI was approved in [1], which states that in RRC IDLE/INACTIVE modes, it's observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI). We will discuss on the following objectives in this contribution.

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

In RAN2#128, RAN2 had the following agreement on subgrouping:

- FFS on the following options

- Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

- Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed.

- Option 3: The formula for UE_ID based PEI subgrouping is reused.

In RAN3#125bis, We agreed the following agreements:

- The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups.

- The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups.

- The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups.

- Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping.

- The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

In RAN3#126, We had the following agreements:

- WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

- It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring for RRC_IDLE and RRC_INACTIVE.

- NG-RAN node can acquire the total number of CN assigned subgrouping via OAM.

- WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

The last used cell and extra bits issues still need to be discussed.

- Discussion

2.1 LP-WUS capability indication in F1

For the CN assigned subgrouping, once the CU includes the CN assigned subgroup ID in the LP-WUS assistance information IE within the F1 PAGING message, it implicitly informs the DU that the paged UE supports CN assigned subgrouping LP-WUS. The DU can then proceed with the subsequent actions based on the received CN assigned subgroup ID. Therefore, there is no need for an additional per-UE indication of the CN assigned subgrouping capability in F1AP.

Next, we analyze UE_ID based subgrouping. According to the RAN3 126 meeting, an LP-WUS subgrouping support indication per-cell was introduced, which is similar to the PEI Subgrouping Support Indication IE included in the Paging Cell Item IEs IE. When the F1 PAGING message does not contain a CN assigned subgroup ID, but the Paging Cell Item IEs IE includes an LP-WUS Subgrouping Support Indication set to TRUE, the DU can infer that the UE supports UE_ID based subgrouping. Consequently, the DU can wake up the UE in the corresponding cell using the UE_ID based subgroup ID.

Thus, with the introduction of the per-cell LP-WUS Subgrouping Support Indication IE included in the Paging Cell Item IEs IE, the DU can be informed whether the paged UE supports LP-WUS capability, which can be applied to both UE_ID based subgrouping and CN based subgrouping. Specifically, there are three cases for the F1 PAGING message:

- Case 1: CN assigned subgroup ID is present, along with an LP-WUS Subgrouping Support Indication set to TRUE.

- Case 2: Only the LP-WUS Subgrouping Support Indication is present and set to TRUE.

- Case 3: Neither CN assigned subgroup ID nor LP-WUS Subgrouping Support Indication is present.

For Case 1 and Case 2, the DU can explicitly determine whether to wake up the UE using UE_ID based LP-WUS or CN based LP-WUS. For Case 3, the DU can also clearly infer that the UE does not support LP-WUS capability and should wake up the UE using PEI or legacy paging procedures.

In summary, for both UE_ID based subgrouping and CN based subgrouping, no additional per-UE capability indication is required. At the same time, supporting to introduce per-cell LP-WUS Subgrouping Support Indication IE included in the Paging Cell Item IEs IE.

Proposal 1: For both UE_ID based subgrouping and CN based subgrouping, no additional per-UE capability indication is required.

Proposal 2: For both UE_ID based subgrouping and CN based subgrouping, introducing LP-WUS Subgrouping Support Indication IE included in the Paging Cell Item IEs IE.

2.2 Last used cell issue

In RAN3#125bis we discussed whether the anchor gNB provides a new IE for CN assigned subgroup ID into the Xn paging message for RAN paging, and decided to postpone this agreement until RAN2 reaches a conclusion about that UE can use the LP-WUS outside the last serving cell.

But as far as we know, RAN2 has not discussed this issue. And referring to PEI, the lastUsedCellOnly flag in SIB1 is configurable, and there is no mandatory constraint that PEI can only be used for the last used cell.

In case LP-WUS is used in any cell, it is beneficial for the UE because LP-WUS can always be used. If the network finds that there are too many LP-WUS transmissions when the TA/RNA consists of a large number of cells, it can configure LP-WUS to be used only for the last used cell. In summary, this should be a configurable feature, and the new IE for CN assigned subgroup ID should be introduced into the Xn paging message for RAN paging.

Proposal 3: RAN3 agree the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

2.3 UE_ID based subgrouping

RAN1 made a working assumption that the number of LP-WUS subgroups is 32, which is different from 8 in PEI.

According to the agreement in RAN2#128 meeting:

- FFS on the following options

- Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

- Option 2: The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed.

- Option 3: The formula for UE_ID based PEI subgrouping is reused.

According to TS 38.473, the Extended UE Identity Index is 16 bits. The following diagram illustrates the UE_ID required for paging, PEI, and LP-WUS:

Fig 1 the UE_ID required for paging, PEI, and LP-WUS

As shown in the diagram, even with option 3, when the UE uses eDRX and the number of LP-WUS subgroups exceeds 16, the existing 16-bit Extended UE_ID is insufficient. Therefore, a new IE should be introduced into F1 paging to include extra bits for UE_ID based LP-WUS subgrouping, but the exact number of additional bits will depend on the conclusion from RAN2.

The diagram of 5G-S-TMSI is as follows, where TMSI is usually generated randomly.

Fig 2 diagram of 5G-S-TMSI

Proposal 4: A new IE should be introduced into F1 paging to include extra bits for UE_ID based LP-WUS subgrouping, but the exact number of additional bits will depend on the conclusion from RAN2.

2.4 Reply LS

In SA2#166 Meeting, SA2 send a reply LS to RAN2/3 to ask the following issue:

SA2 S2-2412876 2) Initial Registration may be performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS) that analyses the UE’s UE Radio Access Capabilities to create the UE Radio Capability for Paging Information and send it to the AMF, see clause 5.4.4.3 of TS23.501. It will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS. SA2 invites RAN2 and RAN3 to investigate this and provides solution if any issue is identified. SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

We think there are two solutions:

- Option 1: OAM ensures that all gNBs within a specific area are configured to support the LP-WUS feature.

- Option 2: When the UE switches to a gNB that supports the LP-WUS feature, it reports the UE Radio Capability again. The specific process details are defined by RAN2.

Proposal 5: There are two methods to resolve the loss of UE LP-WUS capability information:

- Option 1: OAM ensures that all gNBs within a specific area are configured to support the LP-WUS feature.

- Option 2: When the UE switches to a gNB that supports the LP-WUS feature, it reports the UE Radio Capability again. The specific process details are defined by RAN2.

- Conclusions

In this contribution, we discussed supporting LP-WUS indicating paging monitoring in RRC_IDLE/INACTIVE mode, and following are the proposals.

Proposals:

Proposal 1: For both UE_ID based subgrouping and CN based subgrouping, no additional per-UE capability indication is required.

Proposal 2: For both UE_ID based subgrouping and CN based subgrouping, introducing LP-WUS Subgrouping Support Indication IE included in the Paging Cell Item IEs IE.

Proposal 3: RAN3 agree the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

Proposal 4: A new IE should be introduced into F1 paging to include extra bits for UE_ID based LP-WUS subgrouping, but the exact number of additional bits will depend on the conclusion from RAN2.

Proposal 5: There are two methods to resolve the loss of UE LP-WUS capability information:

- Option 1: OAM ensures that all gNBs within a specific area are configured to support the LP-WUS feature.

- Option 2: When the UE switches to a gNB that supports the LP-WUS feature, it reports the UE Radio Capability again. The specific process details are defined by RAN2.

- References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)