3GPP TSG-RAN WG3 Meeting #127	R3-250682

# Athens, Greece,  17– 21 Feb, 2025

Title: 	Discussion on UE performance metric for both non-split and split architecture

Source: 	CMCC

Agenda item: 	11.4

Document Type:	Discussion

- Introduction

In RAN3 #124 meeting, agreements as following have been achieved for split architecture:

Transferring Measured UE performance from CU-UP to CU-CP.

And in RAN3 #125 meeting,

The details of signaling measured UE performance metrics from DU to CU-CP and/or from CU-UP To CU-CP needs further discussion during normative phase. The above overrides the agreement “Transferring Measured UE performance from CU-UP to CU-CP.” from RAN3 #124.

And in RP-242385, the WI description is agreed that:

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743.

The detailed objectives of the WI are listed as follows:

- Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: Split architecture support for Rel-18 use cases

Note: Coordination with RAN2, SA5 when needed if any.

In RAN3 #126 meeting, the UE performance metric including Average throughput DL/UL, Average Packet Loss DL, Average Packet Delay DL/UL are discussed and LS has been sent to SA5 on clarification of how to calculate the Average throughput DL/UL.

.2	Discussion

This paper will focus on the discussion on transferring data between gNB-CU and gNB-DU for AI/ML Load Balance and Mobility Enhancement use cases. Since the topic is hardly discussed in previous R19 meetings, we propose to discuss the following with priority in this meeting.

In RAN3#125bis, RAN3 received LS R3-245021 from SA5, and SA5 provides the following information:

SA5 would like to response to the questions with the following:

RAN3 evaluated carefully the two specifications and made the following observations with respect to TS 28.552 and TS 28.558:

- TS 28.558 does not define M4 measurement.

- There seems to be a misalignment between the two specifications with respect to which entity performs the measurements. In particular, RAN3 identified the following cases:

- The “Average delay DL air-interface” is measured by the gNB-DU in TS 28.552 and by NRCellCU (for non-split and 2-split scenario) and GNBCUUPFunction (for 3-split scenario) in TS 28.558.

- The “Average delay DL in gNB-DU” is measured by the gNB-DU in TS 28.552 and by NRCellCU (for non-split and 2-split scenario) and GNBCUUPFunction (for 3-split scenario) in TS 28.558.

- The “UL PDCP SDU Loss Rate” is measured by the GNBCUUPFunction and NRCellCU in TS 28.552 and by GNBCUUPFunction in TS 28.558.

From RAN3 perspective, the above misalignment should be corrected in TS 28.558 based on TS 28.552.

From SA5 perspective, the misalignment between TS28.558 and TS 28.552 has been corrected in TS28.558v18.1.0. The missing M4 measurement has be added into TS28.558 as attached.

Observcation1: It is clear from this LS reply that Average delay DL needs to involve measurement collected from gNB-DU.

  - Average Throughput UL/DL

In RAN3 #126 meeting, it has been agreed that:

for Average UE Throughput DL/UL: these metrics could be calculated as specified in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2.

6.3.1.4.1	Average DL UE throughput in gNB

c)	This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC architecture [16]) and for each supported S-NSSAI.

If ThpTimeDl>0, ThpVolDlThpTimeDl×1000 [kbit/s]

If ThpTimeDl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission,, otherwise

6.3.1.4.2	Average UL UE throughput in gNB

c)	This measurement is obtained according to the following formula based on the "ThpVolUl" and "ThpTimeUl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC architecture [16]) and for each supported S-NSSAI.

If ThpTimeUl>0, ThpVolUlThpTimeUl×1000 [kbit/s]

If ThpTimeUl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission otherwise

And LS has been sent to SA5 to clarify it:

For the Average UE Throughput DL and Average UE Throughput UL, RAN3 would like to ask SA5 whether the formulas in fields c) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558, respectively, refer to measurements that are specified with a per-UE granularity or with a per-DRB granularity as the measurement name in field e) of the same mentioned clauses starts with “DRB.”. RAN3 would appreciate an explanation of why the measurement name starts with the “DRB.” prefix.

Proposal 1: The Average UE Throughput DL and UL are measured through the per-DRB level throughput for the UE, and averaged via the data volume/time for all these DRB.

According to TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2., the Average UE Throughput DL and UL are calculated through the Data Volume in RLC level:

|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |
| --- | --- |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer). |

For split architecture, it is obvious that the Average UE Throughput DL and UL are measured in gNB-DU.

Proposal 2: For split architecture, the average Throughput UL/DL is calculated on RLC level in gNB-DU. It is proposed to transfer the Average Throughput UL/DL from gNB-DU to gNB-CU through F1 interface.

2.2 Average Packet Loss DL/UL

According to TS28.554 clause 6.3.1.3, for non-split architecture, the DL packet loss is measured as the DL packet loss rate on Uu interface (i.e., RLC SDU). For slicing scenario, it may include the loss that can’t meet the delay threshold packets.

For split architecture, the DL packet loss also include the DL F1-U Packet Loss Rate, while the UL packet loss include the UL PDCP SDU Loss Rate and UL F1-U Packet Loss Rate.

Proposal 3: For split architecture, the Average Packet Loss DL shall consider both DL packet loss rate on Uu interface and DL F1-U Packet Loss Rate.

Currently, the Average Packet Loss UL is not taken as UE performance feedback for non-split architecture. However, for the split- architecture, the information on the UL PDCP SDU Loss Rate and F1-U Packet Loss Rate are still useful for the AI/ML as UE performance feedback.

Proposal 4: It is proposed to add Average Packet Loss UL as an optional UE Performance feedback. For split architecture, it shall consider both UL PDCP SDU Loos Rate and F1-U Packet Loss Rate.

In AI/ML for NGRAN, the UE level performance feedback and slice level performance feedback shall be collected. How to use the per-DRB level measurement to get the average UE level or slice level measurement is needed to be discussed in RAN3.  From TS 28.558 clauses 6.3.1.2, you can see for the DL Packet Loss Rate：

6.3.1.2.1	DL Packet Loss Rate on Uu

a)	This measurement provides the DL Packet (i.e., RLC SDU) Loss rate on Uu interface. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and per supported S-NSSAI. This measurement is also referred to as DL M7 in TS 37.320 [9].

b)	CC

c)	This measurement is obtained according to the definition in clause 4.2.1.5.1 of TS 38.314 [8], named "Packet Uu Loss Rate in the DL per DRB per UE". The measurement is performed per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and per supported S-NSSAI.

d)	Each measurement is an integer value. The number of measurements is equal to the number of QoS levels multiplied by the number of supported S-NSSAIs.

e)	DRB.PacketLossRateUuDlUe.Filter, 
Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellDU

g)	5G-S-TMSI

h)	One usage of this measurement is to support ML training and performance evaluation.

And in in clause 4.2.1.5.1 of TS 38.314:

Protocol Layer: RLC

Table 4.2.1.5.1-1: Definition for Packet Uu Loss Rate in the DL per DRB per UE

| Definition | Uu Packet Loss Rate in the DL per DRB per UE. One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed Definition: M(T,drbid)=Dloss(T,drbid)*1000000N(T,drbid)+Dloss(T,drbid), where explanations can be found in the table 4.2.1.5.1-2 below. |
| --- | --- |

Table 4.2.1.5.1-2: Parameter description for Packet Uu Loss Rate in the DL per DRB per UE

| M(T,drbid) | Packet Loss Rate in the DL per DRB per UE. Unit: number of lost packets per transmitted packets per DRB * 106, Integer. |
| --- | --- |
| Dloss(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, for which at least a part has been transmitted over the air but not positively acknowledged, and it was decided during time period T that no more transmission attempts will be done. If transmission of a packet might continue in another cell, it shall not be included in this count. |
| N(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, which has been transmitted over the air and positively acknowledged during time period T. |
| T | Time Period during which the measurement is performed, Unit: minutes. |
| drbid | The identity of the measured DRB. |

For different DRBs, the traffic is different, which means N(T, drbid) are different. To get an average DL Packet Loss Rate on Uu interface, it is reasonable to average all the Dloss through Packets for all related DRBs:

M(T)=∑Dloss(T,drbid)*1000000∑(NT,drbid+DlossT,drbid)
If there is no traffic under a DRB within the measurement duration time, N and Dloss for this DRB will be 0 and it won’t impact the average UE or Slice-level Packet Loss Rate.

6.3.1.3.3	      DL F1-U Packet Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at the gNB-DU). It is a measure of the DL packet loss on the F1-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]), and subcounters per supported S-NSSAI. This measurement is also referred to as DL M7 in TS 37.320 [9].

b)	SI

c)	This measurement is obtained as: 1,000,000* Number of missing DL GTP sequence numbers (TS 29.281 [15]), representing packets that are not delivered to lower layers, of a data radio bearer, divided by Total number of UL GTP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by gNB-CU-UP to gNB-DU until the GTP sequence number of the last packet. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC architecture [16]) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.F1UpacketLossRateDlUe, and optionally DRB.F1UPacketLossRateDlUe.QoS where QoS identifies the target quality of service class, and DRB.F1UPacketLossRateDlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	NRCellDU

g)	5G-S-TMSI

h)	One usage of this measurement is to support ML training and performance evaluation.

In the similar way, the UE or Slice-level DL F1-U Packet Loss Rate shall also be averaged among all its DRBs in the same way, using all the Dloss to be divided by all the Packets: 1,000,000* Number of missing DL GTP sequence numbers (TS 29.281 [15]), representing packets that are not delivered to lower layers, of a data radio bearer, for all DRBs, divided by Total number of UL GTP sequence numbers (also including missing sequence numbers) of a bearer for all DRBs.

F1UPacketLossRateDlUe(T,drbid)=∑Dloss(T,drbid)*1000000∑(NT,drbid+DlossT,drbid), here Dloss is the missing DL GTP sequence numbers, N is the Total number of DL GTP sequence numbers.

Proposal 5: It is proposed that the UE or Slice-level Average DL Packet Loss Rate on Uu shall be the average among all related DRBs, using the total Dloss to be divided by total packets in these DRBs.

Proposal6: It is proposed that the UE or Slice-level Average DL F1-U Packet Loss Rate shall be the average among all related DRBs, using the total Dloss to be divided by total packets in these DRBs.

We can see that the UE or Slice-level Average DL Packet Loss Rate on Uu are measured on gNB-DU. Also the UE or Slice-level Average F1-U Packet Loss are also measured on gNB-DU. gNB-DU needs to send the information to gNB-CU through F1 interface, and CU sum the Packet Loss Rate on Uu and F1-U up, to get the final UE or Slice-level Average DL Packet Loss Rate, and then transfer it through Xn as the UE performance feedback metric.

Since the above loss rates are mainly measured in gNB-DU, it is obvious the information shall be transferred to CU-CP to get the total Packet Loss.

Proposal 7: It is beneficial to transfer the UE or Slice-level Average DL Packet Loss Rate on Uu and UE or Slice-level Average F1-U Packet Loss from gNB-DU to gNB-CU in split architecture. gNB-CU shall add both up as the UE or Slice-level Average DL Packet Loss Rate.

For UL Packet Loss Rate:

6.3.1.3.1	UL PDCP SDU Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface. Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]), and subcounters per supported S-NSSAI. This measurement is also referred to as UL M7 in TS 37.320 [9].

b)	SI

c)	This measurement is obtained as: 1,000,000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC architecture [16]) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurements are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.PacketLossRateUlUe, and optionally
DRB.PacketLossRateUlUe.QoS where QoS identifies the target quality of service class, and DRB.PacketLossRateUlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction; NRCellCU (applicable to all split scenarios);

NRCellCU (only applicable to 2-split scenario).g)	S-TMSI

h)	One usage of this measurement is to support ML training and performance evaluation.

6.3.1.3.2	UL F1-U Packet Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss on the F1-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as UL M7 in TS 37.320 [9].

b)	SI

c)	This measurement is obtained as: 1,000,000* Number of missing UL GTP sequence numbers (TS 29.281 [15]), representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL GTP sequence numbers (also including missing sequence numbers) of a bearer, starting from the GTP sequence number of the first packet delivered by gNB-DU to gNB-CU-UP until the GTP sequence number of the last packet. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC architecture [16]) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.F1UpacketLossRateUlUe, and optionally DRB.F1UPacketLossRateUlUe.QoS where QoS identifies the target quality of service class, and DRB.F1UPacketLossRateUlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction

g)	5G-S-TMSI

h)	One usage of this measurement is to support ML training and performance evaluation.

As the DL Packet Loss Rate, for different DRBs, the traffic is different, which means the number of PDCP SDUs are different. To get an average UL SDU Loss Rate, it is reasonable to average all the Dloss through Packets for all related DRBs under this UE or Slice:

PacketLossRateUlUe(T,drbid)=∑Dloss(T,drbid)*1000000∑(NT,drbid+DlossT,drbid)

In the similar way, the UE or Slice-level UL F1-U Packet Loss Rate shall also be averaged among all its DRBs in the same way, using all the Dloss to be divided by all the Packets: 1,000,000* Number of missing UL GTP sequence numbers (TS 29.281 [15]), representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL GTP sequence numbers (also including missing sequence numbers) of all DRBs under the UE or Slice.

Proposal 8: It is proposed that the UE or Slice-level Average UL PDCP Packet Loss Rate, UL F1-U Packet Loss Rate shall be the average among all related DRBs uder the UE or Slice, using the total Dloss to be divided by total packets in these DRBs in split architecture.

Since the UL PDCP SDU Loss Rate and UL F1-U Packet Loss Rate are measured on CU-UP, CU-UP shall send the information to CU-CP through E1 interface, and add them up as the UE or Slice-level Average UL Packet Loss Rate, transfer it through Xn interface as the UE performance feedback.

Proposal 9: It is beneficial to send the UE or Slice-level UL PDCP SDU Loss Rate and UL F1-U Packet Loss Rate from CU-UP to CU-CP through E1 interface, and CU-CP shall add them up as the UE or Slice-level UL Loss Rate in split architecture.

  - Average Packet Delay UL/DL

In split architecture, the delay can be measured in different layers which exist in different unit.

According to TS 28.558 clause 6.3, the UE Average Packet Loss DL includes the Average delay DL air-interface, Average delay DL in gNB-DU (RLC SDU delay DL), Average delay DL on F1-U (GTP packet delay DL on F1-U) and Average delay DL in CU-UP (PDCP SDU delay DL). All these can be measured in DRB granularity.

The delay DL air-interface, RLC SDU delay, GTP packet delay DL on F1-U are all measured in gNB-DU, while PDCP SDU delay DL is measured in CU-UP.

So in split architecture, the UE Average Packet Loss DL needs to be transferred from gNB-DU to gNB-CU, and CU-UP to CU-CP.CU-CP shall sum up all the delays in different levels for each DRB.

Proposal 10: It is proposed that in split architecture, the end to end UE average packet loss DL shall include the the delay measured in both gNB-DU and gNB-CU. CU-CP shall sum up these delays in different layers as the per-DRB’s end to end Packet Delay DL.

Also according to TS 28.558 clause 6.3, the UE Average Packet Loss UL includes UL PDCP packet average delay, Average delay UL on over-the-air interface, Average RLC packet delay in the UL and Average PDCP re-ordering delay in the UL.

The delay UL on over-the-air interface and RLC packet delay in UL are measured in gNB-DU, while the UL PDCP packet average delay, PDCP re-ordering delay UL is measured in CU-UP. So both meansurement in gNB-DU and CU-UP need to be transferred to CU-CP, and CU-CP shall sum up all theses delays in different layers as the end to end delay for each DRB.

Proposal 11: It is proposed that in split architecture, the end to end UE average packet loss UL shall include the the delay measured in both gNB-DU and gNB-CU. CU-CP shall sum up these delays in different layers as the per-DRB’s end to end Average Packet Delay UL.

Meanwhile, for the slice level UE performance, the average packet delay UL/DL can be measured in per-DRB level.  All above average delay can be calculated together from end to end in per-DRB level.

If the UE traffic or slice traffic are distributed in several DRBs, how to average the packet loss delay DL/UL among these different DRB level measurements? In RAN3, it was never clearly discussed.

The packet delay is not the same as throughput or packet loss, it is not meaningful to add up the delay for different DRB level delay. The delay shall be the time span from first sending packet to last receiving packet. So it is reasonable to use the worst DRB’s delay under a UE or Slice to present the UE or slice’s packet delay.

Proposal 12: It is proposed to use the worst DRB’s Average Packet Delay UL/DL under the UE to represent the UE’s Average Packet Delay UL/DL as the UE performance feedback. For slice-level delay, use the worst DRB’s Average Packet Delay UL/DL under the slice to represent the slice’s Average Packet Delay UL/DL.

3	Summary

In this contribution, we provide additional input considerations on the remaining issues on AI/ML for NG-RAN split architecture use cases, following observations and proposals are made:

Observcation1: It is clear from this LS reply that Average delay DL needs to involve measurement collected from gNB-DU.

Proposal 1: The Average UE Throughput DL and UL are measured through the per-DRB level throughput for the UE, and averaged via the data volume/time for all these DRB.

Proposal 2: For split architecture, the average Throughput UL/DL is calculated on RLC level in gNB-DU. It is proposed to transfer the Average Throughput UL/DL from gNB-DU to gNB-CU through F1 interface.

Proposal 3: For split architecture, the Average Packet Loss DL shall consider both DL packet loss rate on Uu interface and DL F1-U Packet Loss Rate.

Proposal 4: It is proposed to add Average Packet Loss UL as an optional UE Performance feedback. For split architecture, it shall consider both UL PDCP SDU Loos Rate and F1-U Packet Loss Rate.

Proposal 5: It is proposed that the UE or Slice-level Average DL Packet Loss Rate on Uu shall be the average among all related DRBs, using the total Dloss to be divided by total packets in these DRBs.

Proposal6: It is proposed that the UE or Slice-level Average DL F1-U Packet Loss Rate shall be the average among all related DRBs, using the total Dloss to be divided by total packets in these DRBs.

Proposal 7: It is beneficial to transfer the UE or Slice-level Average DL Packet Loss Rate on Uu and UE or Slice-level Average F1-U Packet Loss from gNB-DU to gNB-CU in split architecture. gNB-CU shall add both up as the UE or Slice-level Average DL Packet Loss Rate.

Proposal 8: It is proposed that the UE or Slice-level Average UL PDCP Packet Loss Rate, UL F1-U Packet Loss Rate shall be the average among all related DRBs uder the UE or Slice, using the total Dloss to be divided by total packets in these DRBs in split architecture.

Proposal 9: It is beneficial to send the UE or Slice-level UL PDCP SDU Loss Rate and UL F1-U Packet Loss Rate from CU-UP to CU-CP through E1 interface, and CU-CP shall add them up as the UE or Slice-level UL Loss Rate in split architecture.

Proposal 10: It is proposed that in split architecture, the end to end UE average packet loss DL shall include the the delay measured in both gNB-DU and gNB-CU. CU-CP shall sum up these delays in different layers as the per-DRB’s end to end Packet Delay DL.

Proposal 11: It is proposed that in split architecture, the end to end UE average packet loss UL shall include the the delay measured in both gNB-DU and gNB-CU. CU-CP shall sum up these delays in different layers as the per-DRB’s end to end Average Packet Delay UL.

Proposal 12: It is proposed to use the worst DRB’s Average Packet Delay UL/DL under the UE to represent the UE’s Average Packet Delay UL/DL as the UE performance feedback. For slice-level delay, use the worst DRB’s Average Packet Delay UL/DL under the slice to represent the slice’s Average Packet Delay UL/DL.

4. Reference

- 3GPP RAN3#125bis Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM)

- RP-242385 WID Enhancements for Artificial Intelligence (AI) Machine Learning (ML) for NG-RAN (ZTE, NEC), 3GPP RAN#105

- DRAFT R3-244848 TR38.743 of AIML for NG-RAN_v1 (Release 19)