3GPP TSG-RAN WG3 Meeting #127	             R3-250685

Athens, GR, Feb 17th- Feb 21th, 2025

Agenda Item:	16.2

Source:	CMCC

Title:	(TP to TS 38.300) Discussion on RAN Architecture for Ambient IoT

Document for:	Other

1	Introduction

In RAN#106 meeting, a WID on Solutions for Ambient IoT (Internet of Things) in NR has been approved. During the WI phase, only Topology 1 will be standardized and the following objectives in the WID may have impact on RAN3 are identified:

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. - RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we will provide our preliminary views on RAN architecture aspects for Ambient IoT.

2	Discussion

2.1	A-IoT Architecture

First of all, the study on Architecture for Ambient IoT is carried out simultaneously in RAN3 and SA2. Before specifying the necessary architectural aspects to support Topology 1 in RAN side, we can take the SA2 conclusion with the principles and aspects for Topology 1 into account. The following contents are excerpted from TR 23.700-13-110:

8.1.2 Architecture to Support Topology 1 8.1.2.1 General The principles and aspects in this clause are agreed to support Topology 1: - The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1: - Send requests that trigger BS Reader(s) (via AIoT RAN) to perform AIoT operations, either directly or via an AMF. NOTE 2: NGAP is used when the AIOTF directly communicates with the AIOT RAN (i.e. over the Nx reference point) or indirect communication with AIOT RAN via an AMF (i.e. over the N2 reference point). NOTE 4: It is not expected a deployment will use both direct communication between AIOT RAN and an AIOTF and indirect communication between AIOT RAN and an AIOTF via an AMF. The deployment choice can be based on, for example, using a direct communication path for a local deployments. - The signalling transport for NGAP at the A-IoT RAN node is SCTP/IP.

It is noted that there are two deployments supported by SA2, i.e., direct deployment and indirect deployment, and a deployment simultaneously using direct communication and indirect communication is not existing. The difference between two deployments is AIOTF supporting Ambient IoT functionality communicates with A-IoT RAN node either directly or via an AMF. For both direct and indirect deployments, the basic principle in A-IoT is to design a unified solution to support necessary architectural, and signalling and procedures between A-IoT RAN and A-IoT CN.

Proposal 1: For both direct and indirect deployments, the basic principle in A-IoT is to design a unified solution to support necessary architectural, and signalling and procedures between A-IoT RAN and A-IoT CN.

From the TR 38.769, the XX interface symbolizes the interface between A-IoT CN and A-IoT RAN on which certain A-IoT specific functions are performed, and we concluded that XX interface in Topology 1 is NG-C interface. In SA2 TR, it also mentions the NGAP is used when the AIOTF directly communicates with the A-IOT RAN or indirect communication with A-IOT RAN via an AMF. Currently, the definition of XXAP is the interface used to perform the A-IoT specific operation by signalling A-IoT procedures. However, it is still undecided that the XXAP is realised by including AIOTF information in NGAP or by carrying a new protocol layer over NGAP.

Observation 1: It is still undecided that the XXAP is realised by including AIOTF information in NGAP or by carrying a new protocol layer over NGAP.

For ease of observation, the protocol stacks including both direct path and indirect path deployment for two options are illustrated as follows:

Option1: Including AIOTF information in NGAP Option2: Carrying a new protocol layer over NGAP

Regard to the Option 1, the AIOTF information is included in the NGAP, and the A-IoT messages between the A-IoT RAN node and AIOTF can be supported by NGAP. It is not required to introduce a new application protocol for Option1, while some new NGAP signalling procedures will be introduced and some existing NGAP signalling procedure will be enhanced to support A-IoT functions. In both deployments, the AIOT information is visible to AIOTF carried in NGAP messages, but the AIOT information is encoded in a AIOTF container which is transparently to the AMF in indirect path deployment.

Observation 2: If including AIOTF information in NGAP, it is not required to introduce a new application protocol, while some new NGAP signalling procedures will be introduced and some existing NGAP signalling procedure will be enhanced to support A-IoT functions.

Regarding to the Option 2, the AIOTF information is carried in a new protocol layer over NGAP, temporarily called XXAP. Similar as NRPPa protocol layer, a new A-IoT protocol layer should be specified to support the A-IoT signalling procedures between a NG-RAN node and the AIOTF. In both deployments, the AIOT information is visible to AIOTF, but it is encoded in NAS PDU which is transparently to the AMF in indirect path deployment. Meanwhile, the NGAP also should be enhanced to carry the new uplink or downlink A-IoT layer message over the NGAP, e.g., introducing the A-IoT layer transport messages to carry AIOTF-PDU which contains the A-IoT message that is transferred without interpretation in the AMF.

Observation 3: If carrying a new protocol layer over NGAP, a new A-IoT protocol layer should be specified to support the A-IoT signalling procedures between a NG-RAN node and the AIOTF. The NGAP also should be enhanced to carry the new uplink or downlink A-IoT layer message over the NGAP.

In sum, Option 2 introduces a new protocol, and Option 1 does not. In addition, NGAP will be enhanced to support the AIOTF information transfer in both options. Since both options have changes to the NGAP spec, we prefer to take an approach that modifies as few protocols as possible, so we would like to take Option 1 where only NGAP will be enhanced without introducing new spec.

Proposal 2: It is proposed that XXAP is realised by including AIOTF information in NGAP.

2.2	A-IoT Reader Selection

As the conclusion recorded in TR 38.769, the reader selection may need coordination between A-IoT RAN node and A-IoT CN. In the SI phase, companies provide two options on reader selection. One option is the A-IoT RAN node selects the readers based on information from the AIOTF, the information could be a list of readers or the A-IoT service area information. The other option is AIOTF selects the reader(s) and indicates the selected reader(s) to the A-IoT RAN node. Finally, the description of reader selection written in the TR includes the two options above.

Observation 4: The description of reader selection written in the TR includes the option on both AIOTF dominated selection and A-IoT RAN node dominated selection.

During WI phase, RAN3 needs to continue working on how to specify the reader selection mechanism. Regarding the reader selection, RAN3 can also make reference to SA2 conclusion which is shown as follows:

### 8.1.1 General The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work: 1. A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality. … b. For topology 1, the AIOTF selects AIOT RAN nodes and optionally a list of the BS readers. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN. NOTE 1: Providing the UE Reader list to the RAN only applies to the RRC-based option. For topology 1, NGAP is terminated at an AIOT RAN node. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g. BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g. their corresponding service area when the BS Readers are integrated in the network. NOTE 2: Whether RAN3 will specify BS reader information (e.g. BS reader ID, service area) reporting to AIOTF over NGAP depends on RAN3 decision and if so SA WG2 may include this feature in SA WG2 specifications. NOTE 3: If the request from the AIOTF to the AIOT RAN does not include a BS reader list, then AIOT RAN can use all available BS readers. NOTE 4: The BS reader’s service area needs to coordinate with RAN WG(s).

It can be observed that the AIOTF selects A-IoT RAN nodes and optionally a list of the BS readers, which means both AIOTF dominated selection and A-IoT RAN node dominated selection are supported by SA2. From RAN3 point of view, both selection mechanism needs to be supported in WI phase. If the reader list is not provided by AIOTF, the selected A-IoT RAN node can choose all available readers within its scope. If the reader list is provided, the A-IoT RAN node should firstly check the reader selection information provided by the AIOTF to confirm whether the reader is available. If the reader is available, then the A-IoT RAN node should use the reader to perform A-IoT service. If the reader is not available, the A-IoT RAN node can use another reader to perform A-IoT service. If none of the reader in the A-IoT RAN node is available, the A-IoT RAN node should reject the request.

Proposal 3: Both AIOTF dominated selection and A-IoT RAN node dominated selection are supported in WI phase.

If the A-IoT RAN node dominates the reader selection, it means the AIOTF may not have awareness of actual location of the reader, and the A-IoT RAN node selects reader based on the reader’s service area information. From the SA2 TR, SA2 does not give a clear explanation on this type of information and ask RAN3’s definition. From our interpretation, the reader’s service area is uniformly designed for the network to enable the reader to perform A-IoT service effectively, which can be compared to the concept of NR cell. Each reader will only serve one service area for ease of management. The AIOTF should be preconfigured with the mapping relation between reader’s service area and the geographical area by OAM. Once receiving the external inventory area from the AF, the AIOTF can transfer the external geographical area to service area. Besides, the AIOTF also be informed the reader’s service areas within one A-IoT RAN node which helps it select the suitable A-IoT RAN node to perform A-IoT service. In order to known by AIOTF, the supported reader’s service areas within one A-IoT RAN node may be configured by the OAM or optionally reported by the A-IoT RAN node through the NG Setup procedure or RAN Configuration Update procedure.

Observation 5: The reader’s service area is uniformly designed for the network to enable the reader to perform A-IoT service effectively.

Proposal 4: The AIOTF should be preconfigured with the mapping relation between reader’s service area and the geographical area by OAM.

Proposal 5: In order to known by AIOTF, the reader’s service areas within one A-IoT RAN node may be configured by the OAM or optionally reported by the A-IoT RAN node through the NG Setup procedure or RAN Configuration Update procedure.

If the AIOTF dominates the reader selection, the AIOTF needs to know supported reader ID and reader’s service area. Based on this assistance information, the AIOTF can choose the suitable reader to perform inventory service. In order to known by AIOTF, the supported reader ID and reader’s service area may be configured by OAM or optionally provided by the A-IoT RAN node through the NG Setup procedure or RAN Configuration Update procedure.

Proposal 6: In order to known by AIOTF, the supported reader ID and reader’s service area may be configured by OAM or optionally provided by the A-IoT RAN node through the NG Setup procedure or RAN Configuration Update procedure.

No matter which node dominates the reader selection, in order to avoid complex radio resource management between A-IoT RAN nodes which RAN3 does not study in SI. The deployment needs to be limited to readers controlled by single A-IoT RAN node within a certain service area, where the A-IoT RAN node can fully control and allocate resource for each reader. If multiple readers are involved in an A-IoT service, it is necessary to find out a solution to avoid simultaneous A-IoT service operation towards A-IoT devices initiated by multiple readers. The simple way for the AIOTF/A-IoT RAN node is to adopt a TDM (Time Division Multiplexing) approach to request the readers to initiate A-IoT paging.

Observation 6: If multiple readers are involved in an A-IoT service, it is necessary to find out a solution to avoid simultaneous A-IoT service operation towards A-IoT devices initiated by multiple readers.

Proposal 7: The deployment needs to be limited to readers controlled by single A-IoT RAN node within a certain service area, where the A-IoT RAN node can fully control and allocate resource for each reader.

Proposal 8: The AIOTF/A-IoT RAN node can adopt a TDM approach to request the readers to initiate A-IoT paging.

3	Conclusion

Based on the discussions mentioned above, in this contribution we provide following observations and proposals on RAN architecture aspects for Ambient IoT:

Observation 1: It is still undecided that the XXAP is realised by including AIOTF information in NGAP or by carrying a new protocol layer over NGAP.

Observation 2: If including AIOTF information in NGAP, it is not required to introduce a new application protocol, while some new NGAP signalling procedures will be introduced and some existing NGAP signalling procedure will be enhanced to support A-IoT functions.

Observation 3: If carrying a new protocol layer over NGAP, a new A-IoT protocol layer should be specified to support the A-IoT signalling procedures between a NG-RAN node and the AIOTF. The NGAP also should be enhanced to carry the new uplink or downlink A-IoT layer message over the NGAP.

Observation 4: The description of reader selection written in the TR includes the option on both AIOTF dominated selection and A-IoT RAN node dominated selection.

Observation 5: The reader’s service area is uniformly designed for the network to enable the reader to perform A-IoT service effectively.

Observation 6: If multiple readers are involved in an A-IoT service, it is necessary to find out a solution to avoid simultaneous A-IoT service operation towards A-IoT devices initiated by multiple readers.

Proposal 1: For both direct and indirect deployments, the basic principle in A-IoT is to design a unified solution to support necessary architectural, and signalling and procedures between A-IoT RAN and A-IoT CN.

Proposal 2: It is proposed that XXAP is realised by including AIOTF information in NGAP.

Proposal 3: Both AIOTF dominated selection and A-IoT RAN node dominated selection are supported in WI phase.

Proposal 4: The AIOTF should be preconfigured with the mapping relation between reader’s service area and the geographical area by OAM.

Proposal 5: In order to known by AIOTF, the reader’s service areas within one A-IoT RAN node may be configured by the OAM or optionally reported by the A-IoT RAN node through the NG Setup procedure or RAN Configuration Update procedure.

Proposal 6: In order to known by AIOTF, the supported reader ID and reader’s service area may be configured by OAM or optionally provided by the A-IoT RAN node through the NG Setup procedure or RAN Configuration Update procedure.

Proposal 7: The deployment needs to be limited to readers controlled by single A-IoT RAN node within a certain service area, where the A-IoT RAN node can fully control and allocate resource for each reader.

Proposal 8: The AIOTF/A-IoT RAN node can adopt a TDM approach to request the readers to initiate A-IoT paging.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things;

[3] 3GPP TR 38.769 V19.0.0, Study on solutions for Ambient IoT (Internet of Things) in NR;

5	Text Proposal to TS 38.300 BL CR

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A2X	Aircraft-to-Everything

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

A-IoT	Ambient IoT

AIOTF	Ambient IoT Function

A-IoT RAN	Ambient IoT Radio Access Network

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

AR	Augmented Reality

ARP	Allocation and Retention Priority

/******* Unchanged part skipped *******/

IAB	Integrated Access and Backhaul

IFRI	Intra Frequency Reselection Indication

IoT	Internet of Things

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

L2	Layer-2

L3	Layer-3

LBT	Listen Before Talk

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.X	Ambient IoT

## 16.XX	RAN Architecture

In this clause, the general principles and requirements related to the Ambient IoT RAN architecture are given. The Ambient IoT network architecture is supported with two deployments:

-	the direct path deployment where the AIOT RAN node communicates with the AIOTF directly, or;

-	the indirect path deployment where he AIOT RAN node communicates with the AIOTF indirectly via AMF.

In case of the AIOT RAN node and the AIOTF communicate directly, the A-IoT RAN node communicates with the AIOTF by means of NGAP, the AIOTF information is carried via NG interface, the NG protocol stack is illustrated in Figure 16.x-1 as below:

Figure 16.x-1 A-IoT NG protocol stack (direct path)

In case of the AIOT RAN node and communicates with the AIOTF indirectly via AMF, the A-IoT RAN node communicates with the AMF by means of NGAP, the AIOTF information is transparently to the AMF carried via NG interface, the NG protocol stack is illustrated in Figure 16.x-2 as below:

Figure 16.x-2 A-IoT NG protocol stack (indirect path)

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>