3GPP TSG-RAN WG3 Meeting #127	             R3-250688

Greece, GR, 17th – 21th Feb 2025

Agenda Item:	16.3

Source:	CMCC

Title:	Discussion on A-IoT device location reporting

Document for:	Discussion

1	Introduction

In RAN#106 meeting, a WID on Solutions for Ambient IoT (Internet of Things) in NR has been approved. During the WI phase, only Topology 1 will be standardized and the following RAN3-led objectives have been identified within the general scope described in the WID:

- RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we will provide our preliminary views on device location reporting.

2	Discussion

During SI phase, we studied the case of locating ambient IoT devices and the following contents related to A-IoT device locating are captured in the TR 38.769.

### 6.9.1 General A-IoT device location information may be used for the following purposes: (a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the A-IoT service. Locating an Ambient IoT device at "reader ID granularity" is useful for both purposes and is to be supported. NOTE: "reader ID granularity" is likely not sufficient for purpose (b) and may be refined in the future. ### 6.9.2 Topology 1 For support of A-IoT device location, the A-IoT RAN node may report the A-IoT RAN node ID to the A-IoT CN in the Inventory Report or Command Response messages. The A-IoT RAN node may additionally report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded. NOTE 1: Whether multiple Reader ID(s) can be included by the A-IoT RAN node in the Inventory Report/Command Response message needs further discussion. NOTE 2: Definition and format of the A-IoT RAN node ID and Reader ID needs further discussion. NOTE 3: Whether and how to have awareness of Reader location in A-IoT CN needs further discussion.

From the WID, the scope of RAN3 restricts the granularity for device location reporting as reader ID level, so we will start with how to report reader ID to support device location reporting. As the location of the A-IoT device, the AIOTF should be aware of the reader ID received through the A-IoT procedure over NGAP. Regarding the procedure, it is unnecessary to introduce a locating procedure for A-IoT device to solely report device ID.

The inventory and command procedure/signalling can be reused to report the reader ID. The purpose of the inventory procedure is to find the A-IoT device and collect the A-IoT device identifier. Once A-IoT device is found, the reader ID which represents the location of device can be reported associated with the A-IoT device ID. In addition, the reader ID can be also included reported by the command procedure. As the command procedure is always triggered after the inventory procedure, if the reader is not available provided by the A-IoT RAN node due to the resource congestion, it may change a new reader to command A-IoT device. After the command procedure, the A-IoT RAN node will update the reader ID.

Proposal 1: The inventory and command procedure/signalling can be reused to report the reader ID.

For the inventory operation, the Inventory Report procedure can be used to report the reader ID which is included in INVENTORY REPORT message. For the command operation, the Command Request procedure can be used to report the reader ID which is included in COMMAND RESPONSE message.

Proposal 2: The reader ID is included in INVENTORY REPORT message and COMMAND RESPONSE message.

As the scope of inventory may be a large warehouse, the inventory operation may require multiple readers deployed in a large area to perform inventory operation for finding all devices. Thus, multiple reader IDs can be included in the INVENTORY REPORT message in the inventory operation.

Proposal 3: Multiple reader IDs can be included in the INVENTORY REPORT message in the inventory operation.

Concerning the command operation is only performed towards single A-IoT device, the AIOTF or A-IoT RAN node will only choose one reader to perform command operation, so one A-IoT device will only response once to the reader in a command transaction. Hence, only one reader ID is reported in the COMMAND RESPOSNE message when AIOTF performs command operation.

Proposal 4: Only one reader ID is reported in the COMMAND RESPOSNE message when AIOTF performs command operation.

For the definition of the A-IoT RAN node, the objective of RAN3 already give a clear description that the A-IoT RAN node is assumed as an architecture of aggregated gNB. As one AIoT RAN node may serve one or multiple readers, the reader can be considered as a new term needs to be defined in the existing specification.

Proposal 5: The A-IoT RAN node is assumed as an architecture of aggregated gNB, and the reader can be considered as a new term needs to be defined in the existing specification.

Regarding the awareness of reader location, in Topology 1, the reader within the A-IoT RAN node is considered as stable or not moving. Hence, the direct way for A-IoT CN obtaining the reader location is configured by OAM.

Proposal 6: The direct way for A-IoT CN obtaining the reader location is configured by OAM.

3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some observations and proposals on A-IoT device location reporting:

Proposal 1: The inventory and command procedure/signalling can be reused to report the reader ID.

Proposal 2: The reader ID is included in INVENTORY REPORT message and COMMAND RESPONSE message.

Proposal 3: Multiple reader IDs can be included in the INVENTORY REPORT message in the inventory operation.

Proposal 4: Only one reader ID is reported in the COMMAND RESPOSNE message when AIOTF performs command operation.

Proposal 5: The A-IoT RAN node is assumed as an architecture of aggregated gNB, and the reader can be considered as a new term needs to be defined in the existing specification.

Proposal 6: The direct way for A-IoT CN obtaining the reader location is configured by OAM.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things;

[3] 3GPP TR 38.769 V19.0.0, Study on solutions for Ambient IoT (Internet of Things) in NR;