3GPP TSG-RAN WG3 Meeting #127               	                                                 			R3-250691

Athens, GR, Feb 17th - Feb 21th, 2025

Agenda Item:	8.1

Source:	CMCC

Title:	Discussion on UAV information reporting

Document for:	Discussion

Introduction

During RAN3#125bis meeting, RAN3 sent an LS to SA2 and cc SA1, asking whether to convey flight information over NGAP for control and management of the R18 UAV UEs and legacy devices equipped in an aerial vehicle. The content of LS is shown as follows:

During RAN3#125bis meeting, RAN3 has discussed whether to convey flight information e.g. altitude and velocity, from RAN to CN for control and management of the R18 UAV UEs and legacy devices (i.e. a Rel-15/16/17 device and non-UAV Rel-18 device) equipped in an aerial vehicle, based on operator’s contribution. RAN3 would like to ask for the guidance from SA2 whether it is a requirement for NG-RAN node to provide flight information to CN for the following cases: 1. R18 UAV UEs; 2. Legacy devices (non-UAV device);

In this meeting, we receive the reply LS from SA2 and SA1 and their reply is excerpted below:

SA2 Reply: - On Case 2, SA2 has discussed and concluded that no such requirements nor solutions exist for non-UAV legacy devices. - On Case 1, SA2 does not have any requirement dealing with Rel-18 UAV UEs, but SA2 believes current SA2 work for Rel-19 UAS_Ph3 on an unrelated use case requires AMF to get flight information such as altitude and velocity. SA2 is specifying a new feature “Pre-flight Planning and In-flight Monitoring for UAVs” in clause 5.12, and USS changeover in clause 5.13 in TS 23.256. In case of in-flight monitoring for UAV, USS/UTM, may request for flight assistance information and that network notifies the USS if UAV deviates from the defined flight path including the UAV altitude and velocity. SA2 agrees that the UAV altitude and velocity can be received from NG-RAN, assuming to reuse the existing feature like eventH1/H2 reporting together with additional trigger from the core network as specified in Rel-19. Availability of such information is applicable to any UAV UE(s) that support the relevant procedures.

SA1 Reply: SA1 would like to reply that there are no explicit service requirements for this function for these devices in TS 22.125.

Now SA2 and SA1 already provided guidance to RAN3, and RAN3 should follow their advice and continue to analyze the possible way for aerial UE information reporting mechanism to meet the requirement of UAV regulation.

Discussion

In the RAN3 LS, we would like to check whether it is a requirement for NG-RAN node to provide flight information to CN for both R18 UAV UEs and legacy devices. The responses from SA1 and SA2 are quite consistent, and neither identify the requirement to control both types of devices. However, for R18 UAV UEs, SA2 believes one of current use case in UAV work item requires AMF to get flight information such as altitude and velocity.

In the case of “In-flight Monitoring for UAVs” defined by SA2, USS/UTM may request for flight assistance information and that network notifies the USS if UAV deviates from the defined flight path including the UAV altitude and velocity. In order to achieve the goal, the Nnef_UAVFlightAssistance_Notify service including the monitoring results sent from NEF to USS needs to include the UAV altitude and velocity information.

Observation 1: In the case of “In-flight Monitoring for UAVs”, the Nnef_UAVFlightAssistance_Notify service including the monitoring results sent from NEF to USS needs to include the UAV altitude and velocity information.

In addition, SA2 defined a new feature “Altitude reporting for aerial UEs” to assist a USS/UTM with aerial UE's flight planning and monitoring, the 5GS may enable configuration of aerial UEs (UAVs) to perform altitude reporting. The whole feature includes two procedures shown and described below, i.e., the procedure of instructing aerial UEs to perform altitude reporting and the procedure of reporting UE's altitude information.

### 5.16.2 Instructing aerial UEs to perform altitude reporting Procedure shown in Figure 5.16.2-1 specifies how the 5GS instructs the aerial UEs (UAVs) to perform altitude reporting, considering requests from a USS/UTM for assistance with UAV's flight. This procedure can be used in conjunction with the NEF-assisted pre-flight planning, in-flight monitoring and USS changeover as specified in clauses 5.12.2, 5.12.3 and 5.13, respectively. Figure 5.16.2-1: Procedure for instructing aerial UEs for altitude reporting - UAS NF/NEF assists the USS/UTM with pre-fligh planning, in-flight monitoring and/or USS changeover based on the received request, which is either a Nnef_RetrieveInfoUAVFlight_Get or a Nnef_UAVFlightAssistance_Create, as specified in clauses 5.12 or 5.13 … Option B: Node-level signalling. Editor's note: This requires RAN to provide further enhancement of the procedure under RAN responsibility. 3b. The UAS NF/NEF may send an Namf_EventExposure_Subscribe request to the AMF to get notified about events of UAV's deviation from the assigned trajectory (i.e. Event ID = "Assigned Trajectory") as specified in clause 5.2.2.3.1 of TS 23.502 [3]. Inside the subscription request, the UAS NF/NEF may include the derived altitude thresholds and indication about reporting periodicity for the UE's altitude reporting. Additionally, the UAS NF/NEF may also include an indication to perform UE's altitude reporting for all aerial UEs in a specific TA(s) or NG-RAN node(s). 4b. The AMF sends the corresponding N2 messages to all applicable NG-RAN node(s). 5b. The NG-RAN node(s) instructs the relevant UE(s) to perform altitude reporting as requested by the UAS NF/NEF (via AMF). NOTE 3: Aspects related to AMF informing NG-RAN regarding instructing UEs to perform altitude reporting are specified in TS 38.413 [24]. ### 5.16.3 Reporting UE's altitude information A procedure for reporting aerial UE's altitude information to the core network is shown in Figure 5.16.3-1. The reporting can be done by the aerial UE based on the received altitude reporting thresholds and indication about reporting periodicity as specified in clause 5.16.2. Figure 5.16.3-1: Procedure for reporting of UE's altitude information 0. An aerial UE receives request to perform its altitude information reporting as specified in clause 5.16.2. The request may include altitude reporting thresholds and, optionally, indication about reporting periodicity. 1. UE's flight altitude meets the previously provided altitude reporting conditions (i.e. altitude thresholds and, optionally, reporting periodicity). … Option B: Node-level signalling. 1a-2b. The UE may provide a report with the altitude information to the NG-RAN. The report may contain the altitude information and, for instance, a time stamp or a report ID indicating when the UE has measured the altitude information that is being reported. NOTE 2: Aspects related to a UE providing altitude information to an NG-RAN node are specified by RAN in TS 38.331 [23]. The NG-RAN node includes the received UE's altitude information in an N2 message to the AMF. 3b. The AMF provides the UAS NF/NEF the altitude information report inside a Namf_EventExposure_Notify request. 4. The UAS NF/NEF together with the USS determines the impact on UAV's flight path based on the received altitude information report; this aspect is outside the scope of the 3GPP.

Regarding Option B in procedure for instructing aerial UEs for altitude reporting, the AMF sends the corresponding N2 messages to all applicable NG-RAN node(s) will have NGAP impact. The intention for the N2 message is to instruct NG-RAN node to report the aerial UE information. As a comparison, concerning Option B in procedure for reporting of UE’s altitude, the NG-RAN node includes the received UE's altitude information in an N2 message to the AMF will have NGAP impact. The intention for the N2 message is to report the received aerial UE’s altitude information to the AMF.

Observation 2: In the case of “Instructing aerial UEs for altitude reporting”, there is NGAP impact on instructing NG-RAN node to report the aerial UE information.

Observation 3: In the case of “Altitude reporting for aerial UEs”, there is NGAP impact on reporting the received aerial UE’s altitude information to the AMF.

From SA2 specification, some features require the UAV altitude and velocity information which can be initially received from NG-RAN node. The reply LS also mentions that the existing feature like eventH1/H2 reporting together with additional trigger from the core network as specified in Rel-19 can be reused.

Regarding the altitude reporting, the eventH1/H2 configured in the ReportConfigNR includes the includeAltitudeUE in the TS 38.331. The procedural text specifies that if the includeAltitudeUE is set to true in the corresponding reportConfig for the measId, the altitudeUE will include the altitude of the UE. Once the aerial UE is configured with eventH1/H2, it will report the altitude in the MeasurementReport.

Regarding the velocity reporting, as specified in RRC spec, the CommonLocationInfo is introduced to transfer detailed location information available at the UE. Within the CommonLocationInfo, the locationCoordinate-r16 carries the velocityEstimate-r16 including the velocity information. The explanation of velocityEstimate-r16 can be found in TS 37.355 reflected as Velocity IE.

From the above analysis, the existing RRC spec supports the NG-RAN node to collect the altitude information and velocity information from the aerial UE.

Proposal 1: The NG-RAN node can collect the altitude information and velocity information from the aerial UE.

To ensure the integrity of the entire reporting procedure, the altitude and velocity information should be transferred from NG-RAN node to 5GC in a report message. As SA2 already designed the feature of reporting of UE's altitude information, it is assumed that this feature can also be enhanced to report UE’s velocity information.

From the above analysis, the NGAP procedure will be used to instruct to report the aerial UE’s information and report aerial UE’s information. In order to minimize RAN3 specification impact, the existing Location Reporting Procedures can be reused. More specifically, the Location Reporting Control procedure is used where the AMF instructs the NG-RAN node to report the altitude and velocity information for the aerial UE. While, the Location Report procedure is used where the NG-RAN node provides the aerial UE’s altitude and velocity information to the AMF. In order to reflect the above modifications, the corresponding NGAP CR is provided in [6].

Proposal 2: The existing Location Reporting Procedures can be reused to perform the altitude and velocity information reporting.

Proposal 3: The Location Reporting Control procedure is used where the AMF instructs the NG-RAN node to report the altitude and velocity information for the aerial UE.

Proposal 4: The Location Report procedure is used where the NG-RAN node provides the UE’s altitude and velocity information to the AMF.

Conclusions

In this contribution, we discuss the existing feature in SA2 which can meet the requirements for UAV regulation, and analyze the impact on NG interface. The following observations and proposals are listed below:

Observation 1: In the case of “In-flight Monitoring for UAVs”, the Nnef_UAVFlightAssistance_Notify service including the monitoring results sent from NEF to USS needs to include the UAV altitude and velocity information.

Observation 2: In the case of “Instructing aerial UEs for altitude reporting”, there is NGAP impact on instructing NG-RAN node to report the aerial UE information.

Observation 3: In the case of “Altitude reporting for aerial UEs”, there is NGAP impact on reporting the received aerial UE’s altitude information to the AMF.

Proposal 1: The NG-RAN node can collect the altitude information and velocity information from the aerial UE.

Proposal 2: The existing Location Reporting Procedures can be reused to perform the altitude and velocity information reporting.

Proposal 3: The Location Reporting Control procedure is used where the AMF instructs the NG-RAN node to report the altitude and velocity information for the aerial UE.

Proposal 4: The Location Report procedure is used where the NG-RAN node provides the UE’s altitude and velocity information to the AMF.

References

- R3-250022, Reply LS on UAV regulation;

- R3-250015, Reply LS on UAV regulation;

- 3GPP TS 23.256, Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking;

- 3GPP TS 38.331, NR; Radio Resource Control (RRC);

- 3GPP TS 38.413, NG-RAN; NG Application Protocol (NGAP);

- R3-25xxxx, Correction on UAV monitoring, CMCC