3GPP TSG-RAN WG3 Meeting #127	R3-250692

# Athens, GR, Feb 17th - Feb 21th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1254 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to support UAV information reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As SA2 defined a new case of in-flight monitoring for UAV in TS 23.256, USS/UTM may request for flight assistance information and network notifies the USS if UAV deviates from the defined flight path including the UAV alititude and velocity, which means the 5GC should be aware of the UAV alititude and velocity information. Currently, the NG-RAN node can collect the altitude information and velocity information from the aerial UEs. To ensure the integrity of the entire reporting procedure, the altitude and velocity information should be reported from the NG-RAN node to the AMF. Existing Location Reporting Procedures, i.e., Location Reporting Control procedure and Location Report procedure, can be reused. The Location Reporting Control procedure is used where the AMF instructs the NG-RAN node to report the altitude and velocity information for the aerial UE. The Location Report procedure is used where the NG-RAN node provides the UE’s altitude and velocity information to the AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add procedural text for Location Reporting Control procedure when Location Reporting Request Type IE is set to aerial UE information. - Add Altitude IE and Velocity IE in the User Location Information IE; - Add new event type “aerial UE information” in Location Reporting Request Type IE; Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, the impact can be considered isolated because the change affects the aerial UE information reporting over NGAP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NG-RAN node cannot perform the altitude and velocity information reporting from aerial UEs over NGAP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.1.2, 9.3.1.16, 9.3.1.65, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

## 8.12	Location Reporting Procedures

### 8.12.1	Location Reporting Control

#### 8.12.1.1	General

The purpose of the Location Reporting Control procedure is to allow the AMF to request the NG-RAN node to report the UE's current location, or the UE's last known location with time stamp, or the UE's presence in the area of interest while in CM-CONNECTED state as specified in TS 23.501 [9] and TS 23.502 [10]. The procedure uses UE-associated signalling.

#### 8.12.1.2	Successful Operation

Figure 8.12.1.2-1: Location reporting control

The AMF initiates the procedure by sending a LOCATION REPORTING CONTROL message to the NG-RAN node. On receipt of the LOCATION REPORTING CONTROL message the NG-RAN node shall perform the requested location reporting control action for the UE.

The Location Reporting Request Type IE indicates to the NG-RAN node whether:

-	to report directly;

-	to report upon change of serving cell;

-	to report UE presence in the area of interest;

-	to stop reporting at change of serving cell;

-	to stop reporting UE presence in the area of interest;

-	to cancel location reporting for the UE;

-	to report upon change of serving cell and to report UE presence in the area of interest.

-	to report aerial UE information.

If the Area Of Interest List IE is included in the Location Reporting Request Type IE in the LOCATION REPORTING CONTROL message, the NG-RAN node shall store this information and use it to track the UE's presence in the area of interest as defined in TS 23.502 [10].

NOTE: 	The NG-RAN reports the UE presence for all set of Location Reporting Reference IDs for inter-NG-RAN node handover.

If the aerial UE information IE is included in the Location Reporting Request Type IE in the LOCATION REPORTING CONTROL message, the NG-RAN node may configure aerial UE for altitude and velocity reporting.

If the Additional Location Information IE is included in the LOCATION REPORTING CONTROL message and set to "Include PSCell” then, if Dual Connectivity is activated, the NG-RAN node shall include the current PSCell in the report. If a report upon change of serving cell is requested, the NG-RAN node shall provide the report also whenever the UE changes the PSCell, and when Dual Connectivity is activated.

If reporting upon change of serving cell is requested, the NG-RAN node shall send a report immediately and shall send a report whenever the UE’s location changes.

If the Event Type IE is set to "stop UE presence in the area of interest", and if the Additional Cancelled Location Reporting Reference ID List IE is included in the Location Reporting Request Type IE within the LOCATION REPORTING CONTROL message, the NG-RAN node shall, if supported, stop reporting UE presence for all received Location Reporting Reference IDs.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.1.16	User Location Information

This IE is used to provide location information of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  | - |  |
| >E-UTRA user location information |  |  |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >NR user location information |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 | This IE is ignored if the NR NTN TAI Information IE is present. | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >>NID | O |  | 9.3.3.42 |  | YES | reject |
| >>NR NTN TAI Information | O |  | 9.3.3.53 |  | YES | ignore |
| >>Mobile IAB-MT User Location Information | O |  | 9.3.1.260 | Indicates the user location information of a mobile IAB-MT, which is co-located with the mobile IAB-DU which serves the UE. | YES | ignore |
| >>Altitude | O |  | INTEGER(-420..10000) | Indicates the altitude of aerial UE as specified in TS 38.331 [18]. | - | reject |
| >>Velocity | O |  | OCTET STRING (SIZE(8)) | Indicates the velocity of aerial UE as specified in TS 38.331[18]. | - | reject |
| >N3IWF user location information |  |  |  |  |  |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TNGF user location information |  |  |  |  | YES | ignore |
| >>TNAP ID | M |  | OCTET STRING | TNAP Identifier used to identify the TNAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the TNGF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TWIF user location information |  |  |  |  | YES | ignore |
| >>TWAP ID | M |  | OCTET STRING | TWAP Identifier used to identify the TWAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | Non-5G-Capable over WLAN device's local IP address used to reach the TWIF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >W-AGF user location information |  |  |  | Indicates the location information via wireline access as specified in TS 23.316 [34]. | YES | ignore |
| >>W-AGF user location information | M |  | 9.3.1.164 |  | - |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.1.65	Location Reporting Request Type

This IE indicates the type of location request to be handled by the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Event Type | M |  | ENUMERATED (direct, change of serving cell, UE presence in the area of interest, stop change of serving cell, stop UE presence in the area of interest, cancel location reporting for the UE, … change of serving cell and UE presence in the area of interest, aerial UE information) |  | - |  |
| Report Area | M |  | ENUMERATED (cell, …) |  | - |  |
| Area of Interest List |  | 0..1 |  |  | - |  |
| >Area of Interest Item |  | 1..<maxnoofAoI> |  |  | - |  |
| >>Area of Interest | M |  | 9.3.1.66 |  | - |  |
| >>Location Reporting Reference ID | M |  | 9.3.1.76 |  | - |  |
| Location Reporting Reference ID to be Cancelled | C- ifEventTypeisStopUEPresinAoI |  | Location Reporting Reference ID 9.3.1.76 |  | - |  |
| Additional Location Information | O |  | ENUMERATED (Include PSCell, ...) |  | YES | ignore |
| Additional Cancelled Location Reporting Reference ID List |  | 0..1 |  |  | YES | reject |
| >Additional Cancelled Location Reporting Reference ID Item |  | 1..<maxnoofAoIMinusOne> |  |  | - |  |
| >>Location Reporting Reference ID to be Cancelled | M |  | Location Reporting Reference ID 9.3.1.76 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAoI | Maximum no. of areas of interest. Value is 64. |
| maxnoofAoIMinusOne | Maximum no. of areas of interest minus one. Value is 63. |

| Condition | Explanation |
| --- | --- |
| ifEventTypeisStopUEPresinAoI | This IE shall be present if the Event Type IE is set to the value "stop UE presence in the area of interest". |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

/******* Unchanged part skipped *******/

id-UserPlaneFailureIndication,

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-UserLocationInformationN3IWF-without-PortNumber,

id-Altitude,

id-Velocity,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

maxnoofAoIMinusOne,

maxnoofBluetoothName,

maxnoofBPLMNs,

maxnoofCAGforMDT,

maxnoofCAGSperCell,

/******* Unchanged part skipped *******/

FROM NGAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM NGAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Container{},

NGAP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

NGAP-PROTOCOL-IES

FROM NGAP-Containers;

#### -- A

AdditionalDLUPTNLInformationForHOList ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF AdditionalDLUPTNLInformationForHOItem

AdditionalDLUPTNLInformationForHOItem ::= SEQUENCE {

additionalDL-NGU-UP-TNLInformation				UPTransportLayerInformation,

additionalQosFlowSetupResponseList				QosFlowListWithDataForwarding,

additionalDLForwardingUPTNLInformation			UPTransportLayerInformation 							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AdditionalDLUPTNLInformationForHOItem-ExtIEs} }	OPTIONAL,

...

}

AdditionalDLUPTNLInformationForHOItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalRedundantDL-NGU-UP-TNLInformation	CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional		},

...

}

AdditionalQosFlowInformation ::= ENUMERATED {

more-likely,

...

}

/******* Unchanged part skipped *******/

AlternativeQoSParaSetIndex ::= INTEGER (1..8, ...)

AlternativeQoSParaSetNotifyIndex ::= INTEGER (0..8, ...)

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQosParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			AlternativeQoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate												OPTIONAL,

guaranteedFlowBitRateUL				BitRate												OPTIONAL,

packetDelayBudget					PacketDelayBudget									OPTIONAL,

packetErrorRate						PacketErrorRate										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MaximumDataBurstVolume	CRITICALITY ignore	EXTENSION MaximumDataBurstVolume		PRESENCE optional		},

...

}

Altitude ::= INTEGER (-420..10000)

/******* Unchanged part skipped *******/

EUTRAencryptionAlgorithms ::= BIT STRING (SIZE(16, ...))

EUTRAintegrityProtectionAlgorithms ::= BIT STRING (SIZE(16, ...))

EventType ::= ENUMERATED {

direct,

change-of-serve-cell,

ue-presence-in-area-of-interest,

stop-change-of-serve-cell,

stop-ue-presence-in-area-of-interest,

cancel-location-reporting-for-the-ue,

...,

change-of-serving-cell-and-UE-presence-in-the-Area-of-Interest,

aerial-ue-information,

}

ExcessPacketDelayThresholdConfiguration ::= SEQUENCE (SIZE(1..maxnoofThresholdsForExcessPacketDelay)) OF ExcessPacketDelayThresholdItem

/******* Unchanged part skipped *******/

UserLocationInformation ::= CHOICE {

userLocationInformationEUTRA	UserLocationInformationEUTRA,

userLocationInformationNR		UserLocationInformationNR,

userLocationInformationN3IWF-with-PortNumber	UserLocationInformationN3IWF-with-PortNumber,

choice-Extensions		ProtocolIE-SingleContainer { {UserLocationInformation-ExtIEs} }

}

UserLocationInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UserLocationInformationTNGF		CRITICALITY ignore	TYPE UserLocationInformationTNGF		PRESENCE mandatory	}|

{ ID id-UserLocationInformationTWIF		CRITICALITY ignore	TYPE UserLocationInformationTWIF		PRESENCE mandatory	}|

{ ID id-UserLocationInformationW-AGF	CRITICALITY ignore	TYPE UserLocationInformationW-AGF		PRESENCE mandatory	}|

{ ID id-UserLocationInformationN3IWF-without-PortNumber	CRITICALITY ignore	TYPE UserLocationInformationN3IWF-without-PortNumber		PRESENCE mandatory	},

...

}

UserLocationInformationEUTRA ::= SEQUENCE {

eUTRA-CGI			EUTRA-CGI,

tAI					TAI,

timeStamp			TimeStamp																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationEUTRA-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationEUTRA-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PSCellInformation	CRITICALITY ignore	EXTENSION NGRAN-CGI				PRESENCE optional},

...

}

UserLocationInformationN3IWF-with-PortNumber ::= SEQUENCE {

iPAddress			TransportLayerAddress,

portNumber			PortNumber,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationN3IWF-with-PortNumber-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationN3IWF-with-PortNumber-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-TAI			CRITICALITY ignore	EXTENSION TAI							PRESENCE optional	},

...

}

UserLocationInformationN3IWF-without-PortNumber ::= SEQUENCE {

iPAddress			TransportLayerAddress,

tAI					TAI								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { UserLocationInformationN3IWF-without-PortNumber-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationN3IWF-without-PortNumber-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UserLocationInformationTNGF ::= SEQUENCE {

tNAP-ID				TNAP-ID,

iPAddress			TransportLayerAddress,

portNumber			PortNumber															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationTNGF-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationTNGF-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-TAI			CRITICALITY ignore	EXTENSION TAI							PRESENCE optional	},

...

}

UserLocationInformationTWIF ::= SEQUENCE {

tWAP-ID				TWAP-ID,

iPAddress			TransportLayerAddress,

portNumber			PortNumber															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationTWIF-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationTWIF-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-TAI			CRITICALITY ignore	EXTENSION TAI							PRESENCE optional	},

...

}

UserLocationInformationW-AGF ::= CHOICE {

globalLine-ID	GlobalLine-ID,

hFCNode-ID		HFCNode-ID,

choice-Extensions		ProtocolIE-SingleContainer { { UserLocationInformationW-AGF-ExtIEs} }

}

UserLocationInformationW-AGF-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalCable-ID 	CRITICALITY 	ignore 	TYPE 	GlobalCable-ID 		PRESENCE 	mandatory }|

{ ID id-HFCNode-ID-new 	CRITICALITY 	ignore 	TYPE 	HFCNode-ID-new 		PRESENCE 	mandatory }|

{ ID id-GlobalCable-ID-new 	CRITICALITY ignore 	TYPE 	GlobalCable-ID-new 	PRESENCE 	mandatory },

...

}

UserLocationInformationNR ::= SEQUENCE {

nR-CGI				NR-CGI,

tAI					TAI,

timeStamp			TimeStamp															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationNR-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationNR-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PSCellInformation						CRITICALITY ignore	EXTENSION NGRAN-CGI											PRESENCE optional	}|

{ ID id-NID										CRITICALITY reject	EXTENSION NID												PRESENCE optional	}|

{ ID id-NRNTNTAIInformation						CRITICALITY ignore	EXTENSION NRNTNTAIInformation 							PRESENCE optional	}|

{ ID id-MobileIAB-MTUserLocationInformation		CRITICALITY ignore	EXTENSION MobileIAB-MTUserLocationInformation 		PRESENCE optional	}|

{ ID id-Altitude								CRITICALITY reject	EXTENSION Altitude 											PRESENCE optional	}|

{ ID id-Velocity								CRITICALITY reject	EXTENSION Velocity 											PRESENCE optional	}|

,

...

}

UserPlaneFailureIndication ::= SEQUENCE {

userPlaneFailureType				UserPlaneFailureType,

uL-NGU-UP-TNLInformation			UPTransportLayerInformation,

dL-NGU-UP-TNLInformation			UPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { { UserPlaneFailureIndication-ExtIEs} }	OPTIONAL,

...

}

UserPlaneFailureIndication-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UserPlaneFailureIndicationReport ::= ENUMERATED {

qos-flows-and-tunnel-to-be-released,

qoS-flows-to-be-moved,

new-transport-address-allocated,

...

}

UserPlaneFailureType ::= ENUMERATED {

gtp-u-error-indication-received,

up-path-failure,

...

}

UserPlaneSecurityInformation ::= SEQUENCE {

securityResult			SecurityResult,

securityIndication		SecurityIndication,

iE-Extensions		ProtocolExtensionContainer { {UserPlaneSecurityInformation-ExtIEs} }	OPTIONAL,

...

}

UserPlaneSecurityInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- V

Velocity ::= OCTET STRING (SIZE(8))

VolumeTimedReportList ::= SEQUENCE (SIZE(1..maxnoofTimePeriods)) OF VolumeTimedReport-Item

VolumeTimedReport-Item ::= SEQUENCE {

startTimeStamp				OCTET STRING (SIZE(4)),

endTimeStamp				OCTET STRING (SIZE(4)),

usageCountUL				INTEGER (0..18446744073709551615),

usageCountDL				INTEGER (0..18446744073709551615),

iE-Extensions		ProtocolExtensionContainer { {VolumeTimedReport-Item-ExtIEs} } OPTIONAL,

...

}

VolumeTimedReport-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

/******* Unchanged part skipped *******/

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-AllowedNSSAI											ProtocolIE-ID ::= 0

id-AMFName												ProtocolIE-ID ::= 1

id-AMFOverloadResponse									ProtocolIE-ID ::= 2

id-AMFSetID												ProtocolIE-ID ::= 3

id-AMF-TNLAssociationFailedToSetupList					ProtocolIE-ID ::= 4

id-AMF-TNLAssociationSetupList							ProtocolIE-ID ::= 5

/******* Unchanged part skipped *******/

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-Altitude												ProtocolIE-ID ::=xxx

id-Velocity                                             ProtocolIE-ID ::=xxx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>