3GPP TSG-RAN WG3 Meeting #127	R3-250693

Athens, GR, 17-21 Feb, 2025

Title: 	Discussion on the emergency services and eDRX

Source: 	Huawei

Agenda item:	8.1

Document Type:	 Discussion

# 1. Introduction

The last RAN2-128 meeting sent an LS on the emergency services and eDRX as follows [1].

In RAN2#128, companies discussed a scenario where a UE is configured with a RAN eDRX cycle in RRC_INACTIVE state when the UE has a PDU session associated with emergency services. RAN2 notice the below description in TS 23.501 which states that an Idle eDRX cycle should not be used by the UE when it has a PDU session associated with emergency services. However, it is unclear whether it is possible for such UE to be released to RRC_INACTIVE state with a RAN eDRX cycle and if it is so how UE behavior may be. #### 5.31.7.2 Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE ##### 5.31.7.2.1 Overview ##### // skipped irrelevant part When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure. // skipped irrelevant part. Based on current specification, a RAN eDRX cycle can be configured by a RAN node when releasing the UE to RRC_INACTIVE state only if the UE is configured with an Idle eDRX cycle by CN. RAN2 assumes that RAN node should be aware whether a UE has a PDU session associated with emergency services so that it can avoid releasing such UE to RRC_INACTIVE state with a RAN eDRX cycle, or even if it is so, there should be means for the RAN node and the UE to not use the configured RAN eDRX cycle. Regarding how RAN node can be aware of such PDU session, RAN2 understands that CN can already indicate this (e.g. via ARP value) but there is no requirement for CN to do this, so, there is case that RAN node may not be aware of this. RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed? # 2 Actions To SA2, CT1 group ACTION: RAN2 would like SA2 and CT1 to consider the scenario described above, provide feedback regarding how the identified issue can be addressed and whether any standard solution is needed?

Though RAN3 is in CC, this contribution provides our views on the received LS, if the online time allows.

# 2. Discussion

First, the LS from RAN2 mentioned that the NG-RAN may not be aware of the emergency PDU session, since there is no requirement for CN to indicate this (e.g., via ARP value). When referring to the TS 38.300, there are some procedure texts on the emergency services, in case of Paging Subgrouping assistance information as excerpted below.

### 9.2.2 Mobility in RRC_INACTIVE #### 9.2.2.1 Overview <Skip the irrelevant> The NG-RAN node takes into account the PEI with Paging Subgrouping assistance information for subgroup paging in RRC_INACTIVE except when the UE context contains an emergency PDU session in which case the PEI with Paging Subgrouping assistance information shall not be used according to TS 24.501 [28].

It can be observed that the NG-RAN can identify the emergency PDU session and the details are referred to TS 23. 501.

- Observation1: the NG-RAN could identify the emergency PDU session and the details are referred to TS 23. 501.

Second, as indicated in the LS from RAN2, the Idle eDRX cycle should not be used by the UE when it has a PDU session associated with emergency services. However, it is unclear whether it is possible to RRC_inactive UE configured with a RAN eDRX cycle when it holds a PDU session associated with emergency services. We consider that the same as idle UE, the RRC inactive UEs should not be configured with RAN eDRX in case of emergency services. Otherwise, the idle behaviour and inactive UE behaviour are not aligned.

RAN3 to consider that from RAN3 perspective, the RAN eDRX should not be configured for RRC inactive UE in case of emergency PDU session, which allows the idle UE behaviour and inactive UE behaviour are aligned.

Given that a RAN eDRX may be configured by a RAN node when releasing the UE to RRC_INACTIVE state only if the UE is configured with an Idle eDRX by CN. There are two possible solutions as follows.

- Option 1: when the AMF identifies the emergency PDU session, it updates the RRC Inactive Assistance Information to exclude the extended idle mode eDRX information. Then the NG-RAN would not configure the RAN eDRX for the inactive UE.

- Option 2: when the NG-RAN itself identifies the emergency PDU session, it would not configure the RAN eDRX for the inactive UE

Both of these two solutions could work. RAN3 can initially discuss these two options.

RAN3 to initially discuss the two solutions below:

  - Option 1: when the AMF identifies the emergency PDU session, it updates the RRC Inactive Assistance Information to exclude the extended idle mode eDRX information. Then the NG-RAN would not configure the RAN eDRX for the inactive UE.

  - Option 2: when the NG-RAN itself identifies the emergency PDU session, it would not configure the RAN eDRX for the inactive UE

In addition, SA2#166 Ad hoc meeting also discussed this issue, but without conclusion.  The main point in SA2 is to check with CT1 whether they foresee any scenarios for the UE to receive SIP signalling as part of the MO emergency session (e.g., as indicated in [2]). Given the discussion is ongoing, RAN3 can continue to monitor the progress, and revisit our agreements if any.

RAN3 to continue monitoring the SA2 progress, then revisit the agreements if any.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- Observation1: the NG-RAN could identify the emergency PDU session and the details are referred to TS 23. 501.

- RAN3 to consider that from RAN3 perspective, the RAN eDRX should not be configured for RRC inactive UE in case of emergency PDU session, which allows the idle UE behaviour and inactive UE behaviour are aligned.

RAN3 to initially discuss the two solutions below:

  - Option 1: when the AMF identifies the emergency PDU session, it updates the RRC Inactive Assistance Information to exclude the extended idle mode eDRX information. Then the NG-RAN would not configure the RAN eDRX for the inactive UE.

  - Option 2: when the NG-RAN itself identifies the emergency PDU session, it would not configure the RAN eDRX for the inactive UE

RAN3 to continue monitoring the SA2 progress, then revisit the agreements if any.

The draft CRs to reflect the above two options in proposal 2 could be found in [3-4], just for reference.

# 4. Reference

- R2-2411041 LS on emergency services and eDRX, RAN2

- S2-2500133 Emergency call back and delayed paging, Qualcomm

- R3-250694	emergency services and eDRX for RRC inactive UE, Huawei

- R3-250695	emergency services and eDRX for RRC inactive UE, Huawei