3GPP TSG RAN WG3 Meeting #127	R3-250703

Athens, Greece, 17 – 21 Feb, 2025

Agenda Item:	13.2

Source:				ZTE Corporation

Title:	TP to TS 38.401 on conditional intra-CU LTM

Document for:	Discussions & Approval

- Introduction

In this meeting, RAN3 has received an LS (R3-250008) from RAN2, which includes the following progress from RAN2 on the conditional intra-CU LTM.

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

The intention of this contribution is to explain our view based on RAN2 progress and capture the potential agreements into a TP.

- Discussion

  - LTM preparation

Step 1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

Based on the above RAN2 agreement, we provide the following agreement.

Proposal 1: The UE sends a MeasurementReport message (L1 measurement result and/or L3measurment result) to the source DU.

Proposal 2: CU may inform DU the L1 measurement result and/or L3 measurement result.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

According to the above agreements, the source DU sends L1 execution condition to the CU for initial conditional intra-CU LTM by UE context modification response message, and candidate DU sends L1 execution condition to CU for subsequent conditional intra-CU LTM by UE context setup response messge. Furthermore, the L1 execution condition is included in the DU to CU RRC Information IE.

Proposal 3: Both source DU and candidate DU(s) send L1 execution conditions to the CU for initial and subsequent conditional intra-CU LTM by UE context setup response and UE context modification response message.

## Proposal 4: The CU shall send L1/L3 execution conditions to the UE via source DU by DL RRC MESSAGE TRANSFER message.

  - Early Sync

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

DL synchronizaion

The UE performs DL synchronization with the LTM candidate cell(s) before receiving the cell switch command. The UE may activate and deactivate TCI states of LTM candidate cell(s), as triggered by the gNB.

Observation 1: For conditional intra-CU LTM, DL sync reuses the same Candidate Cell TCI States Activation/Deactivation MAC CE as that for inter-CU LTM.

UL synchronization

Observation 2: For conditional intra-CU LTM, the network informs the candidate cell’s TA information to UE via new MAC CE, Candidate cell TA is maintained by a new timer.

- Conclusion

Proposal 1: The UE sends a MeasurementReport message (L1 measurement result and/or L3measurment result) to the source DU.

Proposal 2: CU may inform DU the L1 measurement result and/or L3 measurement result.

Proposal 3: Both source DU and candidate DU(s) send L1 execution conditions to the CU for initial and subsequent conditional intra-CU LTM by UE context setup response and UE context modification response message.

## Proposal 4: The CU shall send L1/L3 execution conditions to the UE via source DU by DL RRC MESSAGE TRANSFER message.

Observation 1: For conditional intra-CU LTM, DL sync reuses the same Candidate Cell TCI States Activation/Deactivation MAC CE as that for inter-CU LTM.

Observation 2: For conditional intra-CU LTM, the network informs the candidate cell’s TA information to UE via new MAC CE, Candidate cell TA is maintained by a new timer.

- Text Proposal to TS38.401

8.2.1.x Subsequent Conditional intra-CU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for subsequent Conditional intra-CU LTM. Figure 8.2.1.x-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.x-1: Inter-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L1 or L3 measurement result, e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for subsequent intra-CU conditional LTM, e.g., to trigger PDCCH ordered early RACH) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate conditional intra-CU LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing one candidate cell ID and the CSI resource configuration for subsequent conditional intra-CU LTM. The gNB-CU may provide the LTM configuration ID mapping list to the candidate gNB-DU(s). The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The UE CONTEXT SETUP REQUEST message is sent for each candidate cell and includes a CG-ConfigInfo (subsequent conditional intra-CU LTM)

4.	If the candidate gNB-DU accepts the request of subsequent conditional intra-CU LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell and the target cell ID that was requested from the gNB-CU and the L1execution conditions for subsequent conditional intra-CU LTM, which are generated by the candidate cell to trigger the subsequent conditional intra-CU LTM for other candidate cells when the candidate cell becomes a serving cell. The response message is sent for each requested candidate cell.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration ID mapping list for the accepted target candidate cell(s). The gNB-CU may send the updated CSI resource configuration to the source gNB-DU.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell, and includes the L1 execution conditions for initial conditional intra-CU LTM, which is generated by the initial source cell to trigger the subsequent intra-CU LTM for the candidate cell.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information for subsequent conditional intra-CU LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration, e.g., containing the updated CSI report configuration of the candidate gNB-DU.

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent conditional intra-CU LTM.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message containing both L1 execution condition and L3 execution condition with the subsequent conditional intra-CU LTM configuration.

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13.	The UE may perform DL synchronization with LTM candidate cell(s), as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a subsequent conditional intra-CU LTM candidate configuration.

RAN2 agreement: the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

14. The UE my perform UL synchronization with LTM candidate cell(s), as specified in clause 9.2.3.5.2 in TS 38.300 [3].

15.The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the gNB-CU, which includes the TA values, and the associated PRACH resource information.

16.	The gNB-CU forwards the TA value, and the associated PRACH resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

17. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during the subsequent conditional intra-CU LTM.

18. An L1 or L3 execution condition to trigger initiation of subsequent conditional intra-CU LTM is fulfilled.

19. UE sends Cell Swtich Notification message by a new UL MAC CE to the source gNB-DU, including target cell id and CG resources [Pending to RAN2].

20. The source gNB-DU sends DU-CU Cell Swtich Notification message

19. A Random Access procedure is performed at the candidate gNB-DU, which becomes the target gNB-DU if successful.

20. The target gNB-DU sends a Downlink Data Delivery Status frame to inform the gNB-CU.

21. The target gNB-DU also sends an ACCESS SUCCESS message to the gNB-CU with the target cell ID, to inform the gNB-CU of which cell the UE has successfully accessed.

22.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

23.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

24. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU and indicates to stop the data transmission for the UE.

NOTE 1:	The step 24 may happen before step 23, as soon as the gNB-CU knows which cell the UE has successfully accessed.

NOTE 2:	The gNB-CU may send the UE CONTEXT RELEASE message to inform source gNB-DU to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the source gNB-DU. Any ongoing data forwarding may continue.

25. The source gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source gNB-DU, are sent from the gNB-CU to the target gNB-DU.

26.	The source gNB-DU responds to the gNB-CU with the UE CONTEXT MODIFICATION RESPONSE message.

27.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

28.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

The step 26~27 shall not happen if part of the PSCells in the source gNB-DU remain configured as the candidate PSCell(s) in the subsequent conditional intra-CU LTM.

NOTE X:	The steps 13-26 can be performed multiple times for subsequent conditional intra-CU LTM execution using the SCG LTM candidate configuration(s) provided in step 10.