3GPP TSG-RAN WG3 Meeting #R3-127		           R3-250720

Athens, Greece, Feb 17-21, 2025

Agenda Item:	11.2

Source: 	Rakuten

Title:  	UE Performance Granularity for AI/ML based Network Slicing

Document for:	Discussion and decision

# Introduction

Related to UE performance feedback of the handed over UEs from the target node to the source node, we list below the relevant references:

From 38.743:

Following are the notes from RAN3#125-bis meeting:

Slice UE performance:

Observation based on current specs: Average packet delay UL/DL, average UE throughput (UL/DL), and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI.

The granularity of UE performance:

Op1: per S-NSSAI per PDU Session: ZTE, CMCC, Jio, TI, E///, LGE

Opt2: per S-NSSAI: Lenovo, SS, HW, NEC

Opt3: per-QoS flow groups: CATT, Nok

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

FFS if other finer granularity of UE performance is feasible.

# Discussion

In the previous meetings, it has been discussed and agreed that after successful HO completion from source to target gNB, the target gNB will send UE performance metrics to the source gNB. This will assist the source gNB to evaluate the merits of its mobility decisions and act as feedback to validate the accuracy of its AI/ML training and prediction.

## Granularity of UE performance metric

In RAN3#125-bis meeting, we concluded the discussion of UE performance with a working assumption that a UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node. And it was left for further investigation if other finer granularity of UE performance is feasible.

Based on the contributions received in the previous meeting, these are the various levels at which UE performance feedback is desired by different companies:

- Per S-NSSAI level

- Per PDU level

- Per DRB level

- Per QoS-flow level

The companies have explained why some of these granularities are not suitable for this use case. And many papers have highlighted the lack of clarity in the standard from SA5 on how the measurements mentioned in TS 28.558 are computed, especially for Average DL/UL UE Throughput.

Our thought on this topic is that if a higher granularity of per S-NSSAI level or Per PDU-session level is supported, it might result in loss of integrity while comparing the UE performance at the source node with respect to the performance at the target node.

If the UE performance after handover is compared at the slice level, or the PDU session layer associated with a slice, we believe that the comparison might not be equitable. This might happen because a PDU session with certain QoS flows in a source NG-RAN node might be handed over to the target NG-RAN node with some of the QoS flows being denied admission at the target NG RAN node, as supported in 38.423, shown below. This results in the slice or the PDU session at the target NG-RAN node having different mapping as compared to the source NG-RAN node

Therefore, the performance metrics at per-S-NSSAI and per-PDU session level might not fairly represent the UE performance at the target NG RAN node.

Observation 1: Due to the possibility of partial QoS flow admittance at the target NG RAN node during the handover procedure, the per S-NSSAI level and per-PDU session level UE performance might not result in a fair comparison of the handed over UE between the source and target NG RAN nodes.

For this reason, we propose that finer granularity of UE performance feedback be considered to support the validation of AI/ML prediction at the source NG RAN node.

- We propose the use of finer granularity of UE-level UE performance metrics at per-QoS flow level.

Average DL UE throughput in gNB is defined in 28.558 (Section 6.3.1.4.1) and the following forms are supported as mentioned in field e:

In 28.552 Section 4.2, examples of possible filter values are given as 5QI, QCI, SNSSAI, and PLMN. Furthermore, in Section 4.2.2 Filters naming, the use of multiple filters is given as:

<Performance measurement>_<Filter1>_<Filter2>

Therefore, for supporting the current use case of Network Slicing, to get the UE performance metric at QoS flow level for the relevant S-NSSAI we propose to use

<Performance measurement>_<SNSSAI>_<QoS>

Let’s consider the figure below which shows the mapping of the DRBs and associated QoS flows with respect to each slice in the source cell and the target cell. It’s possible that some QoS flows are not admitted at the target NG RAN node during HO. In the example below, QoS flow #3 is not admitted in the target cell. It is also possible that some of the QoS flows are remapped to different DRBs or different slices at the target NG RAN node In the example below, QoS flow #5 has been remapped to DRB3 in S-NSSAI 2.

Given the above possibility, per-DRB level or per-S-NSSAI level comparison of UE performance of the handed off UE from source to target cell might result in inequitable comparison. Hence, for lower granularities, the option for most fair comparison seems to be at the per-QoS flow level.

# Conclusion

We have the following observations and proposals:

Observation 1: Due to the possibility of partial QoS flow admittance at the target NG RAN node during the handover procedure, the per-PDU session level UE performance might not result in a fair comparison of the handed over UE between the source and target NG RAN nodes.

Proposal 1: We propose the use of finer granularity of UE performance metrics at per-QoS flow level.

# Reference

[1] TS28.552 5G performance measurements

[2] TS25.558 UE level measurements for 5G system

[3] TS38.423 Xn application protocol (XnAP)