3GPP TSG RAN3 meeting #127   	         	                         R3-250735

17th – 21th Feb 2025  Athens, Greece

Agenda Item:	10.3.1

Source:	China Unicom

Title:	Discussion on SON/MDT enhancements for NTN mobility

Document for:	Discussion

Introduction

The Work Item on SON/MDT enhancement has been agreed in RAN#102 (RP-234038) with the following objectives:

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure

This paper discusses the SON/MDT enhancements for NTN mobility.

Discussion

Condition Handover Optimization for NTN network

Time-based CHO Optimization for NTN network

The time-based CHO will configure the t1-Threshold and the duration for the handover, and the UE will execute the handover during the configured time duration. Even if the handover is successful, and there is no radio link failure happened after the CHO, the CHO may also need to be optimized. The CHO may happen at the beginning of the t1-Threshold, or nearly at the end of the t1-Threshold + duration, therefore the gNB also needs to get the CHO related information to further optimize the CHO configuration to avoid the potential handover failure. The condition for the UE to report the CHO related information can be configured by the gNB.

Proposal 1: The gNB can configure the CHO successful report condition for the UE to report the time-based CHO related information.

Proposal 2: The UE can record and report the handover time and time-based CHO related information to the gNB when the condition for the UE to report the time-based CHO related information if satisfied.

Location-based CHO Optimization for NTN network

The location-based CHO will configure location-based CHO condition for the handover, and the UE will execute the handover during the configured condition. Even if the handover is successful, and there is no radio link failure happened after the CHO, the CHO may also need to be optimized. The CHO may happen near the location of distanceThreshFromReference1 or distanceThreshFromReference2, therefore the gNB also needs to get the CHO related information to further optimize the CHO configuration to avoid the potential handover failure. The condition for the UE to report the CHO related information can be configured by the gNB.

Proposal 3: The gNB can configure the CHO successful report condition for the UE to report the location-based CHO related information.

Proposal 4: The UE can record and report the location-based CHO related information to the gNB when the condition for the UE to report the location-based CHO related information if satisfied.

MDT Measurement Enhancement for NTN Network

In RAN3#126 meeting, there is the following agreement:

RAN3 assumes both geographical area and the mapped cell ID as Area Scope of logged MDT for NTN over NGAP are feasible and confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for MDT NTN.

For the geographical area definition, the  MBS NTN can be reused for MDT NTN. RAN2 agreed the following proposal in R2-2410638:

Proposal 1: The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

The reference location configuration is critical for the UE to execute cell re-selection and handover, and how to configure the reference location and the distance threshold for UE and reference location relies on the network deployment for NTN network. When the reference location and the distance threshold are not configured properly, there will be coverage hole or too much overlap coverage, which will lead to UE handover fail or unnecessary measurements for UE. The reference location and distance threshold configuration need to be optimized according to the measurements based on the reference location.

Proposal 5: The reference location and distance threshold configuration need to be optimized for UE cell re-selection and CHO.

For the cell re-selection and CHO case, the distance between the reference location is the critical judgment parameters according the condition configured by the gNB. How to optimize the configuration for cell re-selection and CHO relies on the measurements of the reference signal when the cell re-selection and CHO happen. The MDT measurement can be used for the measurements of the reference signal for certain distance to the reference point, and the reference point can be the reference point configured or will be configured for the cell re-selection and CHO.

Proposal 6: RAN3 agreed to introduce a reference location plus a radius as the area scope for MDT in Ng interface.

Conclusion

Proposal 1: The gNB can configure the CHO successful report condition for the UE to report the time-based CHO related information.

Proposal 2: The UE can record and report the handover time and time-based CHO related information to the gNB when the condition for the UE to report the time-based CHO related information if satisfied.

Proposal 3: The gNB can configure the CHO successful report condition for the UE to report the location-based CHO related information.

Proposal 4: The UE can record and report the location-based CHO related information to the gNB when the condition for the UE to report the location-based CHO related information if satisfied.

Proposal 5: The reference location and distance threshold configuration need to be optimized for UE cell re-selection and CHO.

Proposal 6: RAN3 agreed to introduce a reference location plus a radius as the area scope for MDT in Ng interface.