3GPP TSG-RAN WG3 Meeting #127	R3-250749

Athnes, Greece, 17 - 21 Feb, 2025

Title:	Response document to R3-250093

Agenda Item:	31.2

Source:	Qualcomm Inc, ZTE

Document for:	Discussion/Agreement

# 1	Introduction

In RAN3#125bis, RAN3 received reply LS [1] from RAN2 about SDT signalling optimization for partial UE context transfer procedure and given below.

RAN2 would like to thank RAN3 for the reply LS on SDT signalling optimization for partial context transfer in R2-2404123 (R3-242198).

With regards to the network-based solution discussed in RAN3, RAN2 would like to inform RAN3 that:

- If RRCSetup is received during SDT, the UE will discard any pending UL data and set up a new RRC Connection. Thus, the pending UL data will be lost.

- If RRCSetup is received during SDT, the UE will discard the current INACTIVE AS context and AS security context which requires RRC establishment related signalling to establish AS context and AS security context again

In addition, RAN2 also discussed whether there is any issue if MAC is not reset upon receiving RRCSetup after data is exchanged during SDT. RAN2 discussed whether the network can handle this scenario without performing a MAC reset by network implementation or if a MAC reset is needed. Some companies in RAN2 are not sure whether network implementation can handle this without MAC reset. RAN2 would like to add that if a MAC reset is needed, this would be a non-backward compatible change to Rel-17 and is not desirable.

After RAN2 UE based solution was agreed, some companies in RAN3 realized that it would be better to develop a network based solution without any SDT UE impact to provide latency benefit for all SDT UEs from R17 and onwards. Motivation for developing network based solution without depending on R17 SDT UE capability requirement is really good. But when RAN3 starts developing network based solutions, we also need to  carefully consider end to end signalling overhead and whether all R17 SDT UEs can really get the intended benefit of reduced latency to transition RRC_INACTIVE SDT UE into RRC_CONNECTED state directly upon receiving non-SDT DL downlink data/signalling at the last serving anchor gNB without an additional negative consequences of overall mechanism.

Network based solution proposed in [2] was discussed in RAN3#125bis and following are chairman notes:

Whether MAC reset is needed for network-based solution needs to be further checked.

No issue on AMF redirecting NG connections (i.e., NG connection from old gNB to new gNB), reuse the legacy procedure. Call flow may need to be checked.

No issue for DL data loss by data forwarding.

The network based solution does not apply to UL data/signaling and DL MBS configuration scenario.

To be continued in AI8.3.

In RAN3#126, this issue was discussed again and it was decided not to pressure in Rel-18.

The issues of network based solution: Issue#1: Signalling Overhead due to UE releasing AS context and re-initiating NAS/AS procedures Issue#2: UE buffer data loss due to releasing AS context Issue#3: UE MAC reset issue requiring new SDT UE capability. Issue#4: Both UE and CN cannot guarantee to establish the same PDU session/QoS flow profile with the old one, DL data forwarding cannot be supported The network based solution is not supported in R18 taking the concerns above into account.

In RAN3#127, following contributions are submitted again for Rel-19.

| R3-250093 | Introduction of SDT network-based solution (Nokia, Ericsson, Huawei, LG Electronics, CATT) | discussion |
| --- | --- | --- |
| R3-250094 | Response LS on SDT signalling optimization for partial context transfer (Nokia, Ericsson, Huawei, LG Electronics, CATT) | LS out To: RAN2 CC: |
| R3-250112 | Introduction of SDT network-based solution [SDT-NW-sol] (CATT, Huawei, Nokia, Ericsson, LG Electronics) | CR0456r, TS 38.401 v18.4.0, Rel-19, Cat. B |
| R3-250125 | Introduction of SDT network-based solution [SDT-NW-sol] (LG Electronics, Nokia, Ericsson, Huawei, CATT) | CR1534r, TS 38.473 v18.4.0, Rel-19, Cat. B |
| R3-250157 | Introduction of SDT network-based solution [SDT-NW-sol] (Ericsson, Nokia, Huawei, LG Electronics, CATT) | CR1443r, TS 38.423 v18.4.0, Rel-19, Cat. B |
| R3-250159 | Introduction of SDT network-based solution [SDT-NW-sol] (Huawei, Nokia, Ericsson, LG Electronics, CATT) | draftCR |

In this response document contribution, we discuss primarily focusing on summary of key issues and explain that for network based solution by changing 3GPP release is not going to help and conclude this discussion.

# 2	Discussion

Issue#1: Signalling Overhead due to UE releasing AS context and re-initiating NAS/AS procedures

When UE is exchanging DL/UL data in RRC_INACTIVE state via SDT procedure, if any non-SDT DL data/signalling is received at the last serving gNB (anchor gNB hosting RRC/PDCP context of SDT UE), last serving gNB has to either terminate the ongoing SDT procedure by sending RRC Release message or it has transfer UE context transfer from last serving gNB to receiving gNB via enhanced Xn procedure, which can cause receiving gNB to send RRCSetup message to SDT RRC_INACTIVE state UE during on going active SDT data transfer. During this process, UE may have pending data in both DL/UL buffers in L2 MAC. Upon receiving RRC Setup message during an ongoing SDT procedure, which triggers an RRC connection resume fallback procedure for UE to establish RRC connection which causes following UE actions , e.g. as highlighted below, per TS 38.331:

#### 5.3.3.4 Reception of the RRCSetup by the UE The UE shall perform the following actions upon reception of the RRCSetup: 1> if the RRCSetup is received in response to an RRCReestablishmentRequest; or 1> if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1: 2> if the UE is NCR-MT: 3> indicate to NCR-Fwd to cease forwarding; 2> if sdt-MAC-PHY-CG-Config is configured: 3> instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running; 3> instruct the MAC entity to start the timeAlignmentTimer associated with the PTAG, if it is not running; 2> if srs-PosRRC-Inactive is configured: 3> instruct the MAC entity to stop the inactivePosSRS-TimeAlignmentTimer, if it is running; 2> if srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured: 3> instruct the MAC entity to stop the inactivePosSRS-ValidityAreaTAT, if it is running; 2> if the UE is configured with multicast reception in RRC_INACTIVE: 3> reset MAC; 2> discard any stored UE Inactive AS context and suspendConfig; 2> discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key; 2> release radio resources for all established RBs except SRB0 and broadcast MRBs, including release of the RLC entities, of the associated PDCP entities and of SDAP; 2> release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration, CCCH configuration and broadcast MRBs; 2> indicate to upper layers fallback of the RRC connection; 2> for each application layer measurement configuration with appLayerIdleInactiveConfig absent: 3> forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration; 3> release the application layer measurement configuration; 3> discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission; 3> consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId; 2> stop timer T380, if running; 1> perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5; 1> perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6; 1> if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT; 1> stop timer T300, T301, T319; 1> if T319a is running: 2> stop T319a; 2> consider SDT procedure is not ongoing; (...)

It is clear from the above UE procedure text , upon reception of RRCSetup, the UE releases its security context, all of its data radio bearers and its entire RRC configuration (except default values). The UE also indicates fallback of the RRC connection to upper layers (i.e., NAS). This means that all of the data buffered at the UE L2 is lost and that the network needs to reconfigure all the DRBs at the UE and the complete RRC configuration at the UE from scratch. This leads to lot of additional NAS and RRC signalling , longer service interruption time and requires the network and the UE to perform many unnecessary actions such as establishing user plane resources at both AS and upper layers again. That is why the fallback to RRC connection establishment is supposed to be utilized by the network only in exceptional cases, i.e. in case the retrieval of the UE context from the anchor gNB is not possible.

This additional signalling overhead and interruption defeats the goal of introducing network based solution to reduce latency to quickly move into RRC_CONNECTED state with minimal signalling overhead.

If the UE does not support signalling optimization procedure or is a R17 UE, it is better for the network to use the legacy procedure, i.e. release the UE to RRC INACTIVE and trigger the connection resumption via Paging. The benefits of the new solution over the legacy procedure are unclear as due to user plane reestablishment, the UE will suffer from data loss and additional latency anyway.

Observation 1: Fallback from RRC Resume procedure to RRC connection establishment makes the UE release its security context, all of its data radio bearers and its entire RRC configuration (except default values) causing large signalling overhead and data interruption due to additional procedures to be triggered in both AS and NAS level.

Observation 2: Fallback from RRC Resume procedure to RRC connection establishment causes large signalling overhead and is supposed to be utilized by the network only in exceptional cases, i.e. in case the retrieval of the UE context from the anchor gNB is not possible.

Issue#2: UE buffer data loss due to releasing AS context

If RRCSetup is received during SDT, after partial exchange of data on SDT radio bearers, the UE will discard the stored UE inactive AS context and suspendConfig and release all established radio bearers except SRB0. The associated RLC entities and the PDCP entities and SDAP for these bearers are also released. As a result, any pending data in the user plane protocol stack in both UL and DL will be lost.

Some companies argued that any DL data loss can be received by network implementations and UL data loss has to be recovered by UE implementation. This requires unnecessary complexity for UE implementation complexity and involves interaction between modem and application. This is not worth of both NW and UE added complexity.

Observation 3: Upon receiving RRC Setup message causes UE to reset AS context and all UP buffered data will get lost. This requires additional UE implementation complexity to recover lost UL data and NW implementation to avoid DL data loss. This complexity defeats the goal of network based solution to reduce latency.

Issue#3: UE MAC reset issue requiring new UE capability for R17 SDT.

As described in RRC text procedure, when the UE receives RRCRelease message, MAC will be reset and if the UE subsequently resumes the connection, then the gNB and the UE MAC state (i.e., HARQ Process ID and NDI for each HARQ process) must be aligned. Note that MAC procedures are defined w.r.t UE and if UE does MAC reset then gNB side DU MAC has to reset as well, otherwise there will be mis-sync between UE and gNB MAC, which causes loss of data. During RRC_INACTIVE SDT data transfer, UE will use some HARQ processes for data transfer and if RRCSetup is received in the middle of the ongoing SDT session, then the MAC state for each of the HARQ process needs to be aligned between the gNB and the UE. Upon UE context transfer from last serving gNB to receiving gNB via Xn procedures, receiving gNB has to identify the existing UE context in the DU for the ongoing SDT session and use the RRCSetup procedure in combination with the UE context modification procedure at the DU instead of using UE context setup procedure which is typically used for RRCSetup. This may result in a new procedure or implementation based procedure at the gNB side as well.

Observation 4: For sending RRC Setup message to UE, CU uses F1AP UE context setup procedure towards the DU currently and if the UE context in DU needs to be kept (i.e. for avoiding MAC reset) then a new procedure to keep UE context at the DU will be needed.

On the other hand, if RRCSetup is sent after some data has been exchanged between the gNB and the UE (especially DL data) – e.g. as in RRC_INACTIVE Multicast case, then MAC is reset to align HARQ process ID and NDI, DRX timers between UE and gNB and hence a new UE context in DU will be created created for the UE on the DU by the network.

Observation 5: If any data is exchanged between the gNB and the UE during INACTIVE state (e.g. Multicast reception in RRC_INACTIVE state) then MAC reset is performed upon reception of RRCSetup and network creates a new UE context in DU (which is the general procedure for any RRCSetup today).

Given the above situation and if RRCSetup is to be sent during ongoing SDT session, UE side MAC reset is needed and to create a new context for the UE on the network DU side which is currently the case (and is aligned with the RRC_INACTIVE Multicast).

It is possible to have simultaneous support Multicast RRC_INACTIVE reception and SDT data transfer. In such case, if UE gets RRCSetup message then as per Multicast procedures, UE MAC will be setup. If some companies argue that MAC is not needed for SDT scenario, then it is not clear how it is possible to have MAC reset for Multicast scenario and but No MAC Reset for SDT case. Both are contradictory to each other.

Observation 6: For simultaneous Multicast data reception in RRC_INACTIVE state and SDT data transfer, if RRCSetup message is received by UE, as per MBS procedures MAC will get reset and it is not possible to have no MAC reset for SDT alone.

So, if UE MAC reset is to be performed, then the UE procedure is different from Rel-17 UE behaviour and  it is not possible for the network to send RRCSetup to the UE during SDT after exchange of DL/UL data. Thus, either a new  capability is needed for the UE if we want to enable this feature or RAN2 should clarify that the network is not allowed to send RRCSetup after sending any DL/UL data.

RAN2 discussed this MAC reset issue and there was no clear conclusion. In order to end this discussion quickly in RAN2, reply LS stated that

“In addition, RAN2 also discussed whether there is any issue if MAC is not reset upon receiving RRCSetup after data is exchanged during SDT. RAN2 discussed whether the network can handle this scenario without performing a MAC reset by network implementation or if a MAC reset is needed. Some companies in RAN2 are not sure whether network implementation can handle this without MAC reset. RAN2 would like to add that if a MAC reset is needed, this would be a non-backward compatible change to Rel-17 and is not desirable.”

We believe that MAC Reset is needed at the UE side and this is not aligned with R17 SDT UE behaviour and requires new UE capability, which is not backward compatible with R17 SDT UE behaviour.

Observation 7: Need of UE MAC reset upon receiving RRCSetup during on going SDT transfer means a new UE capability is needed since this is not compatible with Rel-17 SDT UE operation, which defeats the purpose of network based solution.

Even though we fully acknowledge the motivation of introducing network based solution, due to additional signalling overhead, interruption during establishment of AS/NAS context towards UE, need of MAC reset requiring UE capability defeats the purpose of network based solution.

Issue#4: Changing of 3GPP Release for network based solution will not solve the problem.

One of the main issue (Issue#3 in this document) with network based solution described in R3-250093 is that it requires MAC reset and thus needs additional Rel-17 SDT UE capabilities and is clearly stated in RAN2 reply LS shown above.

Since R17 SDT UE new capability is needed, if we try to introduce network based solution in any release after R18 (i.e R19 or later), still requires same new UE capability. By introducing network based solution in different release does not change requirement for new UE capability. Thus, it is not possible to introduce network based solution in any release without having new capability.

Observation 8: Changing 3GPP release for network based solution does not change the R17 SDT UE requirement of new capability for MAC reset.

Thus we propose

Proposal 1: No need for RAN3 to introduce network based solution for Optimization of SDT signalling procedure in case of partial UE context transfer in different 3GPP release.

# 3	Conclusion

In this contribution, we discussed multiple issues for network based solutions intended to reduce latency trigger RRC_INACTIVE SDT UEs to transition into RRC_CONNECETD state when non-SDT DL data/signalling is received at the last serving gNB for the case of partial SDT UE context transfer.

Issue#1: Signalling Overhead due to UE releasing AS context and re-initiating NAS/AS procedures

Observation 1: Fallback from RRC Resume procedure to RRC connection establishment makes the UE release its security context, all of its data radio bearers and its entire RRC configuration (except default values) causing large signalling overhead and data interruption due to additional procedures to be triggered in both AS and NAS level.

Observation 2: Fallback from RRC Resume procedure to RRC connection establishment causes large signalling overhead and is supposed to be utilized by the network only in exceptional cases, i.e. in case the retrieval of the UE context from the anchor gNB is not possible.

Issue#2: UE buffer data loss due to releasing AS context

Observation 3: Upon receiving RRC Setup message causes UE to reset AS context and all UP buffered data will get lost. This requires additional UE implementation complexity to recover lost UL data and NW implementation to avoid DL data loss. This complexity defeats the goal of network based solution to reduce latency.

Issue#3: UE MAC reset issue requiring new SDT UE capability.

Observation 4: For sending RRC Setup message to UE, CU uses F1AP UE context setup procedure towards the DU currently and if the UE context in DU needs to be kept (i.e. for avoiding MAC reset) then a new procedure to keep UE context at the DU will be needed.

Observation 5: If any data is exchanged between the gNB and the UE during INACTIVE state (e.g. Multicast reception in RRC_INACTIVE state) then MAC reset is performed upon reception of RRCSetup and network creates a new UE context in DU (which is the general procedure for any RRCSetup today).

Observation 6: For simultaneous Multicast data reception in RRC_INACTIVE state and SDT data transfer, if RRCSetup message is received by UE, as per MBS procedures MAC will get reset and it is not possible to have no MAC reset for SDT alone.

Observation 7: Need of UE MAC reset upon receiving RRCSetup during on going SDT transfer means a new UE capability is needed since this is not compatible with Rel-17 SDT UE operation, which defeats the purpose of network based solution.

Observation 8: Changing 3GPP release for network based solution does not change the R17 SDT UE requirement of new capability for MAC reset.

Based on above observations, we propose

Proposal 1: No need for RAN3 to introduce network based solution for Optimization of SDT signalling procedure in case of partial UE context transfer in 3GPP release.

# 4	Reference

[1] R2-2407816 :	Reply LS on SDT signalling optimization for partial context transfer

[2] R3-245319 : Switch to RRC Connected after SDT Partial Context Transfer

[3] R2-2405649 : Discussion on RAN3 LS on SDT signalling optimization for partial context transfer

[4] R2-2406458 : SDT signalling optimization for partial context transfer

[5] R2-2404541: SDT signalling optimization for partial context transfer

[6] R2-2401058 : MAC Reset when switching to RRC_CONNECTED

[7] R2-2401363 : MAC Reset in State Transition from RRC_INACTIVE to RRC_CONNECTED