3GPP TSG-RAN WG3 Meeting #127	R3-250751

Athens, GR, Feb 17th – 21st, 2025

Title:	Response to R3-250739 and R3-250656

Agenda Item:	8.3

Source:	ZTE Corporation

Document for:	Discussion and decision

1 Introduction

In previous meeting, companies discussed how to distinguish QoE for broadcast or multicast at UE side in both RAN3 and SA4. RAN3 finally decided to ask SA4 assistance to introduce new MBS information into the QoE configuration container in LS R3-244789[1]. Before this meeting, SA4 informed RAN3 for their update on the TS 26.274 with the detailed correction in the reply LS S4-242151[2]. The intention of this contribution tries to response the RAN3 discussion papers R3-250739[3] and R3-250656[4] and provide our views on the SA4 update.

- Discussion

  - Recapture the Discussion Background

Based on the discussion in RAN3#125, RAN3 decided to ask SA4 to add new MBS indication in the QoE configuration which may be used by UE to notify whether this QoE measurement is for broadcast or multicast. Detail content can be checked in [1] and the content below comes from [1] and will be further discussed in this contribution.

============== content in R3-244789 ==============

Consequently, to eliminate the above ambiguity, RAN3 agreed that the UE should be notified whether the measurement configuration pertains to MBS broadcast or MBS multicast mode. The corresponding RAN3 agreement is:

The UE is explicitly instructed via the QMC measurement configuration container whether to conduct QMC in MBS broadcast mode or in MBS multicast mode.

To implement the above agreement and enable the UE to conduct QMC for the correct MBS mode cooperation with SA4 is needed.

============== content in R3-244789  ==============

It is obvious that RAN3 asked SA4 to introduce new information which is used to distinguish whether this QoE configuration is for MBS broadcast or MBS multicast in the QoE configuration.

Observation 1: RAN3 has requested SA4 to introduce new information to distinguish whether a QoE configuration is for MBS broadcast or MBS multicast within the QoE configuration.

SA4 responded RAN3 requirement and updated its specifications as shown below:

============== content in S4-242152 ==============

|  | @communicationServiceType | OD default= ”all” | When present, this attribute indicates in which communication service type the QoE collection is requested: - The value mbsMulticast refers to the MBS Multicast communication service per clause 21.1 of TS 38.300 [71]. - The value mbsBroadcast refers to the MBS Broadcast communication service per clause 21.1 of TS 38.300 [71]. - The value all refers to all communication service types. When absent, quality metrics collection is requested for all MBS modes. |
| --- | --- | --- | --- |

============== content in S4-242152 ==============

It is clear that SA4 introduced a new attribute “@communicationServiceType” which is used for broadcast or multicast QoE indication. Based on the table show above this attribute has different values: mbsMulticast, mbsBroadcast, all communication service types, or all MBS modes. OD here means Optional with Default Value. More specifically, if a receiver entity reads the QoE configuration container without this attribute, the receiver shall assume that this attribute is configured as “all”.

Observation 2: The new introduced attribute @communicationServiceType may have the following meanings: mbsMulticast, mbsBroadcast, all, or all MBS modes.

Observation 3: If a receiver entity reads the QoE configuration container without this attribute, the receiver shall assume that this attribute is configured as “all” .

  - Response to R3-250739[3]

Before we further discuss how to update either SA4 or RAN3 spec, companies in RAN3 shall firstly sync up our understanding for the following aspects about  @communicationServiceType. Here is the mapping table based on SA4 CR attached in LS.

Table 2.2-1 Mapping relationship between Attribute Value and Communication Service Type

| # | Attribute Value/presence | Communication Service Type |
| --- | --- | --- |
| 1 | Value: mbsMulticast | Multicast |
| 2 | Value: mbsBroadcast | Broadcast |
| 3 | OD value: all | The value all refers to all communication service types. |
| 4 | Presence state: absent | When absent, quality metrics collection is requested for all MBS modes. |

It is not hard for companies to agree the first two values’ meanings: for multicast QoE or broadcast QoE respectively. Based on the description in [3], it is clear that the rest two in the table(#3, #4)share the same meaning. The meaning for this situation is that: the QoE shall be triggered at UE side for both multicast and broadcast.

It is clear that the SA4 defined attribute has 3 values. One of these values is setup as default(e.g. the OD value “all”).

- In SA4 spec, the OD is short for “Optional with Default Value”. From our point of view, this means that if the value of this attribute is set as null or not present, then the UE shall assume this attribute is set as “all”. Which means, this QoE configuration can be triggered regardless of the communication service types.

- Based on the existing mechanism, from MBS point of view, the data transmission way can be classified into 3 categories: broadcast, multicast and unicast(legacy method which has already been introduced before Rel-17).

- Proposal 1: It is proposed that RAN3 clarifies the communication service type may include: broadcast, multicast, and unicast.

- Meanwhile, SA4 also defines that if this new attribute is not absent in the QoE configuration, based on the SA4 spec, UE shall trigger the QoE measurement for all MBS modes which is conflict with the OD value “all”. RAN3 may further make consensus on the understanding for the meaning of the attribute value “all” and the absent of this attribute. Moreover, to avoid any misunderstanding between the absent state and “all” for this attribute, RAN3 may further suggest SA4 to update their description. One possible updating is shown below:

Table 2.2-2 Modified table with RAN3 suggestion

| # | Attribute Value/presence | Communication Service Type |
| --- | --- | --- |
| 1 | Value: mbsMulticast | Multicast |
| 2 | Value: mbsBroadcast | Broadcast |
| 3 | OD value: all | The value all refers to both communication service types defined as above. |
| 4 | Presence state: absent | When absent, quality metrics collection is requested for all communication service types, including broadcast, multicast, and unicast. |

- Proposal 2: It is proposed that RAN3 clarifies whether the following situations share the same meaning:

- The value of @communicationServiceType is null or “all”: Refer to both communication service types defined in this attribute, e.g. mbsMulticast, mbsBroadcast.

- @communicationServiceType is absent: Refer to all communication service types, including at least broadcast, multicast, and unicast.

In addition, it is obvious that RAN3 defined an optional&enumerated type IE, MBS Communication Service Type, with two codepoints “broadcast” and “multicast” for MBS QoE. which means, it is impossible for a QoE configuration which can only trigger the  QoE measurement when UE is using broadcast and/or multicast service.

Observation 4: Based on RAN3 definition, it is no possible for one QoE configuration which can only trigger the  QoE measurement when UE is using any MBS modes.

It is possible for RAN3 to add a new codepoint “all” to the existing MBS Communication Service Type IE and to provide the MBS mode indication in the QoE report. However, there is no explicit benefit in perform such update across multiple specifications. Tthis functionality can also be achieved using the current mechanism without any updates to RAN3.

Considering that one QoE configuration can only map to one kind of communication service type, it is evident that the MCE can determine the communication service type of the received QoE report via the QoE reference ID in the report.

RAN3 shall send our understanding to SA4 with our understanding and ask SA4 to correct the description of the attribute @communicationServiceType.

Proposal 3: RAN3 replies SA4 LS with its understanding of the @communicationServiceType and asks SA4 to update the description of this attribute accordingly. Draft reply LS in annex can be considered as baseline.

2.3 Response to R3-250656[4]

In [4], it explains that RAN3 specs shall refer the definition in SA4 spec. From our point of view, we do not think this is necessary. First, as explained before, the SA4 defined new attribute @communicationServiceType has at least 3 different meanings. Meanwhile the RAN3 defined MBS Communication Service Type IE only has 2 codepoints. It can not map properly. Second, RAN3 firstly defined the MBS communication service type related IE and SA4 tries to follow RAN3’s definition after RAN3 has already defined this. We do not believe that RAN3 needs to wait until SA4 has introduced the corresponding parameter of MBS communication service type in accordance with RAN3's requirements and definitions before we can reference the new parameters defined by SA4 on the existing parameters.

Proposal 4: It is not necessary for RAN3 to add reference at MBS Communication Service Type IE for SA4’s attribute @communicationServiceType.

3 Conclusion

Observation 1: RAN3 has requested SA4 to introduce new information to distinguish whether a QoE configuration is for MBS broadcast or MBS multicast within the QoE configuration.

Observation 2: The new introduced attribute @communicationServiceType may have the following meanings: mbsMulticast, mbsBroadcast, all, or all MBS modes.

Observation 3: If a receiver entity reads the QoE configuration container without this attribute, the receiver shall assume that this attribute is configured as “all” .

- Proposal 1: It is proposed that RAN3 clarifies the communication service type may include: broadcast, multicast, and unicast.

- Proposal 2: It is proposed that RAN3 clarifies whether the following situations share the same meaning:

- The value of @communicationServiceType is null or “all”: Refer to both communication service types defined in this attribute, e.g. mbsMulticast, mbsBroadcast.

- @communicationServiceType is absent: Refer to all communication service types, including at least broadcast, multicast, and unicast.

Observation 4: Based on RAN3 definition, it is impossible for a QoE configuration which can only trigger the  QoE measurement when UE is using any MBS modes.

Proposal 3: RAN3 replies SA4 LS with its understanding of the @communicationServiceType and asks SA4 to update the description of this attribute accordingly. Draft reply LS in annex can be considered as baseline.

Proposal 4: It is not necessary for RAN3 to add reference at MBS Communication Service Type IE for SA4’s attribute @communicationServiceType.

4 Reference

[1]		R3-244789	LS on MBS Communication Service Type				RAN3

[2]		S4-242151	LS Reply on MBS Communication Service Type		SA4

[3]		R3-250739	MBS Communication Service Type for QMC			Ericsson

[4]		R3-250656	Reply LS on MBS Communication Service Type		Huawei

[5]		TS 26.247		Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)			3GPP

Annex Draft reply LS

3GPP TSG-RAN WG3 Meeting #127	R3-25xxxx

Athens, GR, 17-21 Feb, 2025

Title:	[DRAFT] Reply LS on MBS Communication Service Type

Response to:	S4-242151/R3-250026

Release:	Rel-18

Work Item:	NR_QoE_enh-Core

Source:	ZTE Corporation[shall be RAN3]

To:	SA4,

Cc:	RAN2, SA5

Contact person:	Liu Yansheng

Liu.yansheng@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

RAN3 thanks SA4 on reply LS on MBS Communication Service Type.

RAN3 has noticed that, according to the reply LS, SA4 has agreed to specify and include the Communication Service Type in the QMC configuration. This enables the UE to conduct QMC for the correct MBS mode(s). During RAN3 meeting, RAN3 discussed and identified that the attribute @communicationServiceType may lead to different actions by the UE in the following scenarios:

- The value of @communicationServiceType is null or “all”.

- @communicationServiceType is absent.

From RAN3’s perspective, the communication service type may include the following aspects: broadcast, multicast, and unicast. Therefore, “all communication service type” is not as same as “all MBS modes”. In addition, RAN3 defined an optional&enumerated type IE, MBS Communication Service Type, with two codepoints “broadcast” and “multicast”. It is no possible for one QoE configuration to trigger QoE measurement only when the UE is using any MBS modes.

Based on these considerations, RAN3 would like to kindly suggest that SA4 may consider not restricting the QoE measurement of the attribute @communicationServiceType in the absence state to be related solely to MBS modes and uniting the definition of the value “all” and absence of this attribute with the meaning referring to all communication service types.

2	Actions

To SA4

ACTION: 	RAN3 respectfully asks SA4 to take RAN3 understanding into account and provide feedback if necessary.

3	Dates of next TSG RAN WG 3 meetings

RAN3#127-bis			2025-04-07 - 2025-04-11			Wuhan, CN

RAN3#128				2025-05-19-  2025-05-23			Malta, MT