3GPP TSG-RAN WG3 Meeting #127	R3-250752

Athens, GR, 17-21 Feb, 2025

Title:	Response to R3-250190

Agenda Item:	31.1

Source:	Huawei

Document for:	Response

1	Introduction

This is a response paper to R3-250190. This document does not challenge the proposals in the original paper but intends to further clarify potential scenarios and also discuss comments received offline.

2	Discussion

The sequence starts with an initiating node, currently serving a UE, receives information about a neighbour cells served by a candidate UE. As part of ANR, the UE will report all served PLMNs in the candidate node. In addition to this, the initiating node has all information for the specific UE, e.g. handover restriction list etc. The initiating node will therefore be able to determine which node it is reasonable to setup an X2 interface towards and which PLMN that would be possible to use. In the following discussion we use similar definitions as in R3-250190.

2.1 	Re-use existing or define a new IE

It may be possible to re-use existing IEs. There are currently four IEs which includes the PLMN. But enabling the feature by re-using the existing IEs may impact already existing functionality and may lead to backward capability issues. A new IE enables the possibility to use criticality (if desired) and also to define abnormal conditions (if needed). Hence, we see benefits of defining a new IE. This could also be useful in failure scenarios (see below).

2.2	Non-overlapping PLMNs

In most cases (MOCN), the PLMN that is currently serving the UE will also be supported in a valid candidate cell. Hence it would not matter whether the PLMN we provide is the PLMN that is currently serving the UE, or the PLMN that will be serving the UE after the potential mobility event. It is however clear htat the PLMN IDs associated with the node ID cannot be used since there are rules on how to assign them.

In some cases (GWCN in LTE) however, we may have non-overlapping PLMNs configured in two nodes, but where inter-PLMN handovers are allowed between these two PLMN and hence an X2 may be beneficial. In this case, we would have to choose whether the IE indicating the PLMN refers to the PLMN used in the initiating node, or the PLMN to be used in the candidate node. See a detailed example in the annex below.

We believe it would simplify for the configuration in case the PLMN that is used to indicate the TNL is indicated form the candidate’s point of view. Otherwise, the candidate would have to be configured for mapping from PLMN to TNL info not only for the PLMN supported by this node, but for all PLMNs that would potentially send the TNL request.

2.3	Behaviour in failure cases

One important discussion to have would be to discuss the behaviour in case the candidate node is not able to provide a TNL matching the desired PLMN. What is the behaviour in this case?

One solution would be to fail the procedure, but since the exchange consists of four different class 2 procedures, the specification of this failure reporting may be tricky.

Another solution to this is to allow the procedure to continue but to also indicate to the initiating node which PLMN was used when selecting the TNL in the candidate node. This would enable the initiating node to detect the mismatch and try a different approach (if possible).

If we use a new IE, the presence/absence of this indicator could also serve as information for the initiating node to understand whether the candidate support the new TNL mapping functionality.

3. Summary

We propose to discuss the problem and solution in the following steps:

- Confirm the scenario – is the support for this feature needed in specifications?

- Decide whether a new or a legacy IE should be used

- In the initiating request message, decide whether to indicate the PLMN used in initiating node or in the candidate for selecting the TNL

- Decide on behaviour in failure scenarios

Annex- GWCN in LTE

A typical scenario in GWCN which is supported in LTE is depicted in the following Figure.

Initiating eNB A’s primary PLMN is p1, and the network is shared to p3.

Candidate eNB B’s primary PLMN is p2, and the network is shared to p4.

p3 and p4 have shared MME, called GWCN in LTE.

A UE in a cell of initiating eNB A is serving by p3. And it reports an unknown CGI with PLMN p2 and p4 by ANR. The initiating eNB A triggers a X2 TNL address discovery procedure towards the candidate eNB B to setup X2 and handover the UE to the candidate eNB B.  As the initiating eNB A determines that p4 is the EPLMN of p3 based on the UE handover restriction list.

p4 should be indicated to the candidate eNB B in the new IE. Which means that the new IE is to indicate the PLMN used by the candidate eNB B.

Figure : PLMN selection in X2 TNL address discovery in GWCN