3GPP TSG-RAN WG3 Meeting #127	R3-250795

Athens, GR, 17-21 Feb, 2025

Agenda Item:	16.3

Source:	Qualcomm Incorporated (moderator)

Title:	Summary of CB: # AIoT2_LocationReport

Document for:	Discussion and decision

# Introduction

This is the SoD for the following CB:

CB: # AIoT2_LocationReport

- Check the open issues above

- Capture agreements and provide TP is needed

(moderator - QC)

Summary of offline disc R3-250795

The following was agreed online:

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF in Command Report?

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF via legacy location report procedures?

# For the Chair’s Notes

Change the previous agreement to clarify it better:

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

to

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

AIoT RAN node sends the Inventory Report to AIoTF aggregated per Reader

“Global gNB ID + Reader index” is used to uniquely identify the Reader globally.

FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report

Remove the below FFS from Chair’s Notes:

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF via legacy location report procedures

# Discussion

## Reader ID in Inventory Report

The following was agreed in the online session:

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

There was a discussion in the online session whether the Reader ID is sent mandatorily in Inventory Report or can be optional.

If Reader ID is optionally included in Inventory Report, it can be done in the following ways:

- Alt 1: Up to AIoT RAN node implementation whether to include Reader ID in Inventory Report

- Alt 2: Upon explicit request in Inventory Request to report “Reader ID” in Inventory Report

- Alt 3: Only in some cases e.g., only for “Inventory and Command” and not for “Inventory-only”

Q1: Is the Reader ID mandatory or optional in Inventory Report? If optional, which of the alternatives (Alt 1/2/3) do you prefer?

Companies are invited to express their preference on the above options:

| Company | Mandatory or optional (Alt 1/2/3) | Comment |
| --- | --- | --- |

Change the previous agreement:

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

to

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

## How is the Reader ID reported in Inventory Report?

Following 2 options were proposed in R3-250201:

- There are two ways to report Device location at reader ID granularity via Inventory Report:

- Option 1:	report a list of {A-IoT NAS PDU (device identifier) + Reader ID}

- A-IoT NAS PDU 1 (device Identifier 1) + Reader ID 1

- A-IoT NAS PDU 2 (device Identifier 2) + Reader ID 1

- A-IoT NAS PDU 3 (device Identifier 3) + Reader ID 2

- A-IoT NAS PDU 4 (device Identifier 4) + Reader ID 2

- Option 2:	report a list of {Reader ID + List of A-IoT NAS PDU (device identifier)}

- Reader ID 1

  - A-IoT NAS PDU 1 (device Identifier 1)

  - A-IoT NAS PDU 2 (device Identifier 2)

- Reader ID 2

  - A-IoT NAS PDU 3 (device Identifier 3)

  - A-IoT NAS PDU 4 (device Identifier 4)

Q4: Which option do you prefer for sending the Reader ID in Inventory Report?

- Option 1 (per device): A list of {Reader ID + A-IoT NAS PDU (with device ID)}

- Option 2 (aggregated per Reader): A list of {Reader ID + List of A-IoT NAS PDU (with device ID)}

| Company | Option 1 or 2 | Comment |
| --- | --- | --- |

Go with option 2.

AIoT RAN node sends the Inventory Report to AIoTF aggregated per Reader

## Reader ID in Command Report

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF in Command Report?

There were split views on this. So, let’s check this:

Q2: Is the Reader ID also sent in Command Report?

Companies are invited to express their preference on the above options:

| Company | Yes/No | Comment |
| --- | --- | --- |
| Qualcomm | Yes | We think that the Reader ID in Command Report can be useful in the following cases: - If Reader ID is optionally included in Inventory Report - A-IoT device is in the coverage of more than one Reader and the Inventory Report includes all Reader ID(s) in coverage of the A-IoT device. Command may not be addressed to all these Readers and the Reader ID in Command Report confirms whether the A-IoT device is still in coverage (e.g., A-IoT devices can move) - This can be useful even for “Inventory and Command” use case if there are 2 Commands back-to-back after an Inventory. Command Report 1 can include a Reader ID which can be useful for Command 2 |

## Encoding of Reader ID

There were several options proposed in the papers on how to encode “Reader ID”

- Option 1: Reuse TRP ID

- Option 2: Global Reader ID (e.g., 1..255)

- Option 3: AIoT RAN node ID + index of Reader (e.g., 1..64)

Q3: Which option do you prefer for encoding the Reader ID?

| Company | Option 1/2/3 | Comment |
| --- | --- | --- |

“Global gNB ID + Reader index” is used to uniquely identify the Reader globally.

FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report

## Reader ID in legacy location Report procedures

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF via legacy location report procedures?

It is not clear to the moderator if there was any intention to send the Reader ID in legacy ULI procedures. It was the moderator’s view that all companies want to include the Reader ID in AIoT procedures (e.g., Inventory/Command Report) and not in ULI.

Q5. Can we agree that the Reader ID is not included in legacy ULI procedures?

| Company | Yes or No | Comment |
| --- | --- | --- |

## Remove the FFS from Chair’s Notes:

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF via legacy location report procedures

## Reader location

There were some proposals to include the Reader location as well in Inventory/Command Report, besides the Reader ID.

This was also discussed in the agenda item 16.2 and the following was captured in Chair’s Notes:

AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list.

The location of reader is FFS.

FFS on whether signaling is needed, further discussion on the detail information and necessity.

Q6. Do you agree that AIoTF can obtain the location of reader? If yes, do you prefer OAM or signaling based or both?

| Company | Yes/No + OAM/signaling | Comment |
| --- | --- | --- |

Already covered in the other CB

## Dedicated location report procedure

In R3-250097, the following was proposed:

The AIoTF might want to obtain the A-IoT device location independently, not necessarily along with inventory or command. Further, the AIoTF might want to obtain location with a different periodicity compared to the periodicity with which Inventory/Command Reports are sent to AIoTF. We therefore propose to support an independent procedure (“Location Request/Response) for AIoTF to request location from Reader.

Proposal 3: AIoTF should be able to request A-IoT RAN node to provide the location of an AIoT device and obtain the requested location in a dedicated message (other than Inventory and Command) e.g., via Location Request/Response

Q7. Do you agree to support a dedicated location report procedure to request/report A-IoT device location (i.e., Reader ID in this release) besides Inventory/Command?

| Company | Yes/No | Comment |
| --- | --- | --- |

No consensus. Postpone to next meeting.

# Draft TP for TS 38.413

#### 9.3.x.4	Inventory Report Transfer --- Option 1

This IE is transparent to the AMF, if via AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Device ID to Report List |  | 1 |  |  |
| >Device ID to Report Item |  | 1..<maxnoofDeviceIDstoReport> |  |  |
| >>A-IoT NAS PDU | M |  | OCTET STRING |  |
| >>Reader Identity | O |  | 9.3.3.zz |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDeviceIDstoReport | Maximum no. of Device IDs to report. Value is FFS. |

#### 9.3.x.4	Inventory Report Transfer --- Option 2

This IE is transparent to the AMF, if via AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Per Reader Report List |  | 1 |  |  |
| >Per Reader Report Item |  | 1..<maxnoofReaderstoReport> |  |  |
| >>Reader ID | M |  | 9.3.3.zz |  |
| >>Device ID to Report List |  | 1 |  |  |
| >>>Device ID to Report Item |  | 1..<maxnoofDeviceIDstoReport> |  |  |
| >>>>A-IoT NAS PDU | M |  | OCTET STRING |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDeviceIDstoReport | Maximum no. of Device IDs to report. Value is FFS. |
| maxnoofReaderstoReport | Maximum no. of Readers to report. Value is FFS. |