3GPP TSG-RAN WG3 Meeting #127	R3-250818

Athens, Greece, 17th – 21st February 2025

Agenda Item:	16.2

Source:	Ericsson, Huawei, Nokia, ZTE, CMCC, Xiaomi

Title:	(TP to TS 38.300 BL CR) Support of Inventory and command

Document for:	Discussions & Approval

# 1	Introduction

This document intends to provide stage 2 text for a potential BL CR for TS 38.300 and is submitted for agreement at RAN3#127.

NOTE:	Additions and changes as compared to R3-250444 are marked by using a different “user”, avoiding “changes on changes”.

# 3	Text Proposal for TS 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 16.x.3	Inventory procedure

Figure 16.x.3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Editor’s Note 1:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.x.3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request may include information concerning one or several devices or a goup of A-IoT devices or all of A-IoT devices.

Editor’s Note 2:	Specification of further parameterisation of the Inventory Request message (e.g., assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message.

NOTE 1:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Reports may be sent to the A-IoT CN node.

NOTE 2:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices,

### 16.x.4	Command procedure

Figure 16.x.4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Editor’s Note 1:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.x.4-1: Command procedure

1.	Prior to the Command procedure, the Inventory procedure is performed for the concerned devices.

Editor’s Note 2:	Further details on the relation and correlation between the Inventory procedure and the Command procedure needs further discussion may need input from SA2/SA3/RAN2.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB.

3.	The gNB allocate and co-ordinate the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The  gNB replies to the Command Request with the Command Response message.

NOTE 1:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>