3GPP TSG-RAN WG3 Meeting #127	R3- 250845

Athens, Greece, February 17th – 21st 2025

Agenda Item:	17.3

Source:	Ericsson, Deutsche Telekom, China Unicom, CMCC, Lenovo, NEC, Samsung, CATT, Huawei, Rakuten

Title:	Xn impact of On-demand SIB1 for UEs in idle/inactive mode

Document for:	Other

# Introduction

This TP implements the agreements (as in below) from RAN3#127 meeting.

# Conclusion

Proposal 1	It is agreed that “after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts SIB at the first opportunity.

Proposal 2	It is agreed from the signalling point of view, in UL WUS Configuration Request message (naming FFS) sending from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIB as in Proposal 1.

Note: How to implement in the specification the option “start with UL WUS Configuration” is to be further refined.

Text Proposal for BL CR for TS 38.423

*************** Start of TP ***************

Editor’s Note: The procedure, message and the IE names are FFS

### 8.x.x	UL WUS Configuration Provision (FFS)

#### 8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2 “and request NG-RAN node2 to transmit UL WUS configuration information (FFS)”.

“The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to discontinue transmission of UL WUS configuration.  (FFS)”

The procedure uses non UE-associated signaling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

If the “start with UL WUS Configuration” (details on this codepoint/choice of “start with UL WUS Configuration” are FFS) is included in the UL WUS CONFIGURATION PROVISION REQUEST (Naming FFS) message, the Cell A gNB shall, if supported, broadcast UL WUS Configuration (Naming FFS) in SIBx (SIB naming FFS) and reply with the UL WUS CONFIGURATION PROVISION RESPONSE (Naming FFS) message. The Cell A gNB stores the UL WUS configuration information (Naming FFS).

“Cell A stores the UL WUS configuration information after it has been requested to be discontinued (FFS)”

“Cell A removes the UL WUS configuration information after it has been requested to be stopped (FFS)”

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the Cell A gNB cannot broadcast UL WUS Configuration in SIBx (Naming FFS), or a failure occurs during UL WUS Configuration Provision (Naming FFS), the Cell A gNB shall send UL WUS CONFIGURATION PROVISION FAILURE (Naming FFS) message. The message shall contain the Cause IE with an appropriate value.

#### 8.x.x.4	Abnormal Conditions

Void.

### 8.x.y	UL WUS Configuration Transmission Status Update (FFS)

#### 8.x.y.1	General

The procedure uses non UE-associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update, successful operation

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

*************** End of TP ***************