3GPP TSG-RAN WG3 Meeting #127	R3-250872

# Athens, Greece, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1422 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Data Collection Reporting procedure’s interaction with Handover Request procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Jio Platforms (JPL), InterDigital, FiberCop, Huawei, Lenovo, ZTE, Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN-Core |  |  |  |  |  | Date: |  |  | 2025-01-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Rel.18 work item on AI/ML for NG-RAN specified data collection enhancements and signalling support for AI/ML-based use cases. Among the specified enhancements were two new procedures- Data Collection Reporting Initiation (class 1) and Data Collection Reporting (class 2) procedures. The Data Collection Reporting Initiation procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN. After successful execution of the Data Collection Reporting Initiation procedure, the requested metrics are reported to the requesting NG-RAN node over the Data Collection Reporting procedure. For the Measured UE Trajectory and UE performance after handover, the measurement ID of the Data Collection Reporting Initiation procedure is included in the Handover Request message as an implicit indication for reporting of metrics as requested by the corresponding Data Collection Reporting Initiation procedure to the requesting node. The measurement of the requested metrics commences after a successful handover and is reported to the requesting NG-RAN node after at least one of the specified exit conditions is satisfied. The collected measurements are signalled to the requesting node over the following transmission of Data Collection Update message, after at least one of the exit conditions have been satisfied, depending on the periodicity configured by the original Data Collection Reporting Initiation procedure. The text describing the above in TS 38.423 is as follows: “If the Data Collection ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the source NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the target NG-RAN node.”. The statement “… the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover, via the Data Collection Reporting procedure …” may be interpreted as the target NG-RAN node reporting the measurements to the source NG-RAN node immediately after a successful handover via the Data Collection Reporting procedure. While the intention is to specify that the reporting happens via the Data Collection Reporting procedure after a successful handover and after the exit conditions have been triggered and at the specified reporting periodicity of the Data Collection Reporting procedure. The above-described conditions for terminating the feedback collection and signalling the collected feedback, i.e., after the exit conditions are triggered and at the next transmission of the Data Collection Update message, are also described in Clause 8.4.13.2 in TS 38.423, however, this description does not respect the agreement taken in RAN3-119bis stating the following: - UE performance feedback can be reported through one-time reporting or periodic reporting. Therefore, the text in Clause 8.4.13.2 in TS 38.423 prevents the option of periodic reporting of the UE Performance feedback. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The clause describing the interaction between the Handover Request and the Data Collection Reporting and the Data Collection Reporting Initiation procedures in 8.2.1.2 in TS 38.423 is corrected to mention that UE performance reporting as part of Data Collection Reporting is triggered according to clause 8.4.13.2. - The clause describing the Data Collection Reporting Initiation procedure in 8.4.13.2 in TS 38.423 is corrected to clarify that the result of the UE performance collection may be reported periodically during the Collection Time Duration for UE Performance via DATA COLLECTION UPDATE messages and, after collection is terminated, it shall be reported at the next available DATA COLLECTION UPDATE message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misinterpretation of the clause 8.2.1.2 may lead to wrongful signalling of Data Collection Update messages immediately after a successful handover execution, without the triggering of any exit condition. The text in Clause 8.4.13.2 prevents from periodic reporting of the UE Performance metrics, which was agreed by RAN3 during Rel18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 8.4.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGES START

## 8.2	Basic mobility procedures

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

---

Unmodified text omitted

---

Interaction with the Data Collection Reporting and the Data Collection Reporting Initiation procedures:

If the Data Collection ID IE is contained in the HANDOVER REQUEST message , the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover via the Data Collection Reporting procedure and according to clause 8.4.13.2, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the source NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the target NG-RAN node,.

---

Unmodified text omitted

---

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

---

Unmodified text omitted

---

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

The result of the UE performance collection may be reported periodically during the Collection Time Duration for UE Performance via DATA COLLECTION UPDATE messages and, after collection is terminated, it shall be reported at the next available DATA COLLECTION UPDATE message.

CHANGES END