3GPP TSG-RAN WG3 Meeting #128	R3-253131

St. Julian’s, Malta, May 19th – 23rd 2025

Agenda Item:	12.2

Source:	Ericsson, Jio Platforms

Title:	(TP for WAB BL CR for TS 38.401): Functional Aspects of WAB-Nodes

Document for:	Agreement

# Introduction

In this paper we continue the normative discussion on functional aspects of WAB-nodes.

A TP for WAB BL CR for TS 38.423 is provided in Annex 1.

# Discussion

We discuss the following functional aspects of WAB-nodes:

- Additional ULI.

- Xn connection management.

- Multihop topology prevention for WAB.

## Additional ULI

The RAN3#127-bis meeting notes capture the following:

RAN3 to agree the WAB-gNB reports the additional ULI to the network within the NG Setup and RAN Configuration Update procedures.

It is FFS whether the additional ULI shall be removed from NG UE associated procedures

In our understanding, AULI was introduced in UE-associated (UA) NGAP signalling and non-UE-associated (NUA) NGAP signalling for different reasons. Namely, AULI was introduced in UA NGAP signalling to support location-based services for UEs served by WAB-nodes. On the other hand, the reason for introducing the ULI in NUA NGAP signalling was to properly support the PWS message delivery. As explained in R3-251583, if the AULI is only delivered by means of UA NGAP signalling, then the AMF has up-to-date AULI only if there is at least one UE in RRC_CONNECTED state served by the WAB-node. Consequently, to remove the dependency of accurate PWS delivery on whether there are RRC_CONNECTED UEs in the WAB-gNB cell, RAN3 decided to introduce AULI in NGAP NUA signalling.

Observation 1: Additional ULI was introduced in UE-associated NGAP signalling and non-UE-associated NGAP signalling for different reasons.

With respect to the FFS, we think that removing AULI from NUA NGAP signalling is not preferred because it would impact the AMF logic. Roughly speaking, an AMF maintains UE contexts separately from “gNB contexts”. If AULI is kept in both NUA and UA NGAP signalling, an AMF receiving AULI in a UA message is directly able to associate the AULI with a UE, based on the NGAP UE IE received in the UA message. On the other hand, if AULI is removed from UA NGAP signalling, and delivered only by means of NUA NGAP signalling (which does not contain any NGAP UE IDs), to associate the AULI with relevant UEs, the AMF needs to implement an extra logic to search the gNB context and identify the relevant UEs (i.e., the UEs served by the sending WAB-gNB).

Based on the above, we propose keeping the Additional ULI IE in NGAP UA messages.

Proposal 1: Keep the Additional ULI IE in NGAP UE-associated messages.

## Xn connection management

Another RAN3#127-bis agreement states:

Include a WAB-MT Identifier in the XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK e.g. for colocation discovery for resource multiplexing or for WAB node indication.

The exact format of the WAB-MT ID remains to be discussed. The simplest option seems to be that the WAB-MT ID is the C-RNTI of the WAB-MT assigned by the BH-gNB, accompanied with the ID of the BH-gNB’s cell serving the WAB-MT.

Proposal 2-1: The “WAB-MT ID” sent from the WAB-gNB to the BH-gNB is the WAB-MT’s C-RNTI assigned by the BH-gNB.

Proposal 2-2: The WAB-gNB sends to the BH-gNB the ID of the BH-gNB’s cell serving the WAB-MT.

The underlying assumption with respect to prevention of Xn setup between WAB-gNBs is that WAB-nodes will be moving constantly and may be in each other’s vicinity only briefly. In such a scenario, setting up Xn connection(s) between WAB-nodes is indeed not meaningful. However, we can foresee the scenarios in which it may be necessary to set up Xn interface between WAB-gNBs. Some example scenarios are:

- Multiple WAB-nodes moving together, e.g., due to being deployed onboard the same vessel.

- A nomadic WAB scenario, where multiple static WAB nodes are temporarily deployed to provide coverage in a disaster-hit area.

Based on the above, we think that it should be possible to establish Xn interface connections between WAB-gNBs, i.e., that the solution for prevention of inter-WAB Xn interface setup should be optional.

Proposal 2-3: Xn connection between WAB-gNBs can be established.

With respect to Xn setup, the specifications should not mandate whether a WAB-gNB should establish Xn with all or with some of its surrounding gNBs. This should rather be an implementation choice. In that sense, in a scenario where the WAB-MT is handed over to a target BH-gNB that had no Xn connection with the WAB-gNB prior to the HO, service interruption may occur because resource coordination cannot be done without an Xn connection between the WAB-gNB and the BH-gNB, that has not yet been set up. Therefore, service interruption could be reduced by setting up the Xn connection between the WAB-gNB and the target BH-gNB as early as possible. One way to speed up Xn interface setup towards the target BH-gNB is to notify the WAB-gNB as soon as possible about the target (BH-)gNB for the WAB-MT HO.

In case the WAB-gNB already has an Xn connection towards the target BH-gNB (e.g., because the target BH-gNB is a neighbour of the source BH-gNB), the WAB-gNB needs to know which of its Xn neighbours is the new BH-gNB, to be able indicate to the target BH-gNB that it is co-located with the incoming WAB-MT (e.g., for enabling the resource multiplexing for the WAB-node). This also speaks in favour of notifying the WAB-gNB about the target BH-gNB for the WAB-MT HO.

Proposal 2-4: The WAB-gNB should be notified about the target BH-gNB before the WAB-MT HO.

According to the WID, the BH link can be a terrestrial or a non-terrestrial link. This is captured in the WID as follows:

NOTE 5: Backhaul link for WAB-MT can be TN or NTN.

In our view, the WAB-gNB should be aware of whether the BH link is a terrestrial or a non-terrestrial one. This information could be of interest for the WAB-gNB’s PLMN, e.g., for properly setting the QoS of the PDU sessions of the served UEs.

In addition, in resource multiplexing, the WAB-gNB can benefit from this knowledge in case it is the one determining the HNA configuration. Namely, whether the BH link is terrestrial or non-terrestrial says something about the link quality/data rate and possible delays that can be expected. For example, if BH UL data rate is expected to be half the (average) served UE UL data rate, the WAB-gNB could indicate for UL resources a NA twice as much as H (i.e., the WAB-gNB indicates to BH-gNB that it will use on access link half of the number of slots/symbols that could be used on BH link).

Whether the BH link is terrestrial or non-terrestrial also affects the propagation delay, which, in turn, causes mis-alignment between UL and DL frames and affects the way HNA should be configured. Namely, when switching between UL and DL, it could be that a “H” DL slot overlaps with an “NA” UL slot.

Proposal 2-5: The WAB-gNB should be aware of whether the BH link for the WAB-MT is a terrestrial or a non-terrestrial one.

## Multihop topology prevention for WAB

At the RAN3#127-bis meeting, RAN3 acknowledged that Solution 1 for multihop topology prevention is implementation-specific and not subject to specifications changes. In addition, the RAN3#127-bis meeting notes capture the following FFS:

It is FFS whether Solution 3, requiring impacts to SIBs, can be agreed. If there is consensus in RAN3 on an agreement for solution 3, RAN2 involvement is needed.

In our view, Solution 1 offers sufficiently many options of multihop prevention, and no further discussion is necessary.

Proposal 3: No further discussion on WAB multihop topology prevention in Rel-19.

# Conclusion

This paper discusses the functional aspects of WAB-node. The following is observed and proposed:

Proposal 1: Keep the Additional ULI IE in NGAP UE-associated messages.

Proposal 2-1: The “WAB-MT ID” sent from the WAB-gNB to the BH-gNB is the WAB-MT’s C-RNTI assigned by the BH-gNB.

Proposal 2-2: The WAB-gNB sends to the BH-gNB the ID of the BH-gNB’s cell serving the WAB-MT.

Proposal 2-3: Xn connection between WAB-gNBs can be established.

Proposal 2-4: The WAB-gNB should be notified about the target BH-gNB before the WAB-MT HO.

Proposal 2-5: The WAB-gNB should be aware of whether the BH link for the WAB-MT is a terrestrial or a non-terrestrial one.

Proposal 3: No further discussion on WAB multihop topology prevention in Rel-19.

# Annex 1: TP for WAB BL CR for TS 38.401

-------------------------------------------Start of changes-------------------------------------------

# X	Wireless Access Backhaul

## X.1	WAB-node integration procedure

Figure X.1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information needed to establish NG connections towards one or more AMF(s), and the WAB-gNB is service-authorized by the SeGW or by the OAM.

Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step follows legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB can establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s). If the WAB-gNB includes a WAB-MT identifier in the signalling for Xn connection setup, the BH-RAN-node and/or other NG-RAN node(s) can understand that the peer node is the WAB-gNB of the WAB-node.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

## X.4 	WAB-node authorization

WAB-node authorization includes authorization of the WAB-MT’s backhaul support, and service authorization of the WAB-gNB, as defined in TS 23.501 [3].

Authorization of the WAB-MT is different from the service authorization of the WAB-gNB.

Authorization of the WAB-MT is defined in TS 23.501 [3].

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs.

When the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”:

- The WAB-gNB attempts to handover, or releases, the UEs.

- The NG and Xn connections of the WAB-gNB are removed.

- The PDU sessions of the WAB-MT used for backhauling may be released.

It is expected that WAB-MT’s PLMN/SNPN ensures that backhaul PDU sessions of the WAB-MT are maintained long enough for the WAB-gNB to perform UE handover/release and the removal of NG and Xn connections, as specified in TS 23.501 [3].

Once the WAB-node’s service authorization status changes from “not authorized” to “authorized”, the WAB-gNB can use the WAB-MT’s PDU sessions to establish connections to the SeGW, the OAM, the AMF(s) and, optionally, towards the BH-gNB and neighbouring NG-RAN nodes.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### X.7.2	WAB-gNB mobility

#### X.7.2.1 	WAB-gNB mobility with change of UE’s AMF(s)

Due to the WAB-gNB’s mobility, the UEs moving together with the WAB-gNB may have to change AMF.

For the AMF change, a new logical WAB-gNB is instantiated, which establishes NG connection(s) towards one or more new AMF(s). The new logical WAB-gNB may obtain from the OAM the configuration parameters needed to establish the connection(s) to the UE’s new AMF(s), based on, e.g., WAB-node’s location.

The new logical WAB-gNB may activate one or more cell(s) with new TAC, cell ID, and PCI, which depend on the WAB-node’s current location.

The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over from a cell served by the old logical WAB-gNB to a cell served by the new logical WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502 [7], after which the UE’s AMF is changed to the new AMF.

- A UE in RRC_IDLE or RRC_INACTIVE state camping on a cell served by the old logical WAB-gNB reselects a cell served by the new logical WAB-gNB. The reselection may be triggered by the removal of the old logical WAB-gNB’s cells from service. After cell reselection, the UE performs a Mobility Registration Update as defined in TS 23.502 [7], which is triggered by the new TAC broadcasted by the new logical WAB-gNB’s cell. During this Mobility Registration Update, the UE’s AMF is changed to the new AMF.

After all the UEs in RRC_CONNECTED state are handed over, the NG connection(s) between the WAB-node and the old AMF(s) are removed via NG Removal procedure and the old logical WAB-gNB’s cell(s) are removed from service.

## X.8 	Coordination of WAB-gNB and WAB-MT radio resources

Coordination of access link and backhaul link radio resources is needed when the access link and the backhaul link of a WAB-node are operated in the same band (i.e., referred to as the in-band operation). To facilitate resource coordination, the BH-gNB discovers the co-location of the WAB-MT and the WAB-gNB based on a WAB-MT’s ID which the WAB-gNB includes in an XnAP message it sends to the BH-gNB. The resource coordination procedure is conducted between the WAB-gNB and the BH-gNB serving the WAB-gNB’s co-located WAB-MT.

NOTE: 	In-band WAB operation is only considered in scenarios where a WAB-gNB and its co-located WAB-MT connect to the same PLMN. When a WAB-gNB and its co-located WAB-MT connect to different PLMNs, it is assumed that the access link and the backhaul link of a WAB-node are operated in different bands.

## X.9 	Xn connection management

A WAB-gNB can establish an Xn connection with the BH-gNB serving the co-located WAB-MT, and with the neighbouring gNBs, based on the OAM configuration. During setup or update of its Xn connections, the WAB-gNB can indicate an ID of the WAB-MT, thus making the BH-gNB aware of the co-location of the WAB-MT and the WAB-gNB.

Xn connections can also be established between WAB-gNBs.

## X.10 	Avoidance of multihop WAB topology

Avoidance of multihop WAB topology is optional, and it refers to the mechanisms for preventing WAB-MTs from accessing WAB-gNB cells. This includes prevention upon WAB-MT handover and upon initial access.

Upon Xn-based WAB-MT handover, the target WAB-gNB can optionally reject a Handover Preparation procedure that includes PDU session(s) mapped to the WAB-dedicated S-NSSAI(s) used for backhauling.

It is optional and up to implementation how to prevent multihop WAB topology upon initial access. For example, a WAB-gNB can be configured by the OAM to use dedicated cell-level parameters, such dedicated frequencies, NR CGIs, PCIs, TACs, CAGs and potentially other legacy OTA parameters, to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB’s cells.

-------------------------------------------End of changes-------------------------------------------

# Annex 2: [DRAFT] Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control)

3GPP TSG-RAN WG3 Meeting #128	R3-25xxxx

St. Julian’s, Malta, May 19th – 23rd 2025

Title:               Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access)

Response to:	R3-250071/S2-2501324

Release:	Rel-19

Work Item:	FS_VMR_Ph2, NR_WAB_5GFemto-Core

Source:	RAN3

To:	SA2

Cc:	RAN2

Contact Person:

Name:		Filip Barac

E-mail Address:	filip.barac@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:  -

1. Overall Description:

RAN3 thanks SA2 for the LS reply on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI) in S2-2501324.

For access control upon mobility, RAN3 captured the following in the WAB BL CR for TS 38.401:

Upon Xn-based WAB-MT handover, the target WAB-gNB can optionally reject a Handover Preparation procedure that includes PDU session(s) mapped to the WAB-dedicated S-NSSAI(s) used for backhauling.

For access control upon initial access, RAN3 captured the following in the RAN3#127-bis meeting:

Solution 1 is implementation specific and it is not subject to specifications changes.

As a reminder, the definition of Solution 1 is:

For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells.

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meeting:

RAN3#129		Aug 25th – 29th 2025		Bengaluru, India