3GPP TSG-RAN3 Meeting #128	R3-253140

Malta, MT, May 19th – 23rd, 2025

Agenda Item:	16.2

Source: 	Qualcomm Incorporated

Title: 	NG signaling to support Inventory and Command procedures for A-IoT

Document for:	Discussion/Decision

# Background

In this paper, we provide our views on how to support Inventory and Command procedures for Topology 1 of A-IoT.

# Discussion

## Whether to include the Correlation ID outside the Transfer IEs

The following agreements and working assumptions happened in RAN3#127bis:

Include the AIOTF Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages

Introduce a “Correlation ID” in the respective NGAP inventory at least inside the transfer containers

WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.

In case of indirect connectivity (i.e., when gNB is connected to AIOTF via the AMF), the AMF’s only responsibility is to send the Inventory/Command Requests received from the AIOTF to the gNB (based on the gNB ID included outside the Inventory/Command Request Transfer IEs) and to route the responses for Inventory/Command to the AIOTF based on the AIOTF ID included outside the Inventory/Commad Response Transfer IEs.

We don’t think AMF needs to know the Correlation ID or identify the A-IoT sessions (and no need to know whether it is a new session or repeat of an old session).

AMF just needs to route the Inventory related Transfer IEs and Command related Transfer IEs to the appropriate gNB (based on the gNB ID) for request messsages, and to the appropriate AIOTF (based on the AIOTF ID) for response messages

There is no need for AMF to identify the A-IoT session and therefore no need to know the Correlation ID (even in case of parallel Inventory/Command sessions).

Therefore, the following is proposed:

There is no need to include the Correlation ID outside the Inventory related Transfer IEs and Command Transfer IEs

Correlation ID is included only inside the Command related Transfer IEs

## Device Association

The following open issues were identified in RAN3#127bis:

Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.

Introduction of AIOTF A-IoT Device NGAP ID is FFS.

In case AIOTF decides to select readers served by multiple gNBs for Inventory and then subsequently for its follow up Command, the AIOTF may need to assign an AIOTF A-IoT Device NGAP ID to uniquely identify a device per session per gNB/Reader. This is illustrated in the figure below:

Upon receiving the RAN A-IoT Device NGAP ID in Inventory Report, AIOTF can allocate an AIOTF A-IoT Device NGAP ID to uniquely identify the A-IoT device per session per gNB/Reader.

RAN A-IoT Device NGAP ID is included only inside the Inventory Report Transfer IE

It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

Similar to our view in section 2.1, we think that there is no need for AMF to identify parallel Command Request procedures and hence there is no need of including any Correlation ID or Transaction ID or the device association ID(s) outside the Command Transfer IEs. Hence this is proposed:

RAN A-IoT Device NGAP ID and AIOTF A-IoT Device NGAP ID are included only within the Command Transfer IEs (i.e., there is no need to include these IDs or any “Transaction” ID outside the Command Transfer IEs).

## Expected D2R Message Size in Inventory Request

The following FFS was captured in RAN3#127bis:

It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

RAN2 sent this LS to RAN3 and SA2

RAN2 discussed segmentation of D2R messages and made the following assumption on inventory response message size.

Agreements on segmentation

1. For inventory response, RAN2 assumes that segmentation is not applied.  RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size.

RAN2 would like to inform SA2 and RAN3 that RAN2 assumes that the inventory response size can always be provided by the core network to the reader

We propose to confirm RAN2 assumption and support the signalling of “Expected Inventory Response Size” in NG Inventory Request.

Include “Expected Inventory Response size” as part of Inventory Assistance Information in Inventory Request

## Command Type

It was already agreed in the last meeting to include “Command Assistance Information” within Command Request Transfer in NG Command Request message. The contents of this is still FFS.

Similar to Inventory Assistance Information (as mentioned in section 2.3), we think Command Assistance Information can include the “Expected Command Response size”.

With the knowledge of Expected Command Response size, the gNB serving the BS Reader can do appropriate resource allocation. For example, gNB may schedule more resources (or a larger TB) in case of Read Response and smaller number of resources (or a smaller TB) for Write Response (which is just an ACK or NAK).

We therefore think that there is no need for AIOTF to explicitly indicate the Command Type (read, write or disable) in the Command Request and resource allocation can be done based on Expected Command Response Size. Also, the Command is an end-to-end protected information (via A-IoT NAS) and there is no need to expose this to the gNB.

Include “Expected Command Response size” as part of Command Assistance Information in Command Request.

With the knowledge of Expected Command Response size, the gNB serving the BS Reader can implictly know the Command Type and do appropriate resource allocation for Command Responses (e.g., larger TB for Read Response or smaller TB for Write Response).

There is no need to explicitly include the Command Type (read, write, disable) in Command Assistance Information

## Optional presence of Device ID in Inventory Report

There might be scenarios where an AIOT application may want to initiate a procedure which just involves getting an estimate of how many devices satisfy a certain criterion (e.g. number of A-IoT devices which are flood sensors belonging to a certain area which shows water level > X).  In such a scenario, the Device ID(s) are not useful and only the number of devices are useful.

In such a scenario, the device ID may not need to be sent in D2R and hence also in NG Inventory Report. This means including the device ID in Inventory Report should be optional.

AF/AIOTF should indicate to the gNB (Reader) in Inventory Request whether Device ID is to be included in Inventory Report. Based on the indication, the gNB should provide

There might be scenarios where an AIOT application may want to just get an estimate of how many devices satisfy a certain criterion (e.g. number of A-IoT devices which are flood sensors belonging to a certain area which shows water level > X).  In such a scenario, the Device ID(s) are not useful and only the number of devices are useful.

AIOTF indicates to gNB in NG Inventory Request via a “Device ID needed” flag whether Device ID is to be included in NG Inventory Report.

If the “Device ID needed” flag is set to FALSE, gNB informs the A-IoT device via A-IoT Paging that there is no need for A-IoT Device to include “Device ID” in D2R message carrying Inventory Report.

# Summary

Based on the discussion above, we have following observations and proposals:

2.1 Whether to include the Correlation ID outside the Transfer IEs

Observation 1:	AMF just needs to route the Inventory related Transfer IEs and Command related Transfer IEs to the appropriate gNB (based on the gNB ID) for request messsages, and to the appropriate AIOTF (based on the AIOTF ID) for response messages

Observation 2:	There is no need for AMF to identify the A-IoT session and therefore no need to know the Correlation ID (even in case of parallel Inventory/Command sessions).

Proposal 1:	There is no need to include the Correlation ID outside the Inventory related Transfer IEs and Command Transfer IEs

Proposal 2:	Correlation ID is included only inside the Command related Transfer IEs

2.2 Device Association

Proposal 3:	Upon receiving the RAN A-IoT Device NGAP ID in Inventory Report, AIOTF can allocate an AIOTF A-IoT Device NGAP ID to uniquely identify the A-IoT device per session per gNB/Reader.

Proposal 4:	RAN A-IoT Device NGAP ID is included only inside the Inventory Report Transfer IE

Proposal 5:	RAN A-IoT Device NGAP ID and AIOTF A-IoT Device NGAP ID is included only within the Command Transfer IEs (i.e., there is no need to include these IDs or any “Transaction” ID outside the Command Transfer IEs).

2.3 Expected D2R Message Size in Inventory Request

Proposal 6:	Include “Expected Inventory Response size” as part of Inventory Assistance Information in Inventory Request

2.4 Command Type

Proposal 7:	Include “Expected Command Response size” as part of Command Assistance Information in Command Request.

Observation 3:	With the knowledge of Expected Command Response size, the gNB serving the BS Reader can implictly know the Command Type and do appropriate resource allocation for Command Responses (e.g., larger TB for Read Response or smaller TB for Write Response).

Proposal 8:	There is no need to explicitly include the Command Type (read, write, disable) in Command Assistance Information

2.5 Optional presence of Device ID in Inventory Report

Observation 4:	There might be scenarios where an AIOT application may want to just get an estimate of how many devices satisfy a certain criterion (e.g. number of A-IoT devices which are flood sensors belonging to a certain area which shows water level > X).  In such a scenario, the Device ID(s) are not useful and only the number of devices are useful.

Proposal 9:	AIOTF indicates to gNB in NG Inventory Request via a “Device ID needed” flag whether Device ID is to be included in NG Inventory Report.

Proposal 10:	If the “Device ID needed” flag is set to FALSE, gNB informs the A-IoT device via A-IoT Paging that there is no need for A-IoT Device to include “Device ID” in D2R message carrying Inventory Report.