3GPP TSG-RAN3 Meeting #128	R3-253141

Malta, MT, May 19th – 23rd, 2025

Agenda Item:	16.3

Source: 	Qualcomm Incorporated

Title: 	Locating Ambient IoT devices

Document for:	Discussion/Decision

# Background

In this paper, we continue to provide our views for locating an AIoT device at “reader” granularity as concluded in the SI phase.

# Discussion

## Reader ID in Command Response

There is no conclusion yet on whether to also include Reader ID in Command Response. This was briefly discussed in RAN3#127 but there were concerns raised on how this Reader ID can be used.

In our view, including the Reader ID in Command Response can one way to update the A-IoT device and reader association at the AIOTF e.g., the A-IoT devices might have moved to a different Reader’s coverage in the time between Inventory and Command.

Also, this knowledge can be useful at the AIOTF there is a device association maintained at the AIOTF for subsequent Command after the 1st Command or in case of multiple Commands targeting the same A-IoT device

A-IoT devices might move to a different Reader’s coverage in the time between Inventory and Command.

Including the Reader ID in Command Response can be useful to keep the latest  A-IoT device and reader association at the AIOTF and can be useful to know the “last serving reader” in case of back-to-back commands.

gNB may also include the “Reader index” to the AIOTF in Command Response

## Proximity determination

TR38.769 captures a couple of options as the potential design of proximity determination:

Proximity determination is feasible with either of the two following solutions. Potential specification impact or not will not be determined in this study item.

Solution 1:	If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader.

Solution 2:	If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side.

Proximity determination based on device-side measurements is not considered.

The Rel-19 WID clarified to consider the Proximity determination via Solution 1 in TR 38.769 only.

Proximity determination can uniquely identify a device within a very close range, similar to NFC use cases. As shown in the Figure below, this range can be much smaller than the reader's maximum coverage.

Lower power for R2D/D2R within this proximity range can limit the range. How to reduce the transmission power may be dependent on the range requirement, which could be provided by the AIOTF.

For example, the max range is 15m for device 1 in T1; but the proximity range could be <1m. From a reader point of view, it may need to know whether/how to control the transmitting power for R2D based on the range requirement of the proximity determination.

Fig　Low power for proximity determination

Desired range for proximity determination is typically smaller than range for A-IoT communication (i.e., Inventory/Command)

For proximity determination based on Solution 1 in TR 38.769 (to determine “nearby” devices), the reader may need to reduce its R2D transmit power to find A-IoT devices within the proximity determination range.

To enable the gNB to perform proximity determination an in the desired range, AIOTF needs to inform gNB to (1) perform proximity determination and (2) provide the proximity determination range to the gNB so that the gNB can adapt its R2D transmit power accordingly.

This indicator and range can be provided along with Inventory Request or can be provided in a dedicated “Proximity Determination Request” message.

AIOTF requests gNB to perform proximity determination via a “Proximity Determination request” flag in Inventory Request or via a dedicated message “Proximity Determination Request”.

AIOTF may further provide a range requirement for proximity determination in Inventory Request or via a dedicated message “Proximity Determination Request” (so that the gNB knows the desired range for proximity determination and adapt its R2D Tx power accordingly).

# Summary

2.1	Reader ID in Command Response

Observation 1:	A-IoT devices might move to a different Reader’s coverage in the time between Inventory and Command.

Observation 2:	Including the Reader ID in Command Response can be useful to keep the latest  A-IoT device and reader association at the AIOTF and can be useful to know the “last serving reader” in case of back-to-back commands.

Proposal 1:	gNB may also include the “Reader index” to the AIOTF in Command Response

2.2	Proximity determination

Observation 3:	Desired range for proximity determination is typically smaller than range for A-IoT communication (i.e., Inventory/Command)

Observation 4:	For proximity determination based on Solution 1 in TR 38.769 (to determine “nearby” devices), the reader may need to reduce its R2D transmit power to find A-IoT devices within the proximity determination range.

Proposal 2:	AIOTF requests gNB to perform proximity determination via a “Proximity Determination request” flag in Inventory Request or via a dedicated message “Proximity Determination Request”.

Proposal 3:	AIOTF may further provide a range requirement for proximity determination in Inventory Request or via a dedicated message “Proximity Determination Request” (so that the gNB knows the desired range for proximity determination and adapt its R2D Tx power accordingly).