3GPP TSG-RAN3 Meeting #128	R3-253142

Malta, MT, May 19th – 23rd, 2025

Agenda Item:	10.2

Source: 	Qualcomm Incorporated

Title: 	MRO enhancements for R18 mobility mechanisms

Document for:	Discussion/Decision

# Background

In this paper, we continue to discuss MRO enhancements for LTM based on the open issues and agreements last meeting.

# Discussion

## BFR shortly after successful LTM cell switch (“near-failure”)

RAN3 supports network-based solution for the case of BFR shortly after a successful LTM cell switch.

The target DU identifies the BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization

Target DU needs to send the recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

In RAN3#127bis, it was agreed that in case of BFR shortly after successful LTM cell switch, the target DU sends the recovery beam information to CU, which then forwards it to source DU. There was no conclusion on which F1AP message to use.

In our view, we can use the DU-CU ACCESS AND MOBILITY INDICATION for target gNB-DU to send the “recovery beam ID” and the ACCESS AND MOBILITY INDICATION for gNB-CU to forward them to the source gNB-DU. Further, we think that the UE F1AP ID cab be included in both these messages to identify the UE.

In case of BFR “shortly” after a successful LTM cell switch,

  - Target gNB-DU indicates the “recovery beam ID” and UE F1AP ID to gNB-CU via DU-CU ACCESS AND MOBILITY INDICATION

  - gNB-CU forwards the “recovery beam ID” and UE F1AP ID to source gNB-DU via ACCESS AND MOBILITY INDICATION

## 2.2 LTM cell switch failure due to wrong beam

FFS if/how the solution for case 1 (“near failure”) could also be used for the case of LTM cell switch failure due to wrong beam

In the LS, RAN2 also requested RAN3 whether the solution used for near-failure can also be used for RLF scenario and we think it is easily extensible.

Even in case of LTM cell switch failure thereby triggering an RLF and followed up an LTM recovery or reestablishment, the target gNB-DU can indicate the "reconnected beam ID" (similar to recovery beam ID in the “near-failure” scenario) to gNB-CU which can forward it to the source gNB-DU.

Upon receiving the “reconnected beam ID”, the source gNB-DU can compare it against the beam ID it indicated in LTM Cell Switch Command MAC CE and determine whether RACH-less LTM cell switch had failed due to “incorrect beam indicated in LTM CSC”.

Even in case of RLF or reestablishment, Source gNB-DU can still store the UE context (e.g., the information indicated in CSC) for a certain time (in case it receives a “reconnected beam ID” information) or can retrieve the UE context based on the UE Assistant Identifier indicated in the ACCESS AND MOBILITY INDICATION. Even if RLF Report is received at a later time, that can be used for optimizing other purposes and the network-based solution is sufficient to identify and optimize the LTM due to incorrect beam.

Even in case of RLF or reestablishment, Source gNB-DU can still store the UE context (e.g., the information indicated in CSC) for a certain time (in case it receives a “reconnected beam ID” information) or can retrieve the UE context based on the UE Assistant Identifier indicated in the ACCESS AND MOBILITY INDICATION

In case RACH-less LTM cell switch fails (thereby triggering an RLF) and the UE successfully reestablishes (or performs a RACH-based LTM recovery if configured),

  - Target gNB-DU indicates the "reconnected beam ID" and UE F1AP ID to gNB-CU via DU-CU ACCESS AND MOBILITY INDICATION

  - gNB-CU forwards the “reconnected beam ID” and UE F1AP ID to source gNB-DU via ACCESS AND MOBILITY INDICATION

Upon receiving the “reconnected beam ID” from target gNB-DU (via the gNB-CU), the source gNB-DU can compare it against the beam ID it indicated in LTM Cell Switch Command MAC CE and determine whether RACH-less LTM cell switch had failed due to “incorrect beam indicated in LTM CSC”.

Reply to RAN2 that RAN3 have agreed on a NW based solution to optimize indicated beam information for both "near failure" and "LTM cell switch failure"

## 2.3 RACH-less LTM failure due to “outdated” TA

MRO will cover the scenario that RACH-less LTM fails including outdated TA and UE performs RACH based LTM

For PDCCH order triggered early TA acquisition, RAN3 consider the case of RACH-less LTM fails due to outdated TA and UE performs RACH-based LTM failure recovery or RRC re-establishment to the same cell

FFS whether UE based solution or network-based solution is used for LTM failure due to outdated TA

FFS whether LTM failure due to outdated TA is defined as a new failure case or it can be covered by existing failure cases

RAN2 had the following agreements in RAN2#129bis

Do not specify support for reporting difference between the TA value received from the LTM cell switch command and TA value measured by the UE

Do not specify support for reporting of elapsed time between UE obtains the target cell TA and UE receives the cell switch command

Do not specify support for an indication to indicate whether the UE obtains TA before UE receives LTM cell switch command

RAN2 already to not support several UE-based indications to optimize TA value indicated in LTM CSC and the timing of the early acquisition or LTM CSC.

We think a network-based solution can be adopted to optimize the TA value indicated in LTM CSC, similar to the beam optimization as described in sections 2.1.1 and 2.1.2. Even though the target gNB-DU determined the TA value based on Msg1 sent by UE as part of Early UL sync, we think the detection of “outdated TA” can be done in the source gNB-DU as well.

If the UE undergoes reestablishment (or performs a RACH-based LTM recovery if configured) after RACH-less LTM cell switch failure, target gNB-DU can indicate the "TA value indicated in RAR" to source gNB-DU (via the gNB-CU). The source gNB-DU can compare it against the TA value indicated in LTM Cell Switch Command MAC CE and determine whether RACH-less LTM cell switch had failed due to “outdated TA”. Therefore, the following is proposed:

In case RACH-less LTM cell switch fails (thereby triggering an RLF) and the UE successfully reestablishes (or performs a RACH-based LTM recovery if configured),

  - Target gNB-DU indicates the "TA value indicated in RAR" and UE F1AP ID to gNB-CU via DU-CU ACCESS AND MOBILITY INDICATION

  - gNB-CU forwards the “TA value indicated in RAR” and UE F1AP ID to source gNB-DU via ACCESS AND MOBILITY INDICATION

Upon receiving the “TA value indicated in RAR” from target gNB-DU (via gNB-CU), the source gNB-DU can compare it against “TA value indicated in LTM CSC” and determine whether RACH-less LTM cell switch had failed due to “outdated TA”.

To distinguish these new failure types from the existing failure types, we think the following failure types can be introduced in stage-2 specs and the following is therefore proposed:

Define new failure types “LTM cell switch failure due to wrong beam” and “LTM cell switch failure due to outdated TA” in stage-2 specifications.

## 2.4 TA acquisition type (UE-based vs. network-based)

RAN2 agreed the following:

Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

Correspondingly, RAN3 has the following open issue:

FFS whether to support Network based or UE based solution for TA acquisition type (UE-determined or gNB provided)

There is no consensus whether TA acquisition type is always available in the CU or is there a need for the DU to store and retrieve (partial) UE Context?

FFS whether CU is aware of TA acquisition type without any signalling impact in RAN3 for intra-DU and inter-DU LTM cell switch scenarios

We have the following observations:

gNB-CU knows whether it has configured the UE to perform UE-based TA measurement (e.g., via the ltm-UE-MeasuredTA-ID)

gNB-CU knows the LTM target cell upon receiving the DU-CU CELL SWITCH NOTIFICATION with the target cell ID

gNB-CU knows whether early UL sync was performed for the target LTM cell if it had received the TA value for the target LTM cell via DU-CU TA Information Transfer during preparation phase

gNB-CU can assume that the source gNB-DU must have sent the TA value of LTM candidate cells “as soon as possible” to the UE upon sending the TA value of LTM candidate cells in CU-DU TA Information Transfer (i.e., this is NW provided TA)

Based on the above observations, we think that the Source gNB-CU can always determine the TA acquisition type (gNB provided or UE determined).

Also, we think that there is no need for source gNB-DU to store and retrieve UE context to determine the TA acquisition type as source gNB-CU has all the information to determine the TA acquisition type.

Source gNB-CU can always determine the TA acquisition type (gNB provided or UE determined).

There is no need for UE to report the TA acquisition type (gNB indicated or UE determined) to the gNB in RLF Report.  Send an LS to RAN2 that RAN3 has agreed on NW-based solution.

# Summary

2.1	BFR shortly after successful LTM cell switch (“near-failure”)

Proposal 1:	In case of BFR “shortly” after a successful LTM cell switch,

 Target gNB-DU indicates the “recovery beam ID” and UE F1AP ID to gNB-CU via DU-CU ACCESS AND MOBILITY INDICATION

 gNB-CU forwards the “recovery beam ID” and UE F1AP ID to source gNB-DU via ACCESS AND MOBILITY INDICATION

2.2 LTM cell switch failure due to wrong beam

Observation 1.	Even in case of RLF or reestablishment, Source gNB-DU can still store the UE context (e.g., the information indicated in CSC) for a certain time (in case it receives a “reconnected beam ID” information) or can retrieve the UE context based on the UE Assistant Identifier indicated in the ACCESS AND MOBILITY INDICATION

Proposal 2:	In case RACH-less LTM cell switch fails (thereby triggering an RLF) and the UE successfully reestablishes (or performs a RACH-based LTM recovery if configured),

 Target gNB-DU indicates the "reconnected beam ID" and UE F1AP ID to gNB-CU via DU-CU ACCESS AND MOBILITY INDICATION

 gNB-CU forwards the “reconnected beam ID” and UE F1AP ID to source gNB-DU via ACCESS AND MOBILITY INDICATION

Proposal 3:	Upon receiving the “reconnected beam ID” from target gNB-DU (via the gNB-CU), the source gNB-DU can compare it against the beam ID it indicated in LTM Cell Switch Command MAC CE and determine whether RACH-less LTM cell switch had failed due to “incorrect beam indicated in LTM CSC”.

Proposal 4:	Reply to RAN2 that RAN3 have agreed on a NW based solution to optimize indicated beam information for both "near failure" and "LTM cell switch failure"

2.3 RACH-less LTM failure due to “outdated” TA

Proposal 5:	In case RACH-less LTM cell switch fails (thereby triggering an RLF) and the UE successfully reestablishes (or performs a RACH-based LTM recovery if configured),

- Target gNB-DU indicates the "TA value indicated in RAR" and UE F1AP ID to gNB-CU via DU-CU ACCESS AND MOBILITY INDICATION

- gNB-CU forwards the “TA value indicated in RAR” and UE F1AP ID to source gNB-DU via ACCESS AND MOBILITY INDICATION

Proposal 6:	Upon receiving the “TA value indicated in RAR” from target gNB-DU (via gNB-CU), the source gNB-DU can compare it against “TA value indicated in LTM CSC” and determine whether RACH-less LTM cell switch had failed due to “outdated TA”.

Proposal 7:	Define new failure types “LTM cell switch failure due to wrong beam” and “LTM cell switch failure due to outdated TA” in stage-2 specifications.

2.4 TA acquisition type (UE-based vs. network-based)

Observation 2.	gNB-CU knows whether it has configured the UE to perform UE-based TA measurement (e.g., via the ltm-UE-MeasuredTA-ID)

Observation 3.	gNB-CU knows the LTM target cell upon receiving the DU-CU CELL SWITCH NOTIFICATION with the target cell ID

Observation 4.	gNB-CU knows whether early UL sync was performed for the target LTM cell if it had received the TA value for the target LTM cell via DU-CU TA Information Transfer during preparation phase

Observation 5.	gNB-CU can assume that the source gNB-DU must have sent the TA value of LTM candidate cells “as soon as possible” to the UE upon sending the TA value of LTM candidate cells in CU-DU TA Information Transfer (i.e., this is NW provided TA)

Proposal 8:	Source gNB-CU can always determine the TA acquisition type (gNB provided or UE determined).

Proposal 9:	There is no need for UE to report the TA acquisition type (gNB indicated or UE determined) to the gNB in RLF Report.  Send an LS to RAN2 that RAN3 has agreed on NW-based solution.