3GPP TSG-RAN WG3 Meeting #128	R3-253163

Malta, MT, 19-23, May, 2025

Agenda Item:	8.1

Source:	ZTE Corporation

Title:	Discussion on SRS Frequency Hopping

Document for:	Discussions & Approval

1	Introduction

RAN2 replied the LS [1] (R2-2503185) to RAN1 on non-Redcap UE UL SRS frequency hopping for positioning, and also informed RAN3 to consider and determine whether there is any impacts to support non-Redcap UE SRS frequency hopping for positioning.

In this contribution, we share our analysis on the potential impacts and provide the corresponding CRs [2][3][4].

2	Discussion

In RAN1#120 meeting, RAN1 reached the following agreement for enabling non-RedCap UE performing UL SRS frequency hopping for positioning:

  - Extend Rel-18’s UL frequency hopping UL SRS for positioning transmission to non-RedCap UEs in a single carrier

  - UE capability for non-RedCap UEs for UL SRS frequency hopping for positioning transmission

Based on the agreement, RAN2 replied the LS including following agreements in the reply LS [1]:

Agreements: No new RRC parameter is needed for support of SRS frequency hopping for non-RedCap UE. This does not exclude a related capability, to be discussed in RAN1. The current Rel-18 restriction on maximum bandwidth per hop as 20MHz should be restricted to RedCap UE only, not to non-RedCap UE. Inform RAN1 that we expect this spec impact.

In Rel-18, the gNB could configure SRS frequency hopping only for RedCap UEs. Based on the recent agreements, it is now confirmed that SRS frequency hopping is extended to both RedCap and non-RedCap UEs in Rel-19.

- In Rel-19, SRS frequency hopping is supported for all UEs, including both RedCap and non-RedCap UEs.

In Rel-18 NRPPa signaling, a new explicit indicator was not introduced for the request of Tx hopping configuration because when the existing Bandwidth IE in the Requested SRS Transmission Characteristics IE exceeds the bandwidth capability of the RedCap UE, the serving gNB implicitly understands that Tx hopping configuration is needed. Now in Rel-19, SRS frequency hopping is supported for all UEs, including both RedCap and non-RedCap UEs. Therefore, requesting SRS frequency hopping based solely on the Bandwidth IE is no longer sufficient, as the bandwidth of non-RedCap UEs typically exceeds 20 MHz. Thus, a new explicit indication is needed to request Tx hopping configuration in Rel-19 scenarios.

- Introduce the SRS Frequency Hopping Request Indication IE in the Requested SRS Transmission Characteristics IE to enable LMF request Tx hopping configuration from gNB.

- Endorse the corresponding Tel-19 CRs to NRPPa[2] and F1AP[3]. And agree the reply LS[5] to RAN2 to inform RAN3’s decision.

3	Conclusion

Following is our proposals and observations:

- In Rel-19, SRS frequency hopping is supported for all UEs, including both RedCap and non-RedCap UEs.

- Introduce the SRS Frequency Hopping Request Indication IE in the Requested SRS Transmission Characteristics IE to enable LMF request Tx hopping configuration from gNB.

- Endorse the corresponding Tel-19 CRs to NRPPa[2] and F1AP[3]. And agree the reply LS to RAN2 to inform RAN3’s decision.

4	Reference

[1] R2-2503185, Reply LS on non-RedCap UE UL SRS frequency hopping for positioning, ZTE Corporation

[2] R3-253164, Correction on NRPPa for additional indication for the request of SRS frequency hopping, ZTE Corporation

[3] R3-253165, Correction on F1AP for additional indication for the request of SRS frequency hopping, ZTE Corporation

[4] R3-253166, Correction on F1AP for additional indication for the request of SRS frequency hopping, ZTE Corporation

[5] R3-253167, Reply LS on non-RedCap UE UL SRS frequency hopping for positioning, ZTE Corporation