3GPP TSG-RAN3 Meeting #128	R3-253173

Malta, MT, May 19th –23rd 2025

Agenda Item:	18.2

Source: 	Qualcomm Incorporated

Title: 	Support LP-WUS subgrouping in RRC_IDLE/RRC_INACTIVE

Document for:	Discussion/Decision

# Background

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

WID of Low-power wake-up signal and receiver for NR (LP-WUS/WUR) (RP-240801) has the below objective for RAN3:

In this contribution, we discuss the details on supporting LP-WUS subgrouping to indicate paging via LP-WUS for a UE in RRC_IDLE/RRC_INACTIVE states.

# Discussion

## 2.1 LP-WUS Extended UE Identity Index Value IE in F1 Paging

For PEI, Extended UE Identity Index Value IE was introduced in the “Core Network Assistance Information for RRC INACTIVE” sent by AMF to NG-RAN. The format of the IE and description is below.

Extended UE Identity Index Value IE

This IE is used by the NG-RAN node to calculate the Paging Frame and Paging Occasion as specified in TS 36.304 [29], the Paging Frame and Paging Occasion for eDRX and the UE_ID based subgroup ID as specified in TS 38.304 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended UE Identity Index Value | M |  | BIT STRING (SIZE(16)) | Encoded as 5G-S-TMSI mod 65536. |

The Extended UE Identity Index Value IE was introduced because the 10-bit size of the UE Identity Index value IE was insufficient for deriving UE_IDs for various scenarios, such as normal paging, PEI with UE_ID-based subgrouping, and with/without eDRX, without any overlap between the UE_IDs. The 16-bit size for the Extended UE Identity Index Value IE was determined based on the UE_ID formula (e.g., 5G-S-TMSI mod 1024, 5G-S-TMSI mod 4096, etc.) for these different scenarios as defined in TS38.304[1].

Observation 1: For PEI, Extended UE Identity Index Value IE was introduced in the “Core Network Assistance Information for RRC INACTIVE” sent by AMF to NG-RAN.

Observation 2: For PEI, the size of the Extended UE Identity Index Value, is dependent on the formula defined for PEI UE ID computation.

gNB-CU forwards the received Extended UE Identity Index Value IE to the gNB-DU via F1 AP paging message, to assist the gNB-DU determine UE_ID.

Observation 3: gNB-CU sends the Extended UE Identity Index Value IE in the F1 AP paging message sent to gNB-DU

In RAN3#127bis, RAN3 discussed whether to introduce a new IE to include extra bits for UE_ID based LP-WUS subgrouping, i.e. the Extended UE Identity Index Value IE into F1 paging. However, no agreement has been made and RAN3 concluded to wait for information from RAN2 to decide.

RAN2 sent an LS to RAN3 [2], R2-2503182, indicating the progress in RAN2 on LP-WUS UE_ID based subgrouping formula with key information as below.

- For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping

- Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X.

- X is based on 32 subgrouping number. Details can be discussed in the running CR.

Observation 4: RAN2 agreed on ‘UE_ID=5G-S-TMSI mod X’ and X is based on 32 subgrouping number.

To support the 32 subgroup IDs for LP-WUS, we need 5 bits in addition to the bits used in determining the UE ID for normal paging, eDRX and PEI support. Existing 16-bit size Extended UE Identity Index Value IE has only one extra bit, which is not enough to support LP-WUS.  Thus, we need to introduce a new IE ‘Further Extended UE Identity Index Value IE’ of size 20 bits to indicate the UE Identity value to the RAN when LP-WUS is supported. When LP-WUS is not supported by the UE, CN use the existing 16-bit size Extended UE Identity Index Value IE to indicate the UE Identity value to the RAN.

Observation 5: Existing 16-bit size Extended UE Identity Index Value IE has only one extra bit available, which is not enough to support 32 subgroup IDs for LP-WUS (i.e. needs 5 more bits).

Proposal 1: 20 bits are needed to indicate the UE identity value, derived based on 5G-S-TMSI, from CN to RAN, to assist RAN derive UE_ID for various scenarios, such as normal paging, PEI, LP-WUS and with/without eDRX, without any overlap between the UE_IDs.

Proposal 2: Introduce a new IE ‘Further Extended UE Identity Index Value IE’ of size 20 bits to indicate the UE Identity value to the RAN when LP-WUS is supported.

Proposal 3: When LP-WUS is not supported by the UE, CN use the existing 16-bit size Extended UE Identity Index Value IE to indicate the UE Identity value to the RAN.

## 2.2 CN Subgroup ID range

RAN1 agreed “One codepoint for each PO corresponding to all the subgroups in the PO” is used. That is a Paging message that has to be sent to all UE subgroups need to use a common subgroup ID which gets mapped to a common codepoint by the PHY layer based on some rules. Paging message for all UE subgroups is only relevant for the case of RAN paging for RRC_INACTIVE UEs, i.e. UE based LP-WUS subgrouping ID, and not CN based subgrouping ID.

Observation 6: Common subgroup ID, used for sending Paging message for all UE subgroups, which gets mapped to a common codepoint by the PHY layer based on some rules is only relevant for the case of RAN paging for RRC_INACTIVE UEs, i.e. UE based LP-WUS subgrouping ID, and not CN based subgrouping.

RAN2/RAN1 did not decide on whether a value of ‘0’ or ‘31’ is used for common subgroup ID. RAN2 agreed on the below UE based LP-WUS subgroup ID determination formula in RAN2#129. In this formula, subgroupsNumForUEID is the subgroup number for UE_ID based subgrouping for LP-WUS. RAN1/RAN2/RAN3 WGs agreed that the max number of subgroups is 31, i.e. the subgroupsNumForUEID used in the formula will never exceed 31. Hence, the UE Subgroup ID determined using the formula/mod operation below will be between 0 and 30. A value of ‘0’ for common subgroup ID will conflict with the UE specific subgroup ID and should not be reserved as the common subgroup ID. Instead, a value of ‘31’ should be used.

On sub-grouping

- For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Observation 7: UE Subgroup ID determined using the mod operation with subgroupsNumForUEID (max #of subgroups is 31) will be between 0 and 30. A value of ‘0’ for common subgroup ID will conflict with the UE specific subgroup ID and should not be reserved as the common subgroup ID. Instead, a value of ‘31’ should be used as the common subgroup ID.

In RAN3#127bis, RAN3 agreed that “The maximum number of subgroups is decided to 31.”. However, RAN3 did not agree on the range, whether to define as 0 to 30 or 1 to 31. As clarified in observation 7, using a range of 1 to 31, i.e. CN assigning a value ‘31’ for CN subgroup ID for a specific UE will result in conflict with the common subgroup ID used for all UE subgroups for UE based LP-WUS subgrouping ID. Thus, the range of CN Subgroup ID should be 0 to 30.

Even for PEI, CN subgroup ID range is defined as 0 to 7 for supporting 8 subgroup IDs. We can follow the same for LP-WUS and start the range from ‘0’ instead of ‘1’.

Proposal 4: Define the range for “LP-WUS CN Subgroup ID” in LP-WUSPS Assistance Information IE as ‘0 to 30’

# Summary

Below are the observations and proposals discussed in this document.

Observation 1: For PEI, Extended UE Identity Index Value IE was introduced in the “Core Network Assistance Information for RRC INACTIVE” sent by AMF to NG-RAN.

Observation 2: For PEI, the size of the Extended UE Identity Index Value, is dependent on the formula defined for PEI UE ID computation.

Observation 3: gNB-CU sends the Extended UE Identity Index Value IE in the F1 AP paging message sent to gNB-DU

Observation 4: RAN2 agreed on ‘UE_ID=5G-S-TMSI mod X’ and X is based on 32 subgrouping number.

Observation 5: Existing 16-bit size Extended UE Identity Index Value IE has only one extra bit available, which is not enough to support 32 subgroup IDs for LP-WUS (i.e. needs 5 more bits).

Observation 6: Common subgroup ID, used for sending Paging message for all UE subgroups, which gets mapped to a common codepoint by the PHY layer based on some rules is only relevant for the case of RAN paging for RRC_INACTIVE UEs, i.e. UE based LP-WUS subgrouping ID, and not CN based subgrouping.

Observation 7: UE Subgroup ID determined using the mod operation with subgroupsNumForUEID (max #of subgroups is 31) will be between 0 and 30. A value of ‘0’ for common subgroup ID will conflict with the UE specific subgroup ID and should not be reserved as the common subgroup ID. Instead, a value of ‘31’ should be used as the common subgroup ID.

Proposal 1: 20 bits are needed to indicate the UE identity value, derived based on 5G-S-TMSI, from CN to RAN, to assist RAN derive UE_ID for various scenarios, such as normal paging, PEI, LP-WUS and with/without eDRX, without any overlap between the UE_IDs.

Proposal 2: Introduce a new IE ‘Further Extended UE Identity Index Value IE’ of size 20 bits to indicate the UE Identity value to the RAN when LP-WUS is supported.

Proposal 3: When LP-WUS is not supported by the UE, CN use the existing 16-bit size Extended UE Identity Index Value IE to indicate the UE Identity value to the RAN.

Proposal 4: Define the range for “LP-WUS CN Subgroup ID” in LP-WUSPS Assistance Information IE as ‘0 to 30’

# References

[1] TS 38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 18)

[2] R2-2503182, LS on LP-WUS subgrouping progress, source: RAN2, To: RAN3, Cc: SA2, SA3