3GPP TSG-RAN WG3 #128	R3-253214

St. Julian's, Malta, 19th – 23rd May 2025

Source:				NEC

Agenda item: 	10.2. MRO Enhancements

Title: 	(TP to BLCR of 38.473) Discussion on MRO for LTM

Document for:				Discussion and decision

- Introduction

During previous RAN3 meetings, MRO enhancements for LTM were discussed, and the following agreements were made:

RAN3#123 bis: Work on scenarios of near failure LTM. Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell. RAN3#124: MRO for LTM: RAN3 to prioritize MCG LTM over SCG LTM. RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study. For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization. For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization. RACH information in LTM: RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM. RAN3#125: MRO for LTM: Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover. FFS to enhance failure scenario. Take the existing detection mechanism descriptions as baseline. For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure. CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed. RAN3#125 bis: For TP to BL CR to TS 38.300: - Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions. - For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM. - Change“successful handover" definition, as proposed in R3-245230. For TP to BL CR to TS 38.401: - Capture wrong LTM prepared cell selection. Add failure type to F1AP ACCESS AND MOBILITY INDICATION message. CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell. RAN3#126: For TP to BL CR to TS 38.473: - In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI. For TP to BL CR to TS 38.401: - For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM. - For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell. RAN3#127 CU can adjust the candidate Cell list for LTM based on UHI. DU should be able to resolve LTM ping-pong based on information over F1. Network based solution is used for LTM candidate cell list. Network based or UE based solution for TA acquisition type? RAN3#127bis BFR shortly after successful LTM cell switch to the wrong beam RAN3 supports network-based solution for the case of BFR shortly after a successful LTM cell switch. The target DU identifies the BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam. In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization Target DU needs to send the recovery beam information to CU and CU forwards it to source DU. CU does not need to send the old beam information to source DU. LTM failure due to outdated TA MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM. RAN3 will not consider the case of LTM cell switch failure due to outdated TA calculated by UE. For PDCCH order triggered early TA acquisition, RAN3 consider the case of RACH-less LTM fails due to outdate TA and UE performs RACH-based LTM failure recovery or RRC re-establishment to the same cell. UHI and ping-pong issue For intra-CU case, CU detects the potential LTM ping-pong issue based on local information, DU decides and solves the LTM ping-pong issue.

In this contribution, we would like to provide our further consideration on MRO for LTM.

- Discussion

2.1 MRO enhancement in case of RLF-report absent

There may be cases that the RLF-report is not available at the CU. For example, it is not availabe because the UE does not have the RLF-report, i.e., before being reported to the network, the content of the RLF-report associated with the LTM failure is replaced by a following connection failure. For another example, the RLF-report cannot be obtained by the gNB-CU due to network congestion, and finally discarded by the UE after 48 hours. When e.g., these cases happen and lead to that RLF-report is not available in the gNB-CU, the gNB-CU can still identify the happening of LTM failure event:

- Too late LTM:  after the configuring of LTM configuration to the UE, if the gNB-CU finds that LTM cell switch is not triggered by the gNB-DU, and the UE re-establishes/re-connectes to another cell, it can identify the happening of too late LTM cell switch.

- Too early LTM: if the gNB-CU finds that a LTM cell switch is triggered by the gNB-DU, the UE does not access to the target LTM cell or soon after the UE accesses to the arget LTM cell, and the UE re-establishes/re-connects to the source cell of the LTM cell switch procedure, the gNB-DU can identify the happening of too early LTM cell switch.

- LTM to wrong cell:  if the gNB-CU finds that a LTM cell switch is triggered by the gNB-DU, the UE does not access to the target LTM cell or soon after the UE accesses to the target LTM cell, and the UE re-establishes/re-connects to a cell which is neither the source cell nor the target cell of the LTM cell switch procedure, the gNB-DU can identify the happening of LTM cell switch to wrong cell.

Similar to Xn based procedures which have already been supported for Layer 3 based mobility, for example by Handover Report message and Failure Indication message, the the source gNB can be provided with handover failure information with RLF-report optionally presented, the gNB-CU can also indicate to gNB-DU the failure information of LTM cell switch without RLF report.

For LTM failure wihout RLF-report available in gNB-CU, in addition to the failure type (i.e., too late LTM, too early LTM and LTM to wrong cell) and UE ID (including gNB-DU F1AP ID and source C-RNTI), some basic information can also be provided to the gNB-DU by the gNB-CU, for example the ID of the source cell/failed cell of the gNB-DU, target cell of the LTM cell switch procedure, and ID of the re-established/re-connected cell.

Proposal 1: When RLF-report is absent in gNB-CU, gNB-CU can indicate to gNB-DU the following LTM failure information:

- Failure type (i.e., too late LTM, too early LTM and LTM to wrong cell)

- UE ID (gNB-DU F1AP ID, source C-RNTI)

- ID of the source/failed cell of the gNB-DU, and ID of target LTM cell

- ID of re-established/re-connected cell

2.2 Ping-pong avoidance for LTM

At the last RAN3 meeting, it was agreed gNB-CU detects the potential LTM ping-pong issue based on local information, and gNB-DU decides and solves the LTM ping-pong issue. Therefore, the pontentional solutions would be that the gNB-CU sends to the target gNB-DU the UHI or ping-pong indicator when ping-pong cell switch is detected.

Firstly, regarding the message to include the UHI or ping-pong indicator, it would be feasible to use CU-DU Cell Switch Notification message, rather than Access and Mobility Indication message as the CU can indicate the occurance of ping-pong immediately after detection of potential ping-pong by using CU-DU Cell Switch Notification meesaage, and another message after the LTM cell switch is not needed.

Secondly, on whether to include UHI or a simple ping-pong indicator, since the gNB-DU still has the UE context information, so we understand that both options can work. And we are fine with any of them.

Proposal 2: If ping-pong LTM cell switch is detected by gNB-CU, information e.g., UHI or ping-pong indicator, is sent to the gNB-DU by CU-DU Cell Switch Notification message

2.3 BFR shortly after successful LTM cell switch to the wrong beam

During last meeting, it was agreed to supports network-based solution for the case of BFR ‘shortly’ after a successful LTM cell switch. However, it is unclear how to define which case is “shortly” and which is not.

For the mobility problem, for legacy Too Early or Too Late Handovers, the gNB detects a RLF occurs ‘shortly’ after a successful handover from a source cell to a target cell based on a threshold value (e.g., Tstore_UE_cntxt) configured by the OAM [TS 38.300]. So similarly, for the case of wrong beam during LTM, the OAM can also configure a threshold value for the gNB-DU to determine whether the case can be considered as BFR ‘shortly’ after a successful LTM cell switch. More specifically, if the time gap between a successful LTM cell switch and a BFR initiated by the UE to the target cell is smaller than the configured threshold value, the gNB-DU can identifiy the case of BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam.

Proposal 3: A threshod is configured by the OAM for the gNB-DU to identifiy the case of BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam.

For the case of LTM cell switch failure due to wrong beam, the remaining issue for this topic is whether the similar optimization could also be applied. In our understanding, if the LTM cell switch fails finally, it may be caused due to multiple reasons, e.g., wrong beam or outdated TA value etc. If the beam selected during a RA procedure later caused by a LTM recovery or RRC Re-establishment at the same cell, is different from the beam selected during the LTM cell switch procedure, it does not mean the LTM failure is caused by wrong beam. This is because the beam selected during the RA procedure is very random, i.e., as long as the RSRP of the beam is above a threshold, the beam can be selected. Therefore, we do not think it is very benefical to apply the similar optimization for the case of LTM cell switch failure due to wrong beam, instead, including L1 measurement result in RLF-report as agreed in RAN2 is sufficient for network to optimize for the case of LTM cell switch due to wrong beam.

Proposal 4: For the case of LTM cell switch failure due to wrong beam, do not support further optimization in RAN3 specification, instead, using L1 measurement result in RLF-report as agreed in RAN2 is sufficient for network to optimize.

2.4 Cell switch information for LTM

In conventional L3 handover procedure, the source gNB provides to the target gNB Handover Request message including a mobility information, which enable later analysis of the conditions that led to a wrong HO. Upon reception of the mobility information, the target gNB shall, if supported, store this information. And when problem of the related HO procedure is detected, for example a too late HO or HO to wrong cell happens for the HO associated with the mobility information, the target gNB may send to the source gNB a Handover Report message including the mobility information.

Simiarly, for LTM, a “cell switch information” which identifies paramenters and conditions of the cell switch procedure can be sent from the source gNB-DU to the gNB-CU, such that when problem of the LTM cell switch procedure is detected, the gNB-CU can send to the source gNB-DU the Access and Mobility Indication message including the “cell switch information” associated with the LTM cell switch procedure. With the cell switch information, the source gNB-DU can further analysis the condition that led to the wrong LTM cell switch.

Proposal 5: Introdcue “Cell Switch Information” in DU-CU Cell Switch Notification message and Access and Mobility Indication message.

# 3. Conclusion

In this paper, we provide our further consideration on MRO for LTM, and have the following proposals:

Proposal 1: When RLF-report is absent in gNB-CU, gNB-CU can indicate to gNB-DU the following LTM failure information:

- Failure type (i.e., too late LTM, too early LTM and LTM to wrong cell)

- UE ID (gNB-DU F1AP ID, source C-RNTI)

- ID of the source/failed cell of the gNB-DU, and ID of target LTM cell

- ID of re-established/re-connected cell

Proposal 2: If ping-pong LTM cell switch is detected by gNB-CU, information e.g., UHI or ping-pong indicator, is sent to the gNB-DU by CU-DU Cell Switch Notification message

Proposal 3: A threshod is configured by the OAM for the gNB-DU to identifiy the case of BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam.

Proposal 4: For the case of LTM cell switch failure due to wrong beam, do not support further optimization in RAN3 specification, instead, using L1 measurement result in RLF-report as agreed in RAN2 is sufficient for network to optimize.

Proposal 5: Introdcue “Cell Switch Information” in DU-CU Cell Switch Notification message and Access and Mobility Indication message.

Annex	TP for LTM for 38.473

### 8.3.9	DU-CU Cell Switch Notification

#### 8.3.9.1	General

The purpose of the DU-CU Cell Switch Notification procedure is to enable the gNB-DU to inform the gNB-CU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-DU to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: DU-CU Cell Switch Notification procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU CELL SWITCH NOTIFICATION message.

Upon reception of the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

Interaction with CU-DU Cell Switch Notification procedure:

If the LTM Cell Switch Information IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

If the TA Information List IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message to transfer the TA value(s).

If the Cell Switch Information IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, store this information.

8.3.9.3	Unsuccessful Operation

Not applicable.

8.3.9.4	Abnormal Conditions

Not applicable.

---------------------------------------------------------------next change-----------------------------------------------------------------

8.11.1	Access and Mobility Indication

8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the LTM Failure information IE, the gNB-DU may take it into account for optimization of LTM cell switch related paramenters.

8.11.1.3	Abnormal Conditions

Not applicable.

---------------------------------------------------------------next change-----------------------------------------------------------------

#### 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| Cell Switch Information | O |  | BIT STRING (SIZE (32)) | Information related to the cell switch; the source gNB-DU node provides it in order to enable later analysis of the conditions that led to a wrong cell switch. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

---------------------------------------------------------------next change-------------------------------------------------------------------

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| >>Cell Switch Information | O |  | BIT STRING (SIZE (32)) | Information provided in the DU-CU CELL SWITCH NOTIFICATION message from the source gNB-DU. | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| LTM Failure information | O |  | 9.3.1.x | Includes information of a LTM cell switch failure event. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

---------------------------------------------------------------next change-------------------------------------------------------------------

9.3	Information Element Definitions

9.3.1	Radio Network Layer Related IEs

[skip]

#### 9.3.1.x	LTM failure Information

The LTM failure Information IE contains information about a LTM failure event.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Assistant Identifier | M |  | gNB-DU UE F1AP ID 9.3.1.5 |  |
| C-RNTI | M |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU |
| LTM failure type | M |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  |
| Source cell CGI | M |  | NR CGI 9.3.1.12 | NR CGI of source cell of the LTM cell switch procedure or failed cell of the gNB-DU. |
| Target cell CGI | O |  | NR CGI 9.3.1.12 | NR CGI of target cell for LTM cell switch procedure |
| Re-establishment cell CGI | O |  | NR CGI 9.3.1.12 | NR CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the LTM failure |
| Cell Switch Information | O |  | BIT STRING (SIZE (32)) | Information provided in the DU-CU CELL SWITCH NOTIFICATION message from the source gNB-DU. |