3GPP TSG-RAN WG3 Meeting #128	 R3- 253215

St Julian’s, Malta, May 19-23, 2025

Agenda Item:	18.2

Source: 	NEC

Title:  	Discussion on LP-WUS UE-ID

Document for:	Discussion and Decision

- 1 Introduction

For UE identity index value:

In last RAN3#127b meeting, the discussion is shown as below:

RAN3 waits to work on extension of Extended UE Identity Index IE until we receive LS from RAN2 regarding final decision on subgroup ID formula and extension of UE_ID.

Once extension of Extended UE Identity Index IE is confirmed, RAN3 to discuss whether the new IE should be extended some bits only for LP-WUS or more bits for future proof.

In last RAN2#129b meeting, the following agreements related to LP-WUS subgroup are made:

LP-WUS is supported with eDRX, FFS on exact impact if any

Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID = 5G-S-TMSI mod X.

X is based on 32 subgrouping number. Details can be discussed in the running CR.

Send LS to RAN3 (CC SA2/SA3) to inform our agreements on UE ID based subgrouping.

Confirm the principle for determining CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping. Details will be discussed in the running CR.

In this contribution, we further discuss how to design UE identify index value.

- 2 Discussion

- 2.1 UE Identity Index Value

Background

The IE – (extended) UE Identity Index Value is used by the target NG-RAN node to calculate the Paging Frame and Paging Occasion for either normal DRX or eDRX, and calculate UE_ID based PEI subgroup ID. And it represents UE_ID in TS 38.304.

Before discussion, let’s take a look at the paging formula:

The PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

UE_ID:

If the UE operates in eDRX as specified in clause 7.4:

- 5G-S-TMSI mod 4096

else:

- 5G-S-TMSI mod 1024

It can be seen that for normal DRX scenario, the UE_ID could be [0 ~ 1023], which means 10 bits are needed to represent UE_ID. For CN paging, the gNB can acquire 5G-S-TMSI from NGAP PAGING message and directly calculate UE_ID, while for RAN paging, the anchor gNB is not aware of UE’s 5G-S-TMSI (I-RNTI is used instead), in order to calculate paging occasion, the anchor gNB needs to directly acquire UE_ID from Core Network Assistance Information for RRC INACTIVE sent by CN and this message includes 10 bit UE Identity Index Value. When it comes to RAN paging exchange between anchor gNB and other gNB, i.e., XnAP RAN PAGING message, the anchor gNB also needs to inform other gNBs of UE_ID. Therefore this is the usage of UE Identity Index Value. It is also noted that the current UE_ID could be [0 ~ 8191] for PEI UE-ID based subgroup and could be [0 ~ 32767] for eDRX case, which means more bits are needed to represent UE_ID, that is reason why Extended UE Identity Index Value (16 bits) is introduced.

Extended UE identity index value for LP-WUS

Back to LP-WUS discussion, RAN2 now has already made progress on UE-ID based subgroup formula and sent LS to RAN3, from RAN3 perspective, we can make the final decision on UE identity index value design based on RAN2 agreement.

There are some key agreements related to UE-ID design:

1. Np (PEI UE-ID based subgroup number) is considered into LP-WUS UE-ID based subgroup formula;

2. UE-ID = 5G-S-TMSI mod X;

3. Value X is calculated based on 32 LP-WUS UE-ID based subgroups;

4. eDRX is supported with LP-WUS.

Now the LP-WUS UE-ID based subgroup formula is shown as the below:

SubgroupID = (floor(UE_ID/(N*Ns*Np)) mod lp-SubgroupsNumForUEID) + (lp-SubgroupsNumPerPO – lp-SubgroupsNumForUEID),

where:

N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1

Ns: number of paging occasions for a PF

Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1

UE_ID: 5G-S-TMSI mod X, X is FFS

lp-SubgroupsNumForUEID and lp-SubgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively

With this understanding, to make sure that all UEs with different UE-ID can be allocated into every PF/PO/subgroup, i.e., no PF/PO/subgroup remains unused, the value of UE-ID should not be less than [N*Ns*Np*lp-SubgroupsNumForUEID].

Currently the max N (PF number) is 256 without eDRX case, and 1024 with eDRX case, since RAN2 supports eDRX case, RAN3 can only assume the maximum value, i.e., N should be based on 1024. Max value of Ns is 4 and value of Np is 8. As for lp-SubgroupsNumForUEID, RAN2 assumes we need to count this value as 32 even though the actual subgroup is 31 (one is for common group but maybe not count as one subgroup as per RAN1 understanding). Therefore, the max UE-ID should be [1024*4*8*32] – 1, i.e., 1048576 – 1 (why minus 1 is due to modulo operation), and value X should be set to 1048576 (20bits). Otherwise, in some cases where the UE supports both PEI and eDRX, UE-ID may be insufficient, e.g., the last few subgroup IDs will never be used or assigned to UEs if value of X is not enough.

Therefore at least 20 bits are needed for UE identity index value, the current UE identity index value (i.e., 16 bits) is not enough for calculating LP-WUS subgroup ID. So that RAN3 can confirm that new type of extended UE identity index value should be introduced.

As for whether the new IE should be extended some bits only for LP-WUS or more bits for future proof. In our understanding, RAN2 is not likely to further extend any value of [N*Ns*Np* lp-SubgroupsNumForUEID] in the future, it is now stable for paging related feature design, also we do not see other use case which also requires to use more bit UE-ID. Therefore only 20 bits are sufficient.

Proposal-1: new extended UE identity index value with 20 bits is introduced.

Proposal-1a: the new extended UE identity index value with 20bits is included in Core Network Assistance Information for RRC INACTIVE IE, RAN PAGING message and F1AP PAGING message.

Furthermore for more information, based on RAN2 discussion, they also need to consider whether to have different value X to differentiate eDRX case and PEI case. As for different case, [N*Ns*Np*lp-SubgroupsNumForUEID] could be different. Shown in the table below.

| UE-ID with LP-WUS subgrouping | If PEI is not configured or not supported | If PEI is configured and supported |
| --- | --- | --- |
| If eDRX is not used | X = 32768 (256*4*32) 15bits | X = 262144 (256*4*8*32) 18bits |
| If eDRX is used | X = 131072 (1024*4*32) 17bits | X = 1048576 (1024*4*8*32) 20bits |

Noted that this table is intended to list every possibility for value of X based on RAN2 discussion, but it does not mean they will specify value of X for all cases.

From RAN3 perspective, as long as the value X is not above 1048576 (or 20bits), UE-ID can work without any problem no matter how many values of X that RAN2 needs, this is because multiple of 2 and modulo operation is adopted (in other word, 20 bit UE identity index value can already represent UE-ID in TS 38.304 for different case). In this sense, as LS contact company, we suggest to send reply LS to RAN2 to inform our agreement and this value X limitation.

Proposal-2: Send reply LS to RAN2, inform RAN2 of RAN3 agreement and value X limitation, i.e., as long as value X is not above 1048576 (or 20bits), UE-ID can work without problem no matter how many values of X that RAN2 needs for different case.

- 3 Conclusions

Proposal-1: new extended UE identity index value with 20 bits is introduced.

Proposal-1a: the new extended UE identity index value with 20bits is included in Core Network Assistance Information for RRC INACTIVE IE, RAN PAGING message and F1AP PAGING message.

Proposal-2: Send reply LS to RAN2, inform RAN2 of RAN3 agreement and value X limitation, i.e., as long as value X is not above 1048576 (or 20bits), UE-ID can work without problem no matter how many values of X that RAN2 needs for different case.

- 4 References