3GPP TSG-RAN WG3 Meeting #128	R3-253231

Malta, MT, 19 – 23 May 2025

Agenda item:	18.2

Source:	Nokia

Title:	Conclusions for LP-WUS open points

WID/SID:	NR_LPWUS

Document for:	Discussion and Decision

# 1	Introduction

At the last RANP#102 meeting, the work item of LP-WUS was agreed in [1] including the following objective:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

The work item has progressed in previous RAN3 meetings reaching already some agreements and working assumptions while some points remain open. This paper reviews the current agreements and propose conclusions for current open points.

# 2	Discussion

The LP-WUS feature has been designed to optimize further energy consumption of the device. It provides similar benefits as prior low power features based on wake-up signals such as PEI but with some differences. Even though the feature applies to both idle, inactive and connected state the main RAN3 impact is related to idle/inactive states.

For example, due to the presence of the new WUR (wake up receiver), some further optimization is reached due to the possibility for the UE to not turn on at all the main radio to monitor the PDCCH even for monitoring the wake-up signal. In this so-called “deep sleep” mode, the wake-up signal is monitored by the WUR instead, enabling further energy saving. Moreover, the main radio could be relaxed from doing some of the measurements taken over by WUR.

A summary of the impacts with resolution of open points is listed below per interface.

NG Impacts

At RAN3#125bis meeting the following was agreed:

The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups.

The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups.

RAN1#118bis further took a working assumption that the maximum number of subgroups for LP-WUS was set to 32.

RAN3#127 therefore agreed the NGAP baseline CR in R3-250840 including the LP-WUS Paging Subgroup Assistance Information IE into the NGAP Paging message and into the NGAP Core Network Inactive Assistance Information IE including the CN subgroup ID IE with range from value 0 to value 31.

Xn Impacts

At RAN1#120, RAN1 further decided one codepoint to wake up all UES: the use case can be e.g. a SIB update or the need to address multiple UES which belong to at least two different groups. For this RAN1 decided that a special codepoint value among the 32 values sent over the air interface will be reserved to “wake up all UEs”. The agreement is shown below:

RAN1#118bis:

Agreement

At least the following codepoints are supported for LP-WUS:

- One codepoint corresponding to each of the subgroups that can be indicated by LP-WUS

- One codepoint corresponding to all the subgroups that can be indicated by LP-WUS

- FFS: Additional codepoints

RAN1#120:

Agreement

Confirm the following working assumption with the modification:

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 31 32.

From RAN3 perspective, the gNB-DU should be able to infer when to use this special codepoint over the air interface: in case of SIB Update the gNB-DU can decide, and for the case of multiple UEs to wake up the F1AP Paging message is only per UE and one UE can only belong to one group.

All this means that the “special LP-WUS codepoint” to wake up all UEs over the air interface doesn’t need to be instantiated over the NG/Xn/F1 Paging. We conclude that only 31 codepoints are needed for the LP-WUS Assistance Information over Xn interface.

Proposal 1: update the FFS on the range of the LP-WUS CN subgroup ID IE in the baseline CR TS 38.473 into 31 values.

Besides, RAN3#126 took the following working assumption:

WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

This working assumption was taken under the assumption that the LP-WUS operation is not unnecessarily restricted to the last serving cell. Due to this working assumption, RAN3#127 agreed the baseline CR for TS 38.423 in R3-250913 with the following editor’s note:

Editor’s note: the inclusion of the LP-WUSPS Assistance Information IE is to be finally confirmed based on whether a UE will be configured to monitor LP-WUS, either only in the last used cell, if indication is sent in the system information, or any other cell (after cell reselection).

RAN2 has not discussed such restriction at all and therefore we can assume that RAN1/RAN2 does not envision such restriction. Therefore, it is proposed to turn this working assumption into agreement and remove the editor’s note from the XnAP baseline CR.

Proposal 2: turn the working assumption into agreement and remove the editor’s note concerning the presence of the LP-WUSPS Assistance Information IE in the XnAP RAN Paging message.

Emergency Call Impact

The handling of emergency call when using LP-WUS was discussed and resolved at RAN3#127bis.

Extended UE Identity Index value

RAN2 has decided that LP-WUS can be used with eDRX. Therefore, we need to extend the UE identity Index value to 20 bits. The work to further extend the UE Identity Index value has been shared across several companies at this meeting. The TP for TS 38.423 is presented in tdoc [3].

Proposal 3: Extend the UE identity Index value to 20 bits over NG/Xn/F1 and agree TP for XnAP in tdoc [3].

# 3	Conclusion and Proposals

This paper has provided a summary of current agreements and has proposed conclusions for current open points. It is update the baseline CRs along the following lines:

Xn Interface (other)

Proposal 1: update the FFS on the range of the LP-WUS CN subgroup ID IE in the baseline CR TS 38.473 into 31 values.

Proposal 2: turn the working assumption into agreement and remove the editor’s note concerning the presence of the LP-WUSPS Assistance Information IE in the XnAP RAN Paging message.

Extended UE Identity Index value

Proposal 3: Extend the UE identity Index value to 20 bits over NG/Xn/F1 and agree TP for XnAP in tdoc [3].

Proposal 1 and proposal 2 are implemented in the TP for BLCR 38.423 in [2]. Proposal 3 is implemented in the TP for BL CR 38.423 in [3].

# 4	References

- RP-240801, Low Power Wake up signal and receiver for NR, Vivo, NTT Docomo, Ericsson, Mediatek, Samsung, Sony

- R3-253233, [TP for BL CR LP-WUS TS 38.423] Resolution of LP-WUS open points

- R3-253232, [TP for BL CR LP-WUS TS 38.423] Further Extension of UE Identity Index Value for LP-WUS